REPORT 2000 - 140

THE KING’S HOSPITAL - A STRUCTURAL HISTORY

OF LOUISBOURG’S BLOCK 13 ROYAL HOSPITAL

1713 - 1960

BY

ERIC KRAUSE

KRAUSE HOUSE INFO-RESEARCH SOLUTIONS

AUGUST 21, 2000

(Revised December 22, 2000)

TABLE OF CONTENTS

PREFACE

NARRATIVE REPORT: A CHRONOLOGY OF BUILT AND PROPOSED FEATURES

PART ONE: 	A CHRONOLOGY OF BUILT AND PROPOSED CONSTRUCTION EVENTS

(1713 - 1960)

PART TWO: 	AN INTERPRETATION OF BUILT EVENTS: HISTORICAL CONSTRUCTION CHECK LIST (1744 - 1745)

(I)	HOSPITAL - Rue d’Orléans

(II)	BAKERY/WASH-HOUSE - Rue Royalle/Rue d’Estrées east Pavilion, housing the Bakery/Wash-House

(III)	STOREHOUSE/STABLES/MENAGERIE- Rue Royalle/Rue Dauphine west Pavilion, housing the Storehouse/Stables/Menagerie

(IV)	PERIMETER PROPERTY FENCES - Rue d’Estrées, Rue Dauphine, and Rue Royalle

(V)	LANDSCAPE FEATURES - Terrace and Inner Courtyard

PART THREE:	REPRODUCED MAPS AND PLANS

(1713 - 1960)

PREFACE

The proposed re-construction date is 1744-1745. This date avoids the genuine problem for a reconstruction project of interpreting the damages, repairs and additional work related to the sieges of 1745 and 1758, and is also the general date chosen for the other reconstructions at the Fortress of Louisbourg (1713-1758).

The source material for this narrative report can be retrieved by accessing the Fortress of Louisbourg computerized “Property” database which is available on public drive K.

NARRATIVE REPORT

A CHRONOLOGY OF BUILT AND PROPOSED FEATURES

PART ONE

A CHRONOLOGY OF BUILT

AND PROPOSED CONSTRUCTION EVENTS

(1713 - 1960)

YEAR: 1721

On March 4-5, a King’s Ordonnance declared that a hospital - to be operated by the Brothers of Charity [but owned by the King] - was to be established opposite the Grand Étang, [within Block 13], and that 6,000 livres of start-up money (with another 6,000 livres pledged for next year) was to be set aside from the fortification funds to meet that purpose. Upon his return from France, Chief Engineer Verville [fortification contractor Isabeau (1719-1724 - died after November 1724) was actually tasked with the work] had laid a part of the [Rue d’Orléans/Rue d’Estrées east portion of the] main [hospital] building’s masonry foundations [in lime mortar], and placed two medals (one of silver, one of copper) in one of its angles. When work that year stopped, a part of the [masonry] wall was declared raised, and the plan was to continue the project the following spring.

YEAR: 1722

In March, the Council in France declared that the construction of the hospital Royal de la Charité was to take precedence over that of the Block 3 Church [which subsequently was never built]. Apparently by March, and certainly sometime that year, an east portion, consisting of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion and a portion of the [Rue d’Orléans east hospital] ward had been sufficiently completed for occupation by the Brothers of Charity. Maps 1725-8, 1725-8a, 1726-1, and 1726-1a illustrates the completed interior layout - ground floor Brother’s [Rue d’Estrées east] right aisle/right pavilion residence, [Rue d’Estrées east] right aisle/right pavilion sacristy, [Rue d’Estrées/Rue d’Orléans] chapel, and a portion of the [Rue d’Orléans east hospital] ward - (with a temporary wall closing off to the west the completed portion of the [Rue d’Orléans east hospital] ward), as well as the exterior features of this portion of the building, with the 1726 plans here assumed to be the more accurate

By this year as well, there had been built a charpente (1/2 timber) outbuilding - with open walls and a roof - housing a small oven. No doubt it was the feature shown on Map 1722-1, located in the inner courtyard immediately to the north of the north wall of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion.

Map 1722-1 illustrated the Isle de l’hospital Royal de la Charité with the following measurements:

(1)	40 toises (240 pieds) along Rue Royalle and Rue d’Orléans

(2)	29 toises 1 pied (175 pieds) along Rue d’Estrées and Rue Dauphine.

Maps 1726-1 and 1726-1 revealed that the completed L-shaped portion of the Brother’s residence/Royal Hospital measured c. 63 pieds along Rue d’Estrées by c. 63 pieds along Rue d’Orléans. Left to be built was another c. 177 pieds along Rue d’Orléans and c. 63 pieds along Rue Dauphine. No other activity was illustrated within the block.

YEAR: 1723

Plans of 1723 did not illustrate any further construction projects within the block, and for a second year, the Brothers continued to occupy the existing hospital Royal. Their presumed oven was once again illustrated, on Map 1723-1, in the inner courtyard immediately to the north of the north wall of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion.

YEAR: 1724

In the King’s storehouse this year were found some [iron] stoves with their pipes for heating the [wards], and some ward beds, as well as some items from the chapel of the [Rue d’Estrées/Rue d’Orléans] lhopital du Roy, included some bells (weighing 147 livres) and some serge for separating the chapel from the [adjoining] ward. Possibly too, from time to time, brick stoves - which were commonly used at Louisbourg, and were certainly assembled in the post siege period for use within the hospital - may have supplemented iron stoves during any emergency.

By year’s end (early November), a project of continuation, to lengthen the existing [Rue d’Orléans] hospital, had resulted in excavation and masonry work that included the [property’s] perimeter fence, the founding of the [Rue d’Orléans west portion of] the exterior walls of the hospital - with at least a part of this work [towards the west] raised 4 pieds above [the foundation of the] ground floor - and the [Rue Royalle east pavilion serving as a] wash-house’s well. Apparently too, it was this year that the excavations for the foundations of the [Rue Royalle east pavilion serving as a] bakery [and wash-house] was also completed as was that for the [Rue Royalle west pavilion serving as a] storehouse/stables/menagerie. The foundations of the Rue d’Estrées wash-house of the [Rue Royalle east] pavilion housing the bakery/wash-house were also partially raised this year.

Map 1724-2 clearly confirmed that extending the hospital (including its [Rue d’Orléans hospital’s] central pavilion) westerly along Rue d’Orléans and down Rue Dauphine to its final position had now taken place. It suggested as well that for the Rue d’Estrées/Rue Dauphine/Rue Royalle perimeter property fences work was also underway. However, it did not illustrate any work on the two [Rue Royalle east and west] pavilions or any other interior Block 13 developments.

In contrast, maps 1725-8 and 1725-8a apparently confirms several 1724 text assertions:

(1)	The defined [Rue Royalle/Rue d’Estrées east] wash-house and [Rue Royalle] bakery pavilion

(2)	The excavated [Rue Royalle/Rue Dauphine west] stables of the [Rue Royalle west] pavilion housing the then proposed stables/wood-shed

Of interest, maps 1725-8 and 1725-8a also describe what was to exist to the west of the existing structure: Sick wards (two on the ground floor and two (suggested) - above); the Brothers’ ground floor and first storey lodgings, basement kitchen with a dépense (storeroom) and ground floor dining room; as well as ground floor latrines, mortuary with its chapel, apothecary, apothecary’s chemistry [storeroom], and guard room. As for the two pavilions, both were described together, as serving as a stables, wood-shed, oven and wash-house. Presumedly, all three structures would employ a braced king post and purlin roof system.

This year 10,000 livres was designated for the current project, and although orders were given that, next spring, the construction of the King's Bastion barracks was to take precedence over all others, this hospital money was not to be diverted to any other work. At season's end, according to a second toisé accounting of November 10, 4,430 livres had been consumed - 230 livres on earth [removal or the like] and 4,200 livres on [lime-mortared] masonry work - leaving 5,570 livres to be carried forward. Accordingly, De Mézy, the Commissaire-Ordonnateur, requested that 20,000 livres be committed to the [1725] hospital construction season.

YEAR: 1721 - 1724

Between 1721 and 1724, Isabeau, the fortification contractor, asked for approximately 21,273 livres for work on the l’hôpital Royal de la charité, covering the following categories:

(Note: The proposed re-construction date is 1744-1745. Only the more useful dimensions and quantities - of which some were averaged - have been extracted - Please consult the transcribed "Toisé provisionnel" and the "Toisé general et difinitif" for complete details)

(1)	Excavations @ 10 livres per cubic toise (c. 850 livres charge)

(a)	Of the right-angled basement of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion and portion of [Rue d’Orléans east hospital large] ward (25 pieds wide x 2 pieds 6 pouces deep)

(b)	For the foundations of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion (3 pieds wide x 2 pieds deep)

(c)	For the foundations of a portion of the [Rue d’Orléans east hospital large ward] of the main building (3 pieds wide)

(d)	For the foundations of the stables (3 pieds wide x 2 pieds deep) of the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie

(e)	For the foundations of the bakery (3 pieds wide x 2 pieds deep) of the [Rue Royalle east] pavilion housing the bakery/wash-house

(f)	For the foundations of the [block’s perimeter property] fences (3 pieds wide x 2 pieds deep)

(g)	Of the well of the [Rue Royalle/Rue d’Estrées east] pavilion housing the wash-house (12 pieds x 12 pieds x 7 pieds deep)

(h)	Of the [western] continuation of the [Rue d’Orléans east hospital] large ward of the main building (24 pieds wide x 1 pieds 2 pouces deep)

(2)	Rough Rubble Stone Masonry Laid in Lime Mortar @ 140 livres per cubic toise (c. 11,200 livres charge)

(a)	For the foundation of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion aisle and a portion of the [Rue d’Orléans east] main [hospital] building (2 pieds 6 pouces thick x 2 pieds 6 pouces high)

(b)	For the wall of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east main hospital building] to roof eave level (2 pieds thick x 19 pieds 6 pouces high)

(c)	For the body (8 pieds long x 5 pieds wide x 7 pieds high), fireplace(s) to the second floor mantle (7 pieds of length x 4 pieds wide x 11 pieds high) and [stack] (5 pieds long x 3 pieds wide x 8 pieds high) of one [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion] chimney to [roof level]

(d)	For the body, oven (8 pieds long x 8 pieds wide x 9 pieds high), and chimney stack (3 pieds 6 pouces long x 3 pieds wide x 8 pieds high) of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion small [inner courtyard] oven to [roof] level

(e)	For the foundation and elevation of an interior bearing wall (20 pieds long x 2 pieds thick x 6 pieds high) in the [Rue d’Estrées east aisle/Rue d’Orléans] basement raised to ground floor level

(f)	For the foundation of the [block’s perimeter property] fences (2 pieds 6 pouces thick x 2 pieds 4 pouces of height)

(g)	For the foundation of a portion of the [Rue Royalle/Rue d’Estrées east] pavilion housing the wash-house (2 pieds 6 pouces thick x 2 pieds 4 pouces of height)

(h)	For a [portion of the height of the] wall of the [block’s perimeter property] fences (2 pieds thick x 3 pieds 6 pouces of height)

(i)	For the foundation of the wall of the main [hospital] building facing Rue d’Orléans (2 pieds 6 pouces thick x 1 pied 6 pouces high)

(j)	For the foundation of the portion of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion [extending towards] the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie (2 pieds 6 pouces thick x 1 pied 6 pouces high)

(k)	For the foundation and elevation of the well of the [Rue Royalle/Rue d’Estrées east] pavilion housing the wash-house (3 pieds thick x 25 pieds 1 pouce of circumference x 7 pieds deep)

(3)	[Imported] Cut Stone @ 3 livres per square pieds (c. 108 livres charge)

(a)	For one corner of cut stones [for quoins] [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east main hospital building wall] (18 pieds of height)

(4)	Hewn Pine Timber @ 45 sols per solive (c. 918 livres charge)

(a)	For the [framing] of the [Rue d’Orléans/Rue d’Estrées east right aisle right pavilion and Rue d’Orléans main hospital] ground floor (8 pouces x 9 pouces thick), first floor (8 pouces x 9 pouces thick), and attic floors (8 pouces x 8 pouces thick)

(b)	For the [framing (4 pouces x 4 pouces thick)] and stringer (2 pouces x 10 pouces thick) of the small stairway in the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion

(c)	For the newel (6 pouces x 6 pouces thick by 6 pieds high)] and stringers and risers (2 pouces x 10 pouces thick) of the large stairway in the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion

(d)	For the roof wall plates and braces (4 pouces x 8 pouces thick), nine trusses (each has - one king post: 8 pouces x 9 pouces thick by 7 pieds 8 pouces of length; 2 principal rafters (each - 4 pouces x 8 pouces thick by 12 pieds long); 2 struts (each - 4 pouces x 4 pouces thick by 4 pieds long) of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and Rue d’Orléans main hospital building]

(e)	For one large roof truss consisting of hip and valley rafters (8 pouces x 8 pouces thick), 2 principal rafters (8 pouces x 8 pouces thick), collar beams (8 pouces x 8 pouces thick) a king post (8 pouces x 8 pouces thick), and 6 braces (each: 4 pouces x 4 pouces thick by 2 pieds of length) for the [Rue d’Estrées/Rue d’Orléans raised] chapel [ceiling]

(f)	For 2 runs of roof purlins (4 pouces x 8 pouces thick totalling 159 pieds of length) of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and Rue d’Orléans main hospital building]

(g)	For the ridge beam (4 pouces x 8 pouces thick by 90 pieds of length) of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and Rue d’Orléans main hospital building]

(h)	For the partition (4 pouces x 8 pouces thick by 20 pieds of length) to close off the [west portion of the [Rue d’Orléans east hospital] ward by a temporary wall]

(i)	For the rafters and eaves (4 pouces x 4 pouces thick with each being 15 pieds long) of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and Rue d’Orléans main hospital building]

(j)	For the rafters and eaves (4 pouces x 4 pouces thick with each being 15 pieds long) of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion small [inner courtyard charpente] oven

(k)	For the rafters (4 pouces x 4 pouces thick) for the [raised] ceiling of the [Rue d’Estrées/Rue d’Orléans] chapel

(l)	For the wall [framing] members (8 pouces x 9 pouces) of the [Rue d’Orléans/Rue d’Estrées east] right/aisle right pavilion small [inner courtyard charpente] oven

(m)	For the roof rafters [4 pouces x 4 pouces thick] of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion small [inner courtyard charpente] oven

(n)	For the ridge beam and other [framing members] (4 pouces x 4 pouces thick) for 9 dormers of the [Rue d’Orléans main hospital] building

(5)	Two-Pouce Thick Pine Floor and Partition Planks @ 20 livres per square toise (c. 2,900 livres charge)

(a) 	For the floors of the ground floor [20 pieds 6 pouces wide by 96 pieds of length), first floor [20 pieds 6 pouces wide by 80 pieds of length], and attic [20 pieds 6 pouces wide by 80 pieds of length] of the [Rue d’Orléans main hospital] building

(6)	One-Pouce Thick [Planed both Sides Pine] Partition Boards @ 14 livres per square toise (c. 714 livres charge)

(a)	For the balustrade (25 pieds long x 2 pieds 9 pouces high) behind the [Rue d’Estrées/Rue d’Orléans] chapel grille

(b) 	Partitions (7 pieds 6 pouces high) for the [Rue d’Estrées/Rue d’Orléans east] chapel corridor

(c)	Partitions (7 pieds 6 pouces high) for the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion rooms

(d)	Partitions (7 pieds 6 pouces high) for unspecified [Rue d’Estrées/Rue d’Orléans east] areas

(e)	For along the 4 sides (each side: 12 pieds 6 pouces of length by 6 pieds high) of the [raised] ceiling, plus a small triangular area (2 pieds of length by 6 pieds high) of the [Rue d’Estrées/Rue d’Orléans] chapel

(f)	For the [raised] ceiling (10 pieds x 10 pieds) of the [Rue d’Estrées/Rue d’Orléans] chapel

(7)	Two-Pouce Thick [Embôiture] Doors @ 36 livres per square toise (c. 54 livres charge)

(a)	For 4 basement doors (each: 5 pieds high x 3 pieds wide) of the [Rue d’Orléans/Rue d’Estrées east] left [sic. right] aisle/right pavilion

(8)	[One-Pouce Thick pine] Doors and Shutters, [with oak embôitures at both ends] @ 30 livres per square toise (c. 450 livres charge)

(a)	For 13 doors (5 pieds 9 pouces high by 2 pieds 6 pouces wide) for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(b)	For 1 door with its architrave (7 pieds 6 pouces high by 3 pieds 6 pouces wide) for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion OR, less likely, for the portion of the Rue d’Orléans east hospital ward]

(c)	For 2 doors (each: 5 pieds 6 pouces high by 4 pieds wide) for the garde meuble of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(d)	For 27 [window] shutters (4 pieds high x 3 pieds wide) for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(e)	For 9 dormer shutters (2 pieds 9 pouces high x 1 pieds 7 pouces wide) for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(9)	Window Frames with their Sashes [members measuring 3 pouces or more x 2 pouces] - @ 52 livres per square toise (c. 468 livres charge)

(a)	For 29 [window] frames with their sashes (3 pieds 9 pouces high x 3 pieds wide) for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(10)	Window Panes for Window Sashes @ 24 sols per square pied (c. 270 livres charge)

(a)	For the glass for 29 window sashes, each sash carrying 20-7 pouce x 8 pouce [glass window panes] for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(11)	Shingles @10 livres per square toise (c. 1,110 livres charge)

(a)	For the [main] slope and hip of the roof of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(b)	For the partition (25 pieds high by 20 pieds 6 pouces wide) which closes off the [west portion of the Rue d’Orléans east hospital ward by a wall]

(c)	For the 9 dormers of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(d)	For the roof (12 pieds long x 10 pieds high) of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion small [inner courtyard] oven

(12)	Large Iron Work @ 5 sols 6 deniers per livres of weight (c. 1,337 livres charge)

(a)	For the fireplace bars, timber iron and timber stirrups of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward]

(13) 	Hardware (c. 894 livres charge)

(a)	118 pairs of strap hinges:

(i)	Of 1 pied 6 pouces of length, with their pintles for doors and shutters for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (40 pairs @ 6 livres per pair)

(ii)	Of 2 pieds of length, with their pintles for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (20 pairs @ 8 livres per pair)

(iii)	Of 1 pied of length, with their pintles for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (58 pairs @ 4 livres per pair)

(b)	Bolts on plates of 6 pouces for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (87 @ 2 livres each)

(c)	[Door] latches for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (8 @ 4 livres each)

(d)	Bernard [door] locks of 7 pouces or 8 pouces long by 4 pouces wide a tour et demy for the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] (7 @ 8 livres each)

YEAR: 1725

This year, on or after February 25, Verrier [the new Chief Engineer] and Ganet, [1725-1737 - the new fortification contractor] continued the work on the lhopital du Roy begun by Isabeau, funded by an additional 15,000 livres. Unfortunately, due to a lack of masons and advance work, only the completion of the [property’s] perimeter walls, a portion of the large face of the hospital - that is, 60 cubic toise of the masonry wall of the [hospital] ward [next to the Rue d’Orléans east pre-existing hospital ward] - some earth transport, and preparatory work for the next season would result.

Maps 1725-8 and 1725-8a illustrates the completed masonry perimeter property walls. They also illustrate that, beginning at the existing [Rue d’Orléans east hospital] ward, the remaining portion of the [Rue d’Orléans/Rue Dauphine] masonry walls of the [hospital] ward had now been completely extended - perhaps 4 pieds in height [above the foundation] as for the initial portion of 1724 - west along Rue d’Orléans and down Rue Dauphine. However, the construction of its other walls had not yet begun, and the building site in behind this year’s Rue d’Orléans’ work remained as it had in 1724 - excavated only.

Noted too was that the bell for the hospital weighed 200 livres, replacing the older, cracked one of 145 livres which had been returned to France. Probably, it was destined for the building’s future bell tower.

YEAR: 1726

Prior to this construction season, Chief Engineer Verrier proposed to define the hospital walls from the [pre-existing Rue d’Orléans east hospital ward] to the [Rue d’Orléans hospital’s] central pavilion as well as to raise the two [Rue Royalle east and west] pavilions, at an estimated cost of 20,000 livres, plus another 3,000 livres for extraordinary work. Apparently with this in mind, Maurepas ordered that 18,000 livres be sent to Isle Royalle. As of December 15, the entire amount remained unspent.

By season end, the hospital walls were well founded - readied for the 1727 construction season - but, disappointingly involving little actual work time, they being built only to window sill level due to a shortage of [imported] cut stone, caused by its late delivery from France. However, unlike his 1725 proposal for 1726, Chief Engineer Verrier had instead founded all rather than just a portion of the walls. This, he thought, would produce a more solid foundational matrix for the remaining building heights.

Readied too for next year and stockpiled was all the wood and hewn timber required for the hospital. Hopes ran high that it would be completed by June, 1727, with the only possible barrier being a shortage of [imported] cut stone and slate. This [imported] cut stone - 730 cubic pieds had already arrived and proposed for 1727 was an order for a further 2,000 cubic pieds - was required for the hospital’s door and window [surrounds if not for elsewhere as well], and the [imported] slate for the building’s roof. Local Isle aux Justeaucorps cut stone was likewise required for the [Rue d’Orléans] hospital, also as a door or gate [surround] material.

Maps 1726-1 and 1726-1a illustrates the construction to window sill level of both the [Rue d’Orléans] hospital and the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion, a now completed [Rue Royalle east] pavilion housing both the [Rue Royalle/Rue d’Estrées east] wash-house and the [Rue Royalle east] bakery - with its previously constructed inner courtyard well) - and possibly a completed [Rue Royalle west] pavilion housing the storehouse/stables/menagerie. Like the [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion and Rue d’Orléans east hospital ward], their roofs were also finished off with shingles rather than slates - and, with fire prevention ladders (installed 1725-1727) as well.

When scaled, maps 1726-1 and 1726-1a appear to illustrate relatively accurate measurements of built [though not of all the proposed] features which had been completed by the end of the 1726 construction season. For example, where comparisons were possible, they closely matched the descriptions contained in the Isabeau Toisé of 1721-1724.

MEASUREMENTS: MAPS 1726 - 1/1726-1a

(Note: All measurements are approximate, and when used in combination may not provide a correct total)

(I) 	HOSPITAL (Rue d’Orléans) MEASUREMENTS

(I)	General - ground level

(a)	Exterior

(i)	Rue d’Estrées		63 pieds

(ii)	Rue d’Orléans	240 pieds

(iii)	Rue Dauphine		63 pieds

(iv)	Terrace side

(east to west)		25 pieds + 38 pieds + 57 pieds + 18 pieds + 78 pieds + 18 pieds + 38 pieds + 25 pieds

(2)	[Rue d’Orléans/Rue d’Estrées east] right aisle/right rubble stone masonry pavilion

(a)	Exterior - ground level and heights

(i)	Perimeter walls

(I)	Rue d’Estrées		63 pieds

(II) 	Rue d’Orléans	25 pieds

(III)	Thickness		2 pieds

(IV)	Height			5 pieds basement height + 18 pieds from ground floor to roof eave

(b)	Interior - ground level

(i)	Length			59 pieds

(ii)	Width				20 pieds

(3)	[Rue d’Orléans east and west hospital] rubble stone masonry wards

(a)	Exterior - ground level and heights

(i)	Foundation

(I)	Thickness		3 pieds(+-)

(II)	Height			Assorted

(ii)	Perimeter walls

(I)	Rue d’Orléans	190 pieds

(II)	Depth			25 pieds

(III)	Thickness		2 pieds, centered upon the foundation, creating a 6 pouce (+-) interior and exterior ledge

(b)	Interior - ground level

(i)	[East] ward

(I)	Length		87 pieds

(II)	Width			20 pieds

(ii)	Central passageway

(I)	Length		20 pieds

(II)	Width			12 pieds

(iii)	[West] ward

(I)	Length		87 pieds

(II)	Width			20 pieds

(4)	[Rue d’Orléans/Rue Dauphine west] left aisle/left rubble stone masonry pavilion

(a)	Exterior - ground level

(i)	Perimeter walls

(I)	Rue Dauphine 	63 pieds

(II)	Rue d’Orléans	25 pieds

(III)	Thickness		2 pieds

(b)	Interior - ground level

(i)	South division

(I)	Length		20 pieds

(II)	Width			20 pieds

(ii)	Central division

(I)	Length		17 pieds

(II)	Width			20 pieds

(iii)	North division

(I)	Length		17 pieds

(II)	Width			20 pieds

(5)	[Rue d’Orléans hospital’s] central rubble stone masonry pavilion

(a)	Exterior - ground level

(i)	Perimeter walls

(I)	Terrace side		78 pieds

(II)	Depth			18 pieds

(III)	Thickness		2 pieds

(b)	Interior - ground level

(i)	Guard room/apothecary’s réserve (storeroom)

(I)	Length		10 pieds

(II)	Width			16 pieds

(ii)	Apothecary

(I)	Length		16 pieds

(II)	Width			16 pieds

(iii)	Mortuary/staircase

(I)	Length		16 pieds

(II)	Width			16 pieds

(iv)	Latrines

(I)	Length		7 pieds

(II)	Width			16 pieds

(v)	Central passageway

(I)	Length		18 pieds

(II)	Width			12 pieds

(II)	BAKERY/WASH-HOUSE (Rue Royalle and Rue d’Estrées) MEASUREMENTS

(1)	[Rue Royalle east] rubble stone masonry pavilion, housing the bakery/wash-house

(a)	Exterior - ground level

(i)	Perimeter walls

(I)	Rue d’Estrées		51 pieds

(II)	Rue Royalle		51 pieds

(III)	Inner courtyard

(north to south)	25 pieds + 25 pieds 6 pouces + 25 pieds 6 pouces + 25 pieds

(ii)	L-shaped perimeter walls

(I)	Rue Royalle		51 pieds

(II)	Rue d’Estrées		51 pieds

(III)	Width			25 pieds

(IV)	Thickness		2 pieds

(b)	Interior - ground level

(i)	Wash-house

(I)	Length		22 pieds

(II)	Width			20 pieds

(ii)	Bakery (fore-room)

(I)	Length		23 pieds

(II)	Width			20 pieds

(iii)	Oven room (rear-room)

(I)	Length		19 pieds

(II)	Width			20 pieds

(III)	STOREHOUSE/STABLES/MENAGERIE (Rue Royalle and Rue Dauphine) MEASUREMENTS

(1)	[Rue Royalle west] rubble stone masonry pavilion, housing the storehouse/stables/menagerie

(a)	Exterior - ground level

(i)	Perimeter walls

(I)	Rue Dauphine		51 pieds

(II)	Rue Royalle		51 pieds

(III)	Inner courtyard

(north to south)	25 pieds + 25 pieds 6 pouces + 25 pieds 6 pouces + 25 pieds

(ii)	L-shaped perimeter walls

(I)	Rue Royalle		51 pieds

(II)	Rue Dauphine		51 pieds

(III)	Width			25 pieds

(IV)	Thickness		2 pieds

(b)	Interior - ground level

(i)	East stable/menagerie

(I)	Length		23 pieds

(II)	Width			20 pieds

(ii)	West stable/menagerie

(I)	Length		46 pieds

(II)	Width			20 pieds

(IV)	PERIMETER PROPERTY FENCES MEASUREMENTS

(1)	Rue Dauphine perimeter property rubble stone masonry wall

(a)	Exterior - ground level and heights

(i)	Foundation

(I)	Thickness		3 pieds

(ii)	Perimeter wall

(I)	Length		62 pieds

(II)	Thickness		2 pieds 6 pouces

(III)	 Height		12 pieds tall (inner courtyard side)

(2)	Rue Royalle perimeter property rubble stone masonry wall [with a central gateway]

(a)	Exterior - ground level

(i)	Perimeter wall

(I)	Length		66 pieds [+ 8 pieds wide gateway opening] + 66 pieds

(II)	Thickness		2 pieds 6 pouces

(3)	Rue d’Estrées perimeter property rubble stone masonry wall

(a)	Exterior - ground level and heights

(i)	Foundation

(I)	Thickness		3 pieds

(ii)	Perimeter wall

(I)	Length		62 pieds

(II)	Thickness		2 pieds 6 pouces

(III)	Height			12 pieds (inner courtyard side)

(V)	LANDSCAPE FEATURES MEASUREMENTS

(1)	Well butting against the south wall of the bakery/wash-house and Rue d’Estrées perimeter property fence

(a)	Exterior - ground level

(i)	Perimeter wall

(I)	Diameter		6 pieds

(2)	Cesspool

(a)	Exterior - ground level

(i)	Perimeter wall

(I) 	External reservoir 	8 pieds x 8 pieds

(II)	Interior vault		12 pieds x 11 pieds

Although this season’s construction was limited, it literally set in stone the final pre-1745 plan of all three buildings and the property’s perimeter fences, the only exceptions being the 1739 modifications to the hospital complex, those of c. 1738-1744 to the storehouse/stables/menagerie, and those of 1744 to the bakery. In contrast, landscape features were still much under development.

Generally, Maps 1726-1 and 1726-1a provide this view of the situation:

(I) 	HOSPITAL (Rue d’Orléans)

Upon completion of the 2 1/2 storey rubble stone masonry hospital complex, the Brothers were to vacate the basement, ground floor and first storey [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion, leaving in place its existing chapel and sacristy, and its remaining rooms (for unspecified uses):

In the northern half of this wing, there was to be established a ground-floor complex [the former Brothers’ residence] with a back-to-back fireplace servicing two rooms defined by the fireplace mass and two wooden partitions. To the south, there was a corridor - developed between the north rooms’ wooden partition wall and a south wooden partition wall - which opened upon the rooms to the north, upon the terrace to the west (by means of an exterior step), to a large U-shaped wooden stairway servicing the first storey, a small stairway beneath the large stairway accessing the basement below (which was also serviced by two exterior basement doors), and upon the sacristy to the south. The sacristy in turn accessed the two-storey chapel, at the corner of Rue d’Orléans and Rue d’Estrées, which had a door centered upon a distinctive partition wall to the west that opened upon a long east ward. If necessary, the chapel could be by-passed by a second doorway leading directly from the ward to the sacristy.

To the west of the chapel, the ground floor and first storey, [Rue d’Orléans] hospital, accessed from the exterior by a central passageway (passage), consisted of long wards serviced by an attached, raised [Rue d’Orléans hospital’s] central pavilion - topped by a bell tower - consisting of latrines, a mortuary with its chapel, an apothecary, an apothecary’s réserve (storeroom), and a guard room:

The ground-floor east and west wards were mirror images of each other, serviced by the arched doorways of the north/south central passageway which divided the hospital complex exactly in half. Each ward also opened upon a room to the north, located in the [Rue d’Orléans hospital’s] central pavilion: latrines (in the eastern portion of the [Rue d’Orléans hospital’s] central pavilion), and the guard’s room with its fireplace (in the western portion). Presumedly, the configuration - two wards (in contrast to the 1726 plans, the profile and description on Maps 1725-8/1725-8a suggests two), latrines (Map 1750-1 clearly illustrates both a discharge outlet for servicing a latrine above and a separate flue for venting cesspool gas from below), guard room, and central passageway (Maps 1725-8/1725-8a and ND-82 (1729) indicate a ward window either in a central passageway door, or in the wall over-looking the chapel) - was the same on the first-storey. Above, in the attic, the floor was finished off with an upper surface of flooring boards or planks.

The ends of the ground floor and first storey flooring joists of the [Rue d’Orléans west hospital] wards were apparently embedded in the perimeter walls. The ends of the attic flooring joists rested upon a wall plate.

The central passageway of the [Rue d’Orléans east and west hospital] wards opened up to the south, by means of an exterior step, upon Rue d’Orléans, and to the north, after passing through the [Rue d’Orléans hospital’s] central pavilion - again by means of an exterior step - upon the terrace. In the [Rue d’Orléans hospital’s] central pavilion, an east door of the central passageway accessed the mortuary with its chapel, and, to the north of it, set against a wooden partition, a stairway which led to a single large upper [Rue d’Orléans hospital] ward (and where perhaps were to be found mirror images of the ground floor mortuary and apothecary room and storeroom but with other but unspecified functions). Likewise, a west door accessed the apothecary with its fireplace [furnace], and to the west of it, closed off from the guard room by a wooden partition, the apothecary’s réserve (storeroom).

From the ground-floor west ward of the hospital a door opened into the new quarters of the Brothers in the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion consisting of basement, ground floor and first storey kitchen, living and sleeping accommodations respectively:

Along the east side of the ground-floor southern portion of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion wing ran a corridor - defined by a wooden partition - that to the south serviced a priest’s room and wooden partitioned-off cabinet, and to the north, a parlour, then a U-shaped stairway set against a wooden partition- which, from the east side, accessed below the basement kitchen/dépense (storeroom) - no doubt indirectly through another basement room to the south of the kitchen fireplace - and above the first storey living quarters for the Brothers - then a doorway with exterior steps to the terrace, and finally the dining room in the extreme north portion of this wing. Behind the stairway, set along its western wooden-partitioned wall, stood a small room, which directly accessed both the parlour and the dining room.

Finally, there was an exterior basement door which also serviced the kitchen.

A number of architectural features included cut stone building corners; chimney caps; a wide, arched central passageway door opening upon the terrace; and a weather-vane upon the bell tower. Window, dormer, and chimney stacks were generally of a balanced design. Structurally - among other things - the roof consisted of a non-braced king post and purlin system and the ends of its ground floor and first storey joists were embedded into the perimeter walls.

(II)	BAKERY/WASH-HOUSE (Rue Royalle and Rue d’Estrées)

In the eastern corner of the property stood the previously completed [Rue Royalle east] rubble stone masonry pavilion housing both the [Rue Royalle/Rue d’Estrées east] wash-house and the [Rue Royalle east] bakery and its existing inner courtyard well:

This pavilion consisted of three ground-floor rooms. Two of these, at the corner of the street, comprised the bakery - the fore-room, entered by a west door facing the inner courtyard - servicing a single oven - and a rear-room partially occupied by the oven mass itself. The third, to the south facing Rue d’Estrées, was the wash-house, likewise with a west doorway opening into the inner courtyard. An unusual shaped fireplace, no doubt for heating water, stood against its north wall.

Presumedly, structurally - among other things - the roof consisted of a non-braced king post and purlin system.

(III)	STOREHOUSE/STABLES/MENAGERIE (Rue Royalle and Rue Dauphine)

In the western corner of the property stood the possibly already completed [Rue Royalle west] rubble stone masonry pavilion housing the storehouse/stables/menagerie:

This pavilion consisted of two ground-floor rooms. One was at the corner of the street, and one was along Rue Royalle accessed from the inner courtyard through an east door. Between the two stood a large door, providing access to the corner room.

Presumedly, structurally - among other things - the roof consisted of a non-braced king post and purlin system.

(IV)	PERIMETER PROPERTY FENCES

Three perimeter property rubble stone masonry fences closed off the remainder of Block 13 from the streets:

Two of the three rubble-stone perimeter property fences - along Rue Dauphine and Rue d’Estrées - and no doubt along Rue Royalle as well, had a sloped rubble stone cap. However, only the Rue Royalle perimeter property fence had a gateway, centered upon the fence, with separate surrounds.

(V)	LANDSCAPE FEATURES - TERRACE AND INNER COURTYARD

A terrace and a lower inner courtyard were still to be developed in detail. The well was already in place:

Beneath both the terrace and the hospital’s interior latrines, an underground cesspool was to be constructed. From the terrace, ramps at either end - servicing the doorways of both the eastern and western pavilions of the hospital complex - and a pair in the centre, opposite the hospital’s exterior central passageway doorway, were to lead to the inner courtyard. In the inner courtyard stood one well, already built against where the Rue d’Estrées perimeter property fence and south wall of the wash-house met.

YEAR: 1727

Set aside was 26,000 livres to continue work on the lhopital du Roy, which amount was entirely spent by November. By August 15, the masonry was up, and the raising of the roof extended at least into September. Perhaps too, the bell tower was constructed this year in concert with the work on the roof, although conceivably it may have been raised at a subsequent time.

Unfortunately, roofing slates were unavailable and the building now stood with a [provisional] board roof. Additional plans, however, now called for the installation of the floors that winter, and the placement of roofing slates, the interior [wall] crépis or enduit roughcasting - subsequently delayed until the slates were placed for the fear of rain-water damage - and other building dispositions by a proposed June 1728 completion date.

An undated Governor St. Ovide memoir, circa December 1727 or earlier [given that it discussed Chief Engineer Verrier continuing the lengthening of the hospital], described several works underway, including that of the hospital. He was concerned that construction on the building had re-commenced following the same pattern as that already built despite [some] fundamental defects. Accordingly, Engineer Verrier would build the [roof] ridge of the new [Rue d’Orléans west hospital] addition 2 full pieds higher and its bottom [or ground floor] floor 1 1/2 pieds lower than that of the older [Rue d’Orléans east hospital] section. He did this because [although] the first [storey] floor [of the earlier portion] was too low, he [still wanted] to bring [its] large ward level with the [new first storey] large ward.

The hospital, by its location, was always too humid. To have achieved a little more dryness, it would have been necessary to raise the ground floor at least 2 good pieds [which no longer was an option]. Of interest, on at least three other occasions - in 1732, 1736, and in 1750 - humidity was again identified as a problem in the hospital.

Unfortunately as well, Verrier had made the arrière-voussures of the new addition from birch planks rather than of flat stone. Consequently, he had sacrificed solidness and durability. As well, Verrier had been forced to demolish 84 running pieds of masonry, of 5-6 pieds in height, of a wall founded 2 pieds higher than the level of the older [Rue d’Orléans east hospital] building’s floor. Furthermore, he had been obliged to rebuild 2 pieds of the perimeter foundation of the new [Rue Royalle] pavilion. Apparently, heavy rains had washed away 2 toise 1 pieds 4 pouces cubic of the two gable ends of the masonry [Rue Royalle east] pavilion that housed the [Rue Royalle east] bakery, along with 1 toise 4 pieds 8 pouces cubic of the masonry sheep fold and another 5 pieds 4 pouces cubic of an area near its door.

Maps 1726-1 and 1726-1a, in yellow, illustrates - in plan and in elevation - the final configurations proposed for the [Rue d’Orléans] hospital and [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion for this and upcoming years. No doubt these plans were more accurate in detail than maps 1725-8 and 1725-8a for the construction period 1723-1727 inclusive, but, nevertheless, should be read along with them, as well as with maps 1750-1 and 1752-10 where appropriate to the 1744-1745 re-construction date.

Map 1729-3 illustrates the completed 8-sided bell tower (as well as some main roof features) of the l’Hopital du Roy with exceptional clarity. Secured approximately 10 1/2 pieds below the main roof ridge, it rose c. 32 1/2 pieds above (not including its weather-vane), was of wood construction, employed some internal structural iron-work, and was to have a slate roof on a [board] sheathing. Of interest, it was without a clock, but topped by a weather-vane.

YEAR: 1728

For this year, an additional 28,000 livres was thought necessary to continue work on the hospital.

Roofing slates from France were again unavailable on Isle Royalle, and the building stood another year with a provisional board roof. Consequently, the planned interior [wall] crépis or crépissage roughcasting, as well as joinery work and other interior dispositions, were put on hold, for fear of water damage. Highlighting this concern was the fact that the existing board roof had already undergone a repair.

Notwithstanding these general assertions, Ganet, the fortification contractor, had actually completed some hospital projects. For example, there was the minor work - valued at 166 livres 16 sols - that he undertook in the older [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion and Rue d’Orléans east hospital ward] section - i.e in the chapel and in the lodgings where the Brother's were still residing - and in the inner courtyard. They included the following:

(1)	Repairing the keepers, springs, etc., of several locks and replacing several keys (lodgings) - 20 livres

(2)	Floor (or board) repairs (lodgings) - 6 livres

(3)	Replacing a door latch (lodgings) - 4 livres

(4)	Repairing a door (lodgings) - 8 livres

(5)	Replacing 183 7x8 pouce glass window [panes] (lodgings) - 109 livres 16 sols

(6)	A lamp bracket (chapel) - 4 livres

(7)	Replacing the [inner court]yard gate key and chain for a serrure a bosse - 3 livres

(8)	A shovel and fire tongs for the stove (unspecified location) - 8 livres

(9)	Cleaning and [replacing] the [glued glazing] paper of [window] panes (unspecified location) - 4 livres

More substantial were other costs totalling 32,015 livres, for which Ganet received payment, but for work which may have occurred earlier than 1728:

(1)	[Excavation] work (1,206 livres)

(2)	Masonry work (1,320 livres)

(3)	Grey [sand stone] (4,789 livres)

(4)	Cut white [lime stone] (2, 709 livres)

(5)	Hewn timber (8,139 livres)

(6)	[Softwood] floors (7,740 livres)

(7)	Partitions (1,100 livres)

(8)	Hardwood floors (927 livres)

(9)	Roof shingles (1,728 livres)

(10)	Pavé (2,357 livres).

YEAR: 1729

Another 20,000 livres was requested for this year to continue and finish the Lhopital du Roy de Louisbourg deservi par les freres de la Charité. Unfortunately, once again the roofing slates failed to arrive from France, and the building remained in the same state as the previous year. Meanwhile, Ganet, the fortification contractor, undertook some projects (25 livres):

(1)	Replacing one vertical and two horizontal [inner court]yard door pieces - 10 livres

(2)	Repairing and replacing the bolts, keepers, and staples of several locks, (unspecified location) - 15 livres

Because Map ND-82 (1729) is problematic in several aspects - e.g. three of the street names are labelled incorrectly, fireplaces in plan and chimney stacks in elevation often do not coincide, and the profile line and its resulting profile do not match - the danger is that this plan may conceptualize rather than illustrate accurately portions of Block 13. Thus, for example, although it depicted the second inner courtyard well for the first time, it positioned it incorrectly. As many later plans - such as maps 1731-3 and 1745-4 would clearly demonstrate - it was not constructed against the north wall of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion as shown, but rather away from it further to the north. However, to illustrate the well against the Rue d’Estrées perimeter property fence - as again many plans confirm - only makes sense, for a place to attach the hardware for lowering and raising the well bucket (as map ND-82 illustrates).

Map ND-82 (1729), also for the first time, illustrated a possible garden and yard layout for the inner courtyard. In this instance, other plans appear to indicate such a development to be too elaborate:

(1)	There were two separate gardens, one immediately to the east and one immediately to the west of the dual central terrace ramps, separated by a central walk-way - enclosed to the north by the masonry face of the terrace’s retaining wall, and to the east, west, and south by piquet garden fences standing less tall than the perimeter property fences.

(2)	Each garden had a braced gate that allowed access from a central walkway that extended north from the central terrace ramps to an open area in front of the central gate in the Rue Royalle perimeter property fence.

(3)	There were two piquet fenced-in open yards to the east and to the west of the gardens, separated by a side walkway that accessed these yards, the terrace to the south (by means of their corner steps), and the open area to the north in front of the central gate in the Rue Royalle perimeter property fence.

(a)	One was placed against the Rue d’Estrées perimeter property fence to the east and against the wall of the masonry [Rue Royalle east] pavilion, housing the bakery/wash-house, to the north, with [gates] opening upon the east side walkway and upon an open area in front of the [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion.

(b)	One was placed against the Rue Dauphine perimeter property fence to the west and against the wall of the masonry [Rue Royalle west] pavilion, housing the storehouse/stables/menagerie, to the north, with [gates] opening upon the west side walkway and upon an open area in front of the [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion.

Other plans suggest a less elaborate development. For example, a more simple variation might have been two smaller, fenced, gardens, located further north, developed against the Rue Royalle perimeter wall, with a wider, central walkway. Defined by the garden and other fences extending southward, the walkway perhaps extended onto the terrace itself. In the inner courtyard, around and behind the garden, other, less formalized yards may then have existed, to meet other requirements (1733-7, 1734-4, 1742-2, 1744-5, 1745-11, 1745-19).

Finally, Map ND-82 (1729) both confirmed and conflicted with aspects of Maps 1726-1/1726-1a:

(1)	Confirmed:

(a)	The hospital complex had cut stone quoins for building corners

(b)	The hospital complex had a simple, non-quoin stone, window surround design

(c)	The hospital complex had dormers

(d)	The roof of the [Rue d’Orléans west] hospital ward consisted of a non-braced king post and purlin system

(e)	The ends of the ground floor and first storey joists of the [Rue d’Orléans west] hospital ward were embedded into the perimeter walls.

(f)	The floor plan for the hospital complex, [Rue Royalle east] pavilion housing the bakery/wash-house, and the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie

(g)	The rubble stone perimeter property fence had a sloped rubble stone cap

(2)	Differed:

(a)	The hospital complex dormers differed in numbers and locations

(b)	The terrace central passageway door entry of the [Rue d’Orléans] hospital central pavilion had cut stone quoin surrounds

(c)	The terrace central passageway door entry of the [Rue d’Orléans] hospital central pavilion was not arched

(d)	The ground floor guard room in the [Rue d’Orléans hospital] central pavilion did not have a fireplace

(e)	The bell tower reassembled the one illustrated on map 1729-3

(f)	The roof of the [Rue d’Orléans west hospital] ward had a raised collar beam without a surface of attic flooring

(g)	The ground floor parlour of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion did not have a fireplace

(h)	The hospital complex had brick chimney stacks with cut stone caps and strings

YEAR: 1730

On April 17, Ganet, the fortification contractor, agreed to a new lhopital du Roy contract. The detailed estimate that year for completing the hospital amounted to 21,909 livres 12 sols:

(1)	Roofing slates (384 square toises @ 30 livres - 11,520 livres)

(2)	Table lead [flashing] for the roof ridge and valleys, etc. (8,000 livres of weight @ 8 sols - 3,200 livres)

(3)	Interior [wall roughcasting] enduit (415 square toises @ 6 livres- 2,490 livres)

(4)	Vertical wall boarding for the upper [hospital] ward and [central] passageway (40 square toises @ 14 livres - 560 livres)

(5)	[Fir] cornices above doors (40 square pieds @ 2 livres 10 sols - 100 livres)

(6)	[Oak or birch] door architraves (6 square toises @ 52 livres - 312 livres)

(7)	[1-pouce thick pine] doors, [with oak embôitures at both ends] (25 square toises @ 25 livres - 625 livres)

(8)	[Window] frames (19 square toises @ 45 livres - 855 livres)

(9)	Glass [window panes] (648 square pieds @ 24 sols - 777 livres 12 sols)

(10)	Iron (1000 livres of weight @ 5 sols 6 deniers - 275 livres)

(11)	Window strap hinges [of 1 pieds of length, with their pintles] (100 pairs @ 4 livres - 400 livres)

(12)	Door strap hinges [of 2 pieds of length, with their pintles] (40 pairs @ 8 livres - 320 livres)

(13)	[Sash locking] bolts (150 @ 2 livres 10 sols - 375 livres)

(14)	Keepers and fasteners (100 livres)

This accounting revealed several facts, that:

(1)	Casement style windows (strap hinges and sash locking bolts) were envisioned

(2)	Interior mortar renderings were not universal (vertical wall boarding)

(3)	A number of doorways were probably better finished than others (fir cornices and oak or birch architraves)

In May, 12,000 livres was authorized for completing the hospital’s joinery work and roof work. Knowing in June that Ganet had left Nantes for Isle Royalle with 100,000 slates and several new workers, and that 50,000 additional slates were also to arrive, Chief Engineer Verrier planned to compete the hospital’s roof before winter set in.

After a 45 day crossing, Ganet arrived from France upon a King's ship together with the following:

(1)	Black slate (50,000)

(2)	Table lead (5,000 livres of weight or 2 1/2 tonneaux)

(3)	Flat iron (4,800 livres of weight or 2 2/5th tonneaux)

(4)	Steel (1/4 tonneaux)

(5)	Sheet-metal (400 livres of weight or 1/5th tonneaux)

(6)	Glass [window panes] (5 casses or 1/2 tonneau)

In July, the roof of the new [Rue d’Orléans west] section of the hospital was slate covered, and, following that, its windows were glazed, and in its wards, wood burning stoves were mounted and beds were placed. In contrast, the roof of the original [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion and portion of the Rue d’Orléans east hospital ward] remained as before, shingled covered and still requiring slates.

For his work this year Ganet submitted a hospital bill for 13,987 livres 18 sols. The building this year now finally completed, the Brothers no doubt moved over to their new quarters in the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion.

YEAR: 1731

Chief Engineer Verrier reported that he needed 9,909 livres to complete the hospital in its entirety. However, in a November description of work done that year, he did not mention the hospital. Map 1731-3, for the first time, illustrated the second inner courtyard well in its proper position as against the perimeter fence, near the north wall of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion.

YEAR: 1732

Ganet, the fortification contractor, undertook some minor projects (175 livres 6 sols) that included the following:

(1)	[Glass window] pane replacements throughout the entire building (257 @ 12 sols each - 154 livres 4 sols)

(2)	Lock replacement in the kitchen (1 @ 8 livres - 8 livres)

(3)	Iron scrappers for cleaning the floors (4 @ 20 sols each - 4 livres)

(4)	One keeper, ring, bolt and iron latch catch (2 livres 10 sols)

(5)	Cleaning and [replacing] the [glued glazing] paper of [window] panes (5 livres)

YEAR: 1733

A January inventory counted 3 iron stoves for the wards but the iron plaques for [brick] stoves were apparently all consumed. The wells in the yard were equipped with iron gallows, and the [terrace] had a single iron balustrade.

Ganet, the fortification contractor, undertook some minor projects (80 livres 8 sols) that included the following:

(1) 	[Glass window] pane replacements (19 @ 12 sols each - 11 livres 8 sols)

(2)	Key replacements (4 @ 20 sols - 4 livres)

(3)	Large table fork for the kitchen

(4)	The cleaning of [probably at least two] wells and their purification with gravel (24 livres)

For the year, for unspecified work, Ganet would submit a final hospital bill for 2,579 livres 5 sols 11 deniers.

Map 1733-7 suggested that the Rue d’Orléans central entrance way was either finished more elaborately than the terrace doorway, or else, had a substantial entrance step. Although maps 1725-8/1727-8a had envisioned a classical doorway at the terrace entry, maps 1726-1/1726-1a called for a more simple design. None illustrated the type of Rue d’Orléans doorway that might have been installed, although maps 1726-1/1726-1a - and, later, map 1750-1 suggest a significant entrance step. Only, in c. 1764, did Gibson Clough who had been at Louisbourg in 1759, tantalizingly illustrate otherwise, that there existed a central portico.

YEAR: 1734

Ganet, the fortification contractor, undertook some minor projects (28 livres 4 sols) on the l'hopital royal de la marine that included the following:

(1)	The replacement of keys (3 @ 1 livres each - 3 livres)

(2)	The replacement of a [door] latch (1 @ 4 livres - 4 livres)

(3)	The repair of locks (2 @ 3 livres together - 3 livres)

An official survey of that year laid down the final measurements for Block Thirteen (which would total 41,760 square pieds) as follows:

(1)	40 toises [240 pieds] along Rue Royalle and d’Orléans

(2)	29 toises [174 pieds] along Rue d’Estrées and Dauphine.

Rue de l’Hôpital was declared altered, where it met Rue d’Orléans in front of the hospital’s central door, to form a quarter circle, to facilitate traffic at that spot.

YEAR: 1735

Ganet, the fortification contractor, undertook some minor repairs and projects (145 livres 18 sols) as follows:

(1)	[Window] pane replacements (109 @ 12 sols each - 65 livres 8 sols)

(2)	Lock repairs (1 @ 1 livres 10 sols - 1 livres 10 sols)

(3)	The repair of the slate roof (44 livres):

(a)	1 roofer (8 days @ 3 livres - 24 livres)

(b)	1 helper (8 days at @ 1 livre - 8 livres)

(c)	Slates (100 @ 6 livres - 6 livres)

(d)	[Flashing] cement (6 livres)

YEAR: 1736

The hospital’s prison - a guard but not a lock-up for sick prisoners was specifically identified in the plans of 1726 - was a small room [perhaps the neighbouring apothecary’s réserve (storeroom)] - generally used to store items of little value - in the ground floor [Rue d’Orléans Hospital] central pavilion, closed off by a door. The room had a small window - to the left upon entering - which overlooked the yard [terrace]. This window, built with [imported] cut stone surrounds and a plank sill, was grilled with 3 iron bars set in a frame.

Another window was located on the yard [terrace?] side nearest to the [Rue d’Estrées/Rue d’Orléans] chapel choir. Its window was secured from within by a sash-bolt. Behind the choir was another window, placed near the corner of the building. It too could be opened, though [perhaps generally] sealed with glued paper. Of interest, the ceiling of the chapel rose two storeys high, allowing windows in the upper ward to overlook it. Possibly too, below, in the chapel door and partition, windows also existed, looking into the lower ward.

On October 11, 1736, Verrier produced a final description and accounting of the completed lhopital du Roy for the work that Ganet, the fortification contractor, had undertaken since February 25, 1725. Although the toise is currently missing from the historical record, the cost is not: 112,402 livres 16 sols 4 deniers. Not included was the work of Isabeau, the first fortification contractor.

YEAR: 1737

The Brothers of Charity asked for two alterations, that there be established a [hospital] ward for sick officers and that there be built a new kitchen to replace the existing one in the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion basement which suffered, during the winter months, from water and [smoke] fumes. They also complained that De Mézy, the Commissaire-Ordonnateur - who didn’t want the Brothers to assume that they owned the hospital - had Chief Engineer Verrier remove the inscription “Charitas” which they had placed above one of the hospital’s doors

Verrier proposed that the officers’ [hospital] ward - for those of the garrison and of visiting King’s vessels - be established in the former [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion of the hospital, whose [original layout] was no longer suitable, and thus without present use. This [hospital] ward would consist of 5 rooms, each with a fire[place], once the old chimneys were removed and re-set in a new interior bearing wall. Placed in the middle of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion, this wall would strengthen the building while providing a useful layout.

In addition, the shingled roof of this section of the hospital - covering the said [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion as well as the [Rue d’Estrées/Rue d’Orléans] chapel - was now old, rotten, and nearly useless. It was proposed to replace it with slate.

YEAR: 1738

Chief Engineer Verrier submitted the plans and estimations for several works that included the approved alteration of the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion to a [hospital] ward for sick officers and the replacement of the acknowledged 18-year old shingled roof with slates. Unfortunately, nothing happened that year because of a shortage of materials that included bricks for the fireplaces/or chimneys.

That year too, the Brothers of Charity had also asked for a henhouse in the [inner court] yard to replace the one in the basement of the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion] which Verrier maintained ought to have been dedicated as a work area. For 1,000 livres, Verrier proposed to raise one in 1739. Apparently, between then and 1745, one was built, attached to the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie.

Maps 1745-4 and 1750-1 (which also included a sheep fold) illustrate the henhouse, though with different door and window arrangements.

YEAR: 1739

By August, the 5 rooms for the officers’ [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion hospital] ward were reported completed (although they remained unfurnished that year and into the next). On October 15, Engineer Verrier submitted a [detailed - now missing] toisé describing the work that Muiron, the new fortification contractor (1737-1745), had apparently undertaken, that year for the said alterations. The project cost the King 8,687 livres 10 sols 1 deniers.

In the northern half of this wing - according to Map 1750-1, there was now a ward consisting of 5 rooms for sick officers. Two were clearly located on the ground-floor with the other 3 possibly on the first storey above, accessed by a large U-shaped wooden stairway. Beneath this stair, a small stairway serviced the basement below. There was one ground-floor dual fireplace complex, located in the officers' ward, serving two rooms, defined by the fireplace mass and one wooden partition. Above, in the first-storey above, there would have been additional fireplaces (or stoves) in the remaining three officers' rooms.

To the south of the ground-floor ward, there was a corridor - defined by the northern masonry bearing wall that carried the officers' fireplaces and by a southern wooden partition - which open upon the 2 officers' rooms to the north, upon the terrace to the west, and upon the sacristy to the south.

This year, the [exterior] crépissage roughcasting for the entire hospital building was now finished, and, possibly in some places - perhaps in the [Rue d’Orléans east hospital] ward - for at the least a second time. There was no mention of a shingled roof replaced in slate, and indeed, it was still shingled as late as 1749.

YEAR: 1740

At the beginning of the new year, Muiron, the fortification contractor, requested slate for repairing the hospital’s roof [and possibly its bell tower as well, if not this year, then certainly some other pre-1745 year].

The hospital did not have the facilities to temporarily hold the insane, and so it was suggested that one of the 5 new officers’ rooms be used for this purpose - since its windows were grilled [with bars] - until the person could be sent to France. Better, however, would be if two small rooms in masonry might be built in the hospital’s tower as a temporary holding place.

YEAR: 1744

Muiron, the fortification contractor, extended an underground hospital [latrine] drain [across the terrace and inner courtyard] so that it now emptied into the Grand Étang.

In addition he altered the layout of the [Rue Royalle east pavilion] housing the bakery, apparently to meet the requirements of making bread for the [English] prisoners of war held at Louisbourg:

(1)	Punched an opening in its Rue Royalle bakery wall for a doorway with its door and associated hardware (Note: Map 1750-1 does not illustrates this door. It shows the only new Rue Royalle doorway to be in the storehouse of the [Rue Royalle west] pavilion then housing the cave/storehouse/horse-cow sheds/ henhouse-sheep fold)

(a)	Creation of the doorway opening (22 livres)

(b)	Interior and exterior hewn timber door frame (12 cubic pieds @ 20 sols - 12 livres)

(c)	[Single] leaf 2-pouce thick plank batten door with bars and diagonal pieces (7 livres)

(2)	Placed an interior separation partition - constructed of Boston boards - (@ 5 livres 12 sols per square toise)

(3)	Built a stairway - constructed of Boston boards - to the attic to serve the Brothers of Charity - (@ 5 livres 12 sols per square toise)

(4)	Strap hinges:

(a)	Of 2 pieds of length, with their pintles (2 pairs @ 6 livres 8 sols per pair - 12 livres 16 sols)

(b)	Of 1 pied 6 pouces of length, with their pintles (2 pairs @ 4 livres 16 sols per pair - 9 livres 12 sols)

(5)	Common locks (2 @ 5 livres each - 10 livres)

(6)	A [door] latch (1 @ 3 livres 4 sols - 3 livres 4 sols)

(7)	[Repointed] the oven's furnace and hearth:

(a)	1/2 barrel of lime (4 livres)

(b)	Sand (10 sols)

(8)	Labour:

(a)	Masons (2 days @ 3 livres per day - 6 livres)

(b)	Helpers (1 day @ 1 livre 10 sols - 1 livre 10 sols)

Once again, Chief Engineer Verrier turned his attention to the problem with the existing kitchen - that it was too small to serve the number of sick people, that 5 1/2 pieds of height from floor to ceiling joists was not sufficient, and that the room was too dark, being illuminated by only cellar lights (abatjours). Since he and Bigot, the Commissaire-Ordonnateur, agreed that a new [masonry] kitchen re-located in the [inner court]yard [attached to the existing Rue d’Orléans/Rue Dauphine west left aisle/left pavilion] would be [desirably] isolated, and also brighter inside, he had produced a 3,959 livres estimate and a plan (Map 1745-4), with construction proposed for 1745. Nothing, however, came of this proposal, and the plans were permanently shelved.

Map 1745-4 did, however, illustrate the existing 1744 building in several interesting ways, some of which were problematic:

(1)	It did not show any [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion kitchen cellar lights facing the inner courtyard

(a)	Given that the exterior door and window as illustrated on maps 1726-1/1726-1a in its north wall still existed at this time, then the 1744 kitchen abatjours (lights) may have been set in its western Rue Dauphine wall.

(2)	It illustrated a [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion terrace doorway with a 4 over 4 light transom set above the door

(3)	It illustrated the roof of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion without dormers

(a)	While maps 1726-1/1726-1a indicate at least one dormer, map 1750-1 shows none, while map 1752-10 shows two small ones

(b)	In 1749, dormer sashes were constructed

(4)	The chimney stacks of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion were of brick construction, with cut stone caps

(a)	In 1738 brick fireplaces and/or chimney stacks were envisioned

(5)	The roof of the [Rue d’Orléans west hospital] ward used purlins and a non-braced, [presumed king post roof system, set] upon a raised collar beam

(a)	Maps 1725-8/1725-8a illustrates a braced king post system, with purlins, set upon a collar beam either embedded in the perimeter walls or placed upon a wall plate

(b)	Maps 1726-1/1726-1a illustrates a non-braced king post system, with purlins, placed upon a wall plate

(c)	Map ND-82 (1729) illustrates a non-braced king post system, with purlins, set upon a raised collar beam

(d)	Map 1750-1 illustrates a braced king post system, with purlins, set upon a raised collar beam

(e)	Between 1721 and 1724, roof king posts, wall plates, braces, collar beams, and purlins were installed in the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion

(6)	The ends of the ground floor and first storey flooring joists of the [Rue d’Orléans west hospital] were embedded in the perimeter walls

(a)	Maps 1726-1/1726-1a, ND-82 (1729), and 1750-1 show embedded joist ends

(7)	The ends of the attic flooring joists of the [Rue d’Orléans west hospital] ward were neither embedded in the perimeter walls nor placed upon a wall plate. Instead, structural iron hardware was inserted into a raised collar beam to strengthen it and to allow for an attic floor finished off with an upper surface of flooring boards or planks.

(a)	Map 1726-1/1726-1a shows a wall plate with a upper surface of attic flooring

(b)	Map Nd-82 (1729) shows a raised collar beam without a surface of attic flooring

(c)	Map 1750-1 shows a raised collar beam without a surface of attic flooring

(d)	The modifications of 1727 may reflect the change to a raised collar beam arrangement

(8)	The rubble stone Rue Dauphine perimeter fence had a sloped cap of another material than rubble stone

(a)	While map 1726-1/1726-1a illustrated a rubble stone cap, both maps 1745-4 and 1750-1 illustrate another material

(9)	The roof finials of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion were apparently lead covered

(10)	The corner terrace steps were of cut stone

(a)	In 1749, the cut stone steps were re-established

(11)	The henhouse has been added to the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie

(a)	The henhouse was mentioned in 1738

(b)	The masonry henhouse addition was illustrated on map 1750-1, though with different door and window arrangements

(12)	The [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion and the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie utilized cut stone surrounds for their doors, windows, and building corners

(a)	Cut stone surrounds were described in 1726 and 1736

YEAR: 1745

In 1745, prior to the siege or earlier, an accounting noted that the [Rue d’Orléans west] hospital and its bell tower required some minor slate [and possibly associated table lead] repairs (1,000 slates).

At this time, according to an English post siege account, the block in 1745 measured 20 rods (330 feet - or 90 paces according to another 1745 English account) along [Rue Royalle and d’Orléans - and hence, along the hospital building as well]. Built of 2-foot thick lime [mortared] [rubble] stone, the Hospital Du Roy or Royal Hospital stood two stories high - with two large [hospital wards] below and similar [two large hospital wards] above - with a bell [tower] with its bell in the middle. Its numerous large sash windows were [grilled with bars] described as iron grates.

At its [Rue d’Orléans east] and [Rue d’Orléans west] ends [of the hospital] stood a large dwelling house. From each of them, running north, along [along Rue d’Estrées and Dauphine respectively] was a 12 foot high, 4-foot thick, 13 rod [214.5 foot] long [perimeter property] wall that met the [Rue Royalle perimeter property] wall, enclosing off about an acre of land used as a garden. At the [Rue Royalle east] and [Rue Royalle west] corner of this wall stood large kitchens [buildings] for cooking and brewing. There were also two wells, an arched [entry] gate with a cross, and two [obvious] sun dials, and a fine garden.

During the siege, the [Rue d’Orléans west hospital] ward suffered some minor damage to its slate roof.

In June, the French agreed that the [Rue d’Orléans] hospital [wards] be divided, between French and English use, which later also included the sharing of the mortuary (dead house). In September, the [Rue d’Estrées/Rue d’Orléans] hospital chapel was considered for use as a barracks. Throughout this period, its [Rue d’Estrées/Rue d’Orléans] chapel was a place for English preaching and prayers. As well, prayers were held in the Rue d’Orléans hospital wards] and, at least once, in the hospital’s upper [Rue d’Orléans] entry. In September, a nauseous smell was so strong in the hospital as to hinder the visiting activities of the chaplain.

YEAR: 1745 - 1748

Considered by them to be a handsome building, the English forces for four years used the hospital first - as they characterized it - as a hospital barracks, then as a hospital now used as a barracks. During this period, they not only undertook repairs and built bunks for the English militia, but, generally, also removed all the hardware, including the iron balustrade in the yard [terrace], and the [Rue d’Estrées/Rue d’Orléans] chapel’s iron [window] grills and grilled door. They also built an upper floor in the [Rue d’Estrées/Rue d’Orléans] chapel by extending the upper [Rue d’Orléans east hospital] ward into that area, and installed 4 large chimneys in the [Rue d’Orléans east hospital] ward, next to the [Rue d’Estrées/Rue d’Orléans] chapel, to serve both floors levels

YEAR: 1749

Upon the return of Louisbourg, the hospital had to be set up once again. Among the items deemed necessary were stoves for the wards, and assorted appointments for the wards, kitchens, wash-house, bakery, Brothers’ rooms, Officers’ rooms [ward], and [main] entry door.

On July 23, Coeuret, the new fortification contractor (c. 1749-1753 if not later as well), was tasked with repairing the hospital complex at a cost of 6,259 livres 6 sols 10 deniers, covering the following categories:

(Note: The proposed re-construction date is 1744-1745. Only the more useful dimensions, quantities, and specific prices have been extracted - Please consult the transcribed "Estat des Ouvrages de reparations" for complete details)

(1)	Exterior Masonry Walls

(a)	Crépissage à pierre apparente roughcasting of the [walls] of the [Rue Royalle west] pavilion housing the horse-cow sheds (@ 45 sols per square toise)

(2)	Interior Masonry Walls

(a)	Scraped the walls of the [Rue d’Orléans hospital] ward, to clean them

(b)	Scraped the walls of the [Rue Dauphine west] lodgings of the Brothers, to clean them

(c)	Lime and sand mortar enduit roughcasting of a portion of the [walls] of the [Rue d’Estrées/Rue d’Orléans] chapel and [Rue d’Estrées] sacristy, [followed by a] a 2-coat lime whitewash for the [Rue d’Estrées/Rue d’Orléans] chapel (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(d)	[Lime and sand mortar] enduit roughcasting of a portion of the [walls] of the [Rue d’Orléans east] small [hospital] ward, [followed by a 2-coat lime whitewash] (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(e)	[Lime and sand mortar enduit roughcasting] of the [walls] of the [Rue d’Orléans west] large [hospital ward] adjoining the lodgings of the Brothers, [followed] by a 2-coat lime whitewash (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(f)	[Lime and sand mortar enduit roughcasting] of the [walls] of the [Rue d’Orléans] upper [hospital wards], [followed] by a 2-coat lime whitewash (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(g)	[Lime and sand mortar enduit roughcasting] of the [walls] of the [Rue d’Orléans/Rue Dauphine] Brothers lodging, refectory (dining room), office, and kitchen, [followed] by a 2-coat lime whitewash to the lodgings of the Brothers (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(h)	[Lime and sand mortar enduit roughcasting] of the [walls] of the 2 corridor passages which separate the [Rue d’Orléans hospital] wards, [followed] by a 2-coat lime whitewash (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(i)	[Lime and sand mortar enduit roughcasting] of the [walls] of the [Rue d’Orléans hospital] apothecary (Enduit @ 5 livres 10 sols per square toise)

(j)	[Lime and sand mortar enduit roughcasting] of the [walls] of [unspecified] stairs, [followed] by a 2-coat lime whitewash, and the [lime and sand mortar enduit roughcasting] of other small lodgings (Enduit @ 5 livres 10 sols per square toise; Whitewash @ 15 sols per square toise)

(k)	A 2-coat lime whitewash in unspecified areas (Whitewash @ 15 sols per square toise)

(3)	Roof

(a)	Shingle repairs to the [Rue Royalle west] pavilion housing the storehouse/horse-cow sheds/henhouse-sheep fold (@ 15 livres per square toise)

(b)	Shingle repairs to the [Rue Royalle east] pavilion housing the bakery/wash-house (@ 15 livres per square toise)

(c)	Slate repairs from the [Rue d’Orléans hospital] central pavilion to and including the [Rue d’Orléans/Rue Dauphine] Brother’s lodging (12 square toises @ 5 livres per square toise, which includes the nails - 60 livres)

(d)	Shingle repairs to the long side of the roof of the said [Rue d’Estrées/Rue d’Orléans east right aisle/right pavilion] which faces the bakery (3 square toises 2 pieds @ 15 livres per square toise- 50 livres)

(e)	[Slate] repairs from, and including, the [Rue d’Orléans hospital] central pavilion to the [Rue d’Estrées] officers’ rooms (25 square toises 3 pieds @ 5 livres per square toise, which includes the nails - 12 livres)

(4)	Floors and Flooring

(a)	Demolition of the floor [planks] and 7 floor beams sealed [into the walls] which the English had constructed when they had extended the upper floor [Rue d’Orléans hospital] ward over the [Rue d’Estrées/Rue d’Orléans] chapel

(b)	Repair of the [Rue d’Estrées/Rue d’Orléans] chapel ground floor, re-using 6 re-used flooring beams taken from the demolished English extended upper [Rue d’Orléans hospital] ward floor over the [Rue d’Estrées/Rue d’Orléans] chapel, and replacement of [flooring planks] and furnishing of nails

(c)	Replacement of a portion of the [Rue d’Orléans hospital] ward floor with [flooring planks]

(d)	Replacement of a portion of the upper and lower floors [of the hospital], re-using the flooring planks from the demolished English extended upper [Rue d’Orléans hospital] ward floor over the [Rue d’Estrées/Rue d’Orléans] chapel

(e)	Sealing of the English beam holes with masonry and other masonry repairs in the [Rue d’Estrées/Rue d’Orléans] chapel

(f)	[Construction] of a double floor under the armoire in the [Rue d’Orléans hospital] apothecary

(g)	[Construction] of 4 armoire doors (and shelving] for the linen room (@ 12 livres per square toise)

(5)	Door and Doorways

(a)	Arched [doorway] in the [Rue Royalle west] pavilion housing the storehouse/horse-cow shed between the storehouse and the horse-cow shed blocked off [with rubble stone masonry] (3 toises 3 pieds 6 pouces @ 120 livres per cubic toise - 70 livres)

(b)	Oak work for 2 architraves for the communication doors of the Brothers lodging

(c)	Oak work for architraves for the [doors] of the large [hospital] wards

(d)	[Construction] of the large 2-leaf entry [Rue d’Orléans hospital] door, with embôitures at either end, made of tongued and grooved [vertical 2-pouce thick] planks (1 toise square 9 pouces @ 32 livres per square toise - 36 livres)

(e)	Provisional repair [of the large 2-leaf] terrace [embôiture] door

(f)	Provisional repair of the lower [inner court]yard (basse cour) door

(g)	[Construction] 2 storehouse doors [with embôitures at either end, made of tongued and grooved vertical 2-pouce thick planks] in the [Rue Royalle west] pavilion housing the storehouse (@ 32 livres per square toise)

(h)	[Construction] of a 1-pouce thick door, with embôitures at either end, made of tongued and grooved [vertical] local boards, for the [Rue Dauphine] refectory (dining room) (@ 22 livres per square toise)

(i)	[Construction] of a 1-pouce thick door, with embôitures at either end, made of tongued and grooved [vertical] local boards, for the [Rue Dauphine kitchen] dépense (storeroom (@ 22 livres per square toise)

(j)	[Construction] of a [2] 1-pouce thick doors, with embôitures at either end, made of tongued and grooved [vertical] local boards, for the two [Rue d’Orléans hospital] wards which communicate with the lodgings of the Bothers (@ 22 livres per square toise)

(k)	[Construction] of a [2] 1-pouce thick doors, with embôitures at either end, made of tongued and grooved [vertical] local boards, for the [Rue Dauphine] rooms above the refectory (dining room) (@ 22 livres per square toise)

(l)	[Construction] of a framed, panelled door for the [Rue Dauphine] parlour door in the corridor (2 pieds 9 pouces @ 30 livres per square toise- 13 livres 15 sols)

(m) 	[Construction] of 2 half-glass/half-panelled doors for the [Rue Dauphine] cabinet in the Brothers rooms above the refectory (dining room) (@ 34 livres per square toise)

(n) 	[Construction] of a half-glass/half-panelled door for the [Rue d’Estrées/Rue d’Orléans] chapel which communicates with the [Rue d’Orléans hospital] wards (@ 34 livres per square toise)

(6)	Windows

(a)	[Construction] of a window sash, with its dormant frame, for the [Rue Dauphine] refectory (dining room) (@ 40 livres per square toise)

(b)	[Construction] of a window sash, with its dormant frame, for the [Rue Royalle east] pavilion housing the wash-house (@ 40 livres per square toise)

(c)	[Construction] of 16 dormer sashes, [with their dormant frames], (@ 40 livres per square toise)

(d)	[Construction] of 3 cellar soupiraux (ventilators) sashes, [with their dormant frames], for the [Rue Dauphine] kitchen (@ 40 livres per square toise)

(e)	[Construction] of 2 Venetian sashes, [with their dormant frames], for the 2 [Rue d’Orléans] upper storey [hospital] ward windows which overlook the [Rue d’Estrées/Rue d’Orléans] chapel (@ 40 livres per square toise)

(f)	Repair of several [window] sashes, [with their dormant frames], throughout the [Rue d’Orléans] building (@ 40 livres per square toise)

(g)	Replacement of 548 7x8 pouce glass [window panes] (213 square pieds @ 28 sols per square pied - 298 livres 4 sols):

(i)	In the [Rue d’Orléans hospital] wards

(ii)	In the [Rue d’Orléans and Rue d’Estrées] chapel

(iii)	In the [Rue d’Orléans/Rue Dauphine] Brother’s lodging and pavilion

(h)	Clean and putty old glass [window panes] (324 @ 1 sol 6 deniers each - 24 livres 6 sols)

(7)	Shutters

(a) 	[Construction] of 2 interior [embôiture] shutters on the door of the partition which separates the [Rue d’Estrées/Rue d’Orléans] chapel from the [Rue d’Orléans hospital] wards (@ 30 livres per square toise)

(b) 	[Construction] of 2 interior [Embôiture] shutters on the 2 windows of the partition which separates the [Rue d’Estrées and Rue d’Orléans] chapel from the [Rue d’Orléans hospital] wards (@ 30 livres per square toise)

(8)	Interior Wooden Partitions

(a)	[Construction] of a 2-pouce thick pine partition to separate the [Rue d’Estrées/Rue d’Orléans] chapel from the wards (9 toises square 9 pouces @ 18 livres per square toise - 164 livres 5 sols)

(b)	Replacement of the partition for the Priest’s room produced from its former planks, together with its stringers, tringles, and nails (3 toises square 2 pieds 5 pouces of wooden material)

(c)	[Construction] of [several 2-pouce thick pine] partitions above the refectory (dining room) to [create] 2 rooms for the Brothers (@ 18 livres per square toise)

(9)	Vertical Interior Wall Cladding (Lambris)

(a)	[Construction] of a portion of the vertical wall cladding in the sanctuary on the side of the sacristy

(b)	[Construction] of a portion of the vertical wall cladding in the room of the aumônerie (chaplain) [using planed, tongued and grooved 1-pouce thick boards] (@ 12 livres per square toise)

(10)	Fireplaces/Potagers and Chimneys

(a)	Brick repairs to the [fireplace] fireback and oven of the [Rue Dauphine] kitchen (@ 175 livres per cubic toise)

(b)	Construction of a new brick [Rue Dauphine kitchen] potager (@ 175 livres per cubic toise)

(c)	The closing off of the [Rue Dauphine kitchen] dépense storeroom fireplace in masonry

(d)	The closing off with [Boston] boards of the [4] large chimneys in the [Rue d’Orléans west hospital] ward (@ 6 livres 10 sols per square pied)

(e)	The 16 cleaning sweeps of chimney [stacks] at different times (16 @ 8 sols - 6 livres 8 sols)

(f)	[Construction and later repair after use] of 4 large brick stoves, with a clay mortar, for heating the [hospital] wards (Construction @ 18 livres each - 72 livres; Post-repair with new bricks and clay @ 4 livres each)

(g)	[Construction and later repair after use] of 1 [large brick stove], with a clay mortar, for the linen room (Construction @ 18 livres each - 18 livres; Post-repair with new bricks and clay @ 4 livres each)

(11)	Hardware

(a)	A lock furnished for the lodgings of the Brothers (@ 6 livres 10 sols each)

(b)	A lock repaired and the furnishing of a new key

(12)	Furnishings

(a)	Demolition of the bunks built for the English Militia [in the hospital]

(b)	[Construction] of a table for cutting bread for the sick

(c)	[Construction] of a table for the decocting pots

(d)	[Construction] of a kitchen table (8 pieds 2 pouces long x 2 pieds wide)

(e)	[Construction] of a refectory (dining room) table

(f)	[Construction] of a standard for the meal-time bell

(g)	[Construction] of a carrier (9 pieds long) for transporting the dead

(h)	[Construction] of assorted utensils for the [Rue Royalle east] pavilion housing the bakery, including a large table (11 pieds long x 6 pieds wide)

(i)	[Construction] of furnishings for the [Rue Dauphine] kitchen

(j)	[Construction] of a portemanteau (rack or stand) for the use of the linen room

(k)	[Construction] of [Rue d’Estrées/Rue d’Orléans] chapel and religious furnishings

(l)	[Construction] of a [Rue Dauphine kitchen] dépense {storeroom} table and shelving

(m)	[Construction] of shelving for the soldier’s garde-robe (wardrobe)

(n)	[Construction] of a large armoire in the [Rue d’Orléans hospital] apothecary

(o)	[Construction] of an armoire in the [Rue d’Orléans/Rue Dauphine] Brother’s rooms

(p)	[Remounting and construction] of beds for the [Rue d’Orléans hospital] wards

(q)	[Modification] of the beds in the [Rue d’Orléans/Rue Dauphine] lodgings of the Brothers

(r)	[Construction] of a 38 pieds long roof oak fire prevention ladder, consisting of three vertical members and rungs (@ 20 sols per running pied - 38 livres)

(13)	Cleaning

(a)	Cleaned the entire [Rue d’Orléans] hospital building from top to bottom

(14)	Winter/storm porches

(a)	[Construction] of a [1/2 timber] winter/storm porch for the [central] passageway door which opens upon the terrace (@ 20 sols per cubic pied)

(i)	[Construction] of a single layer of weatherboarding, of Boston boards (@ 6 livres 10 sols per square toise)

(ii)	[Construction] of a double board roof, of Boston boards (@ 6 livres 10 sols per square toise)

(iii)	[Construction] of the ground-floor flooring, of 2-pouce thick planks (@ 12 livres per square toise)

(b)	[Construction] of a [1/2 timber] winter/storm porch for the kitchen door (@ 20 sols per cubic pied)

(i)	[Construction] of a single layer of weatherboarding, of Boston boards (@ 6 livres 10 sols per square toise)

(ii)	[Construction] of a double board roof, of Boston boards (@ 6 livres 10 sols per square toise)

(iii)	[Construction] of the ground-floor flooring, of 2-pouce thick planks (@ 12 livres per square toise)

(15) 	Latrines

(a)	Crépissage [à pierre apparente] roughcasting of the [walls] of the latrines for the sick (@ 45 sols per square toise)

(b)	[Construction] of 1 door [with embôitures at either end, made of tongued and grooved vertical 2- pouce thick planks] to the latrine for the sick (@ 32 livres per square toise)

(c)	[Construction] of the seat and front for the latrines of the Brothers

(d)	[Construction] of a separation partition in the latrines of the Brothers (2 square toises 4 pieds @ 12 livres per square toise - 32 livres)

(e)	[Construction] of a window sash, with its dormant frame, for the latrines (@ 40 livres per square toise)

(f)	[Construction] of the seat, posts and braces of the latrines, from timber (@ 20 sols per cubic pied)

(g)	[Construction] of a double board roof, of Boston boards, for the latrines for the sick (8 square toises 4 pieds @ 6 livres 10 sols per toise- 56 livres 6 sols 8 deniers)

(16)	Latrine Drains

(a)	Open up the sewage drain of the latrines which ran across the terrace, so that it could be unstopped

(b)	Open up the same sewage drain from the terrace [across the inner courtyard] to the [Grand] Étang, [so that it could unstopped]

(c)	Brick [repairs] to the man hole of the terrace sewage drain

(17)	Terrace

(a)	Cleaned the terrace of clinkers and rubbish

(b)	Brick [repairs] to a portion of the terrace [wall]

(18)	Inner Courtyard

(a)	Cleaned the large [inner court]yard (grande cour) of five years of accumulated dung and rubbish

(19)	Wells

(a)	[Construction] of a well lever and 2 well lips in brick (@ 175 livres per cubic toise)

(b)	[Construction] of a frame made of planks for a [well] lever

Following the repair work, Coeuret outlined what remaining Block 13 repairs were required owing to four years of English militia occupation of the hospital as a barracks. They amounted to 21,735 livres 18 sols as follows:

(Note: The proposed re-construction date is 1744-1745. Only the more useful dimensions, quantities, and specific prices have been extracted - Please consult the transcribed "Estimation des Reparations a faire" for complete details)

(1)	Roof

(a)	[Furnishing] of sheet lead missing from several places from the ridge, hip rafters, valleys and cornice of the [Rue d’Orléans hospital] bell [tower] (36 quintals @ 40 livres - 1,440 livres)

(b)	[Furnishing] of slate [for placement over] the 4 closed-off [Rue d’Orléans west hospital] ward large chimney [stacks] which the English had open [to serve] the [ground] and [first storey] floors which they had used as a barracks (@ 30 livres per square toise)

(c)	[Furnishing] of slate [for repairing] the [canon]-ball and grape-shot holes and the deterioration [of the Rue d’Orléans hospital] over four years (@ 30 livres per square toise)

(d)	Re-shingling of the pavilion which had served as a cellar (@ 12 livres per square toise)

(e)	Re-shingling of the [Rue Royalle west] pavilion housing the cave/storehouse/horse sheds (@ 12 livres per square toise)

(2)	Floors	and Floorings

(a)	Repair of the rotten lower and upper [Rue d’Orléans hospital] ward and lodgings floors [with 2-pouce thick pine planks] (440 square toises @ 18 livres per square toise - 7,920 livres)

(b)	Replacement of 34 21-pieds long ground floor sleepers (357 cubic pieds @ 25 sols per cubic pieds - 428 livres 8 sols)

(3)	Doors

(a)	[Construction] of [unspecified] doors

(b)	[Furnishing] of fine closure door locks

(c)	[Construction] of the [iron] grill in the chapel door which the English had removed (@ 10 sols per livre of weight)

(4)	Shutters

(a)	[Construction] of [unspecified] shutters

(b)	[Furnishing] of fine closure shutter locks

(5)	Windows

(a)	[Construction] of [unspecified window] sashes

(b)	[Furnishing] of fine closure window locks

(6)	Apothecary

(a)	[Construction] of the entire [Rue d’Orléans hospital] apothecary

(7)	Hardware

(a)	[Furnishing] of large and small locks (72 @ 8 livres each - 576 livres)

(8)	Furnishings

(a)	[Construction] of armoires, tables, and benches

(b)	[Furnishing] of fine closure armoire locks

(9)	Terrace

(a)	Removal of four years of accumulated rubbish from the terrace wall

(b)	[Rubble stone masonry] reconstruction of the terrace wall and its ramps (13 cubic toise @ 120 livres per cubic toise - 1,560 livres)

(c)	[Furnishing] of the iron balustrade which the English had removed from the terrace wall (@ 10 sols per livre of weight)

(d)	[Placement] of a cut stone coping stones on the terrace wall, with the [terrace corner] steps at each end, so as to re-establish it, and [prepare] it for the [iron] balustrade (630 square pieds of cut stone facing @ 3 livres 15 sols per square pied - 2,362 livres 10 sols)

(10)	Inner Courtyard

(a)	Removal of four years of accumulated dung and rubbish from the [inner court]yard

(11)	Wells

(a)	[Construction] of the 2 lips of the wells

(b)	Clean the wells

At year’s end, 18,600 livres was ordered for repairs and to lengthen the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion by 20 pieds, from the lodging of the Brothers of Charity [towards the north along Rue Dauphine] for a kitchen and [hospital] ward. The existing kitchen was declared no longer functional, though it did undergo some [fireplace] fireback, oven, potager, well, table, dresser, and cellar soupiraux (ventilator) repairs and new work that would have returned it to a useful state. Indeed, the fireplace repairs were perhaps related to the apothecary’s réserve (storeroom) question of 1750.

YEAR: 1750

On August 15, Coeuret, the fortification contractor, signed off on a major agreement that included the hospital for repairs. There the amount was 2,227 livres 2 sols 2 deniers:

(Note: The proposed re-construction date is 1744-1745. Only the more useful dimensions, quantities, and specific prices have been extracted - Please consult the transcribed "Etat des Ouvrages de reparations Et entretiens, qui ont Eté faits" for complete details)

(1)	Roof

(a)	Replacement of timbers for the [Rue d’Orléans east hospital] ward adjoining the [Rue d’Estrées/Rue D/Orléans] chapel, required by the demolition of 2 large English chimney [stacks] (@ 20 sols per cubic pied)

(b)	A new slate roof above the [Rue d’Orléans first storey] large ward

(c)	New slates placed over the demolished [Rue d’Orléans east hospital] ward adjoining the Rue d’Estrées/Rue D/Orléans chapel 2] stack [openings] (@ 30 livres per square toise)

(d)	[Slate] repairs to several portions of the [hospital] building roof (@ 30 livres per square toise)

(e)	Table lead on the two pillars of the [Rue d’Orléans] bell tower turret, to a portion of the hip-rafters and in-behind (@ 8 sols per livre of weight)

(2)	Floor

(a)	Replacement of [new] timbers, the use of demolition timbers, and the return of a portion of the existing flooring [planks] for the [Rue d’Orléans east hospital] ward adjoining the [Rue d’Estrées/Rue D/Orléans] chapel, required by the demolition of 2 large English chimney [stacks] (New timber @ 20 sols per cubic pied)

(b)	Replacement of new [2-pouce thick pine] flooring [planks] for the [Rue d’Orléans east hospital] ward adjoining the [Rue d’Estrées/Rue D/Orléans] chapel, required by the [demolition of] 2 [fireplace] bays and of the [other 2] fireplace [bays] no longer in use (@ 18 livres per square toise)

(c)	Removal, [re-insertion into the masonry walls], and re-sealing, with mortar, of the floor beams of the [Rue d’Orléans east hospital] ward adjoining the [Rue d’Estrées/Rue D/Orléans] chapel (@ 4 livres 17 sols per barrel of mortar)

(d)	[Furnishing] of two iron stirrups, weighing together 23 livres, for a [Rue d’Orléans/Rue d’Estrées] chapel beam (23 @ 9 sols per livre of weight - 10 livres 7 sols)

(3)	Vertical Interior Planed Tongued and Grooved Board Wall Cladding (Lambris)

(a)	To replace the fireplace [of the first storey Rue d’Orléans hospital] ward where the corners were cut off , made from planed, tongued and grooved [1-pouce thick] boards (1 square toise 1 pieds @ 12 livres per square toise)

(4)	Windows

(a)	[Placement] of 7x8 pouce glass [window panes] (@ 28 sols per square pied):

(i)	In the [hospital] building

(ii)	In [Rue d’Orléans/Rue Dauphine] lodgings of the Brothers

(iii)	In the large armoire of the [Rue d’Orléans] apothecary

(b)	Cleaning and re-puttying of glass [window panes] (696 @ 1 sol six deniers each - 52 livres 4 sols)

(c)	Cleaning of 601 glass [window panes] (601 @ 1 sol each - 30 livres 1 sol)

(5)	Fireplaces

(a) 	Re-establishment, with mortar, of the [Rue d’Orléans] apothecary [masonry] furnace, and of other like works (@ 4 livres 17 sols per barrel of mortar)

(b)	[Construction] of 4 large clay-mortared brick stoves in the [Rue d’Orléans] wards (4 @ 20 livres each - 80 livres)

(c)	Crépissage à pierre apparent roughcasting of the [chimney] stack of the [Rue d’Orléans] apothecary (@ 2 livres 5 sols per square toise)

(d)	Crépissage à pierre apparent roughcasting of the latrine vent-stack (@ 2 livres 5 sols per square toise)

(e)	Table lead on the chimney vent-stack of the latrines (@ 8 sols per livre of weight)

(f)	[Furnishing] of a iron ring, weighing 20 livres, for the furnace of the [Rue d’Orléans] apothecary (20 @ 9 sols of weight - 9 livres)

(6)	Hardware

(a)	[Furnishing] of the rocker-arm for the service bell for the sick [of the Rue d’Orléans wards]

(b)	Repair of the 4 locks, and the furnishing of two keys and the pivot pin for the arc-boutant of the large [Rue d’Orléans entry] door

(c)	Furnishing of 4 fiches a gond and the repair of 2 others, for the [Rue d’Orléans/Rue Dauphine] Priest’s room

(d)	[Furnishing of] 12 tringles, and 3 hooks each with its eye

(e)	[Furnishing of] 18 fiches a vase for the [Rue d’Orléans/Rue Dauphine] rooms of the Brothers (18 @ 40 sols each - 36 livres)

(f)	[Furnishing of] 2 spring bolts (2 @ 30 sols each - 3 livres)

(g)	[Furnishing of] 4 fiches a vase for a small armoire (4 @ 30 sols each - 6 livres)

(h)	[Furnishing of] 4 fiches a vase for a the armoire of the [Rue Dauphine kitchen] dépense (storeroom), together with a hook and its eye

(i)	[Furnishing] of 4 fiches a vase and 2 sash-bolts for the Venetian [sashes] for the [Rue d’Orléans] ground storey [hospital] ward [windows] adjoining the [Rue d’Estrées/Rue d’Orléans] chapel

(j)	[Furnishing] of 8 fiches a vase, 4 spring bolts, and 2 sash-bolts for the [2] Venetian [sashes] for the [Rue d’Orléans] first storey [hospital] ward [windows adjoining the Rue d’Estrées/Rue d’Orléans chapel]

(k)	[Furnishing] of 16 pairs of large fiches a vase, and 10 hooks and eyes for the armoire of the [Rue d’Orléans hospital] apothecary (Hinges @ 3 livres per pair - 48 livres; 10 Hook and eyes @ 10 sols - 5 livres)

(l)	[Furnishing] of 8 holdfast-nails to attach the cornice to the armoire of the [Rue d’Orléans hospital] apothecary (8 @ 6 sols each - 2 livres 8 sols)

(m)	[Furnishing] of 4 hinges for the small armoire of the [Rue d’Orléans/Rue Dauphine] room of the Brothers (4 @ 15 sols each - 3 livres)

(n)	[Furnishing] of 4 fiche a gonds and a tringle with its screw-type eyes for the [Rue d’Orléans/Rue Dauphine] room of the Brothers

(o)	[Furnishing] of 4 spring bolts for the [Rue d’Orléans/Rue Dauphine] room of the Brothers (4 @ 30 sols each - 6 livres)

(p)	[Furnishing] of several small lock-pieces, the repair of locks, and the [Furnishing] of 2 new keys for the [Rue d’Orléans/Rue Dauphine] lodgings of the Brothers

(q)	[Furnishing] of a polished lock a tour et demy for a [Rue d’Orléans/Rue Dauphine] room of the Brothers (1 @ 6 livres each - 6 livres)

(r)	[Furnishing] of 2 fiche a vase and the repair of several fine locks for a [Rue d’Orléans/Rue Dauphine] room [of the Brothers]

(s)	[Furnishing] of 22 locks for the armoire and for the small room (cabinet) for the [Rue d’Orléans hospital] apothecary, [Rue d’Estrées] sacristy, and [Rue d’Orléans/Rue Dauphine] lodgings [of the Brothers] (22 @ 4 livres each - 88 livres)

(t)	[Furnishing] of 3 pairs of strap hinges (3 @ 4 livres per pair - 12 livres)

(u)	[Furnishing] of 3 ordinary latches (3 @ 2 livres 10 sols each - 7 livres 10 sols)

(7)	Furnishings

(a)	[Construction] of beds

(b)	[Construction] of benches [for sitting] around the stoves

(c)	[Construction] of 6 large wooden chandeliers for the [Rue d’Orléans hospital] wards

(d)	[Construction] of rollers

(e)	[Furnishing] of the balance-weight, weighing 6 livres, for the large bell [of the Rue d’Orléans bell tower] (6 @ 9 sols per livre of weight - 2 livres 14 sols)

(8)	Latrines

(a)	Clean out the latrines

(b)	Clean out the [sewage] drain of the latrines

That year a proposal to build a new [Rue Dauphine] extension onto the existing [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion received the attention of Chief Engineer Franquet (later to become Director of Fortifications of the whole of New France), who agreed to consult with Governor Desherbiers and Prévost, the Commissaire-Ordonnateur. Engineer Boucher, who reported to Franquet, passed on the Brothers' of Charity concern that the proposed kitchen was too small to meet the needs of the sick, and that it didn’t have a basement, or anyplace, to prevent the freezing of apothecary supplies - an existing problem they had solved by using an apothecary’s [winter] réserve (storeroom) placed 12 pieds from a modified kitchen’s fireplace. With this in mind, then, he had raised the new building to the [ridge] level of the former one - to which it was now attached - and located the new kitchen and [winter] apothecary’s réserve (storeroom) or décharge (room) [at ground floor level], and [above] lodgings for three brothers.

According to Engineer Boucher, work on the new [Rue Dauphine] extension to house the new kitchen, the winter apothecary réserve (storeroom), and two Brothers, or more, had commenced this year. His 1750-1 plan which illustrates the new proposed building clearly differed in approach to that of Chief Engineer’s proposal of 1744. However, in some important respects, it also differed from Boucher’s later 1752-10 plan, suggesting that the 1750 plan had served merely as a proposal rather than as an precise illustration of final work.

In September, Engineer Boucher reported that he was unable to date to repair the floors of the [hospital] wards. Since spring, they had been filled with the sick. However, he had prepared all the required wood.

YEAR: 1751

According to several accounts, the large hospital [complex] entirely occupied [Block 13]. The four wards - two ground-floor and two upper ones - were furnished with 74 beds for 74 sick people. There were other buildings as well, serving the Brothers and other uses.At about the same time [circa 1750-1751], a proposal envisioned a new [aqueduct] to bring water to the infectious latrines as well as to provide a source for laundering clothes.

Prévost, the Commissaire-Ordonnateur, reported that the two large ground floor [hospital] wards had assumed a bad odour, caused, he thought, when used by the English as a barracks. After some false starts to rid the place of the odour, they did the following:

(1)	Scraped, and several times whitewash, the walls

(2)	Replace the rotten floors

(3)	Added two new windows to each [hospital] ward to brighten them up

(4)	Grilled [with iron bars] all the ground floor windows of the [hospital] wards, thus allowing them to lower the sills a good 4 pouces, which further [brightened up the rooms].

Plans were also made for further improvements, at the least cost:

(1)	To increase the size of the [hospital] wards

(2)	To establish a dormitory for sick officers

(3)	To establish a [Rue d’Estrées/Rue d’Orléans] chapel

(4)	To establish other conveniences

According to Engineer Boucher this work had also included roughcasting the exterior walls of the hospital during the good weather. He also noted that the four new ground-floor windows, all of which had been placed in the south [Rue d’Orléans] wall to brighten up the [hospital] wards and bring in more air, were larger than the original ones. To bring the old ones to same level as the new ones, he therefore lowered them, and added one row of glass window [panes] to each.

Although Prévost had replaced the ground-floor ward flooring, he left those above in place since they were still useable. However, he would cover them with a half planks to strengthen and seal them off from the floor below which had been suffering from certain unspecified annoyances.

On maps such as 1751-7 and 1751-10, the [Rue Dauphine] kitchen extension on the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion first appears. In November, 1751, Boucher reported that the Brothers were already using the new kitchen, but given that on 1752-10 he noted that the building was only completed in May, 1752, the rest of the building may still have been incomplete.

YEAR: 1752

In October, Engineer Boucher reported that he had submitted a [detailed - now missing] toisé describing the new [Rue Dauphine] kitchen building. Fortunately, this new situation map 1752-10 partially illustrated:

On the ground floor was the kitchen - with its window sink, its well, a potager, and a fireplace with an oven - and to it was attached an dépense (storeroom). Above, on the first storey, was the refectory (dining room), the office, and a small room designated as a buffet. Above that a knee-walled attic, consisting of three wood-revetted rooms serving as lodgings for 2 Brothers and 3 hospital attendants. For communicating between these three floor levels, a new staircase had been installed.

Architectural features included chimney caps, and windows and dormers of a balanced design. Structurally, among other things, the roof consisted of a braced king post and purlin system and the ends of its first storey and attic joists were embedded into the perimeter walls. The well had a pulley system. Unlike as shown on map 1750-1, toothing stones - placed to facilitate further construction - an exterior sink drain, and [Rue d’Orléans] street and terrace/inner courtyard exterior building perimeter pavé were not apparent.

Finally, the old basement kitchen of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion now served as a brewery and a place for decocting for the sick, and, above it, the former refectory (dining room) was now a parlour and heated public area. As for the former parlour, map 1752-10 identified it a room (note: 1750-1 had envisioned it more specifically as a garde manager). This section’s ground floor corridor remained as before, as did the basement room or cave to the south of the former kitchen into which the stairs from the ground floor exited, providing an interior access to the kitchen.

Maps 1750-1 and 1752-10 helped to explain what was built in Block 13 prior to the siege of 1745:

(1)	On map 1752-10 (and 1750-1 but with a different arrangement), Engineer Boucher clearly illustrated at least three soupiraux (ventilators) facing the inner courtyard of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion, and at least another two opening upon Rue Dauphine. Two illuminated the kitchen, two the basement room located to the south of the kitchen, and a third the stairs leading to the ground floor. The inner courtyard ventilators may be the ones added in 1749; those facing the street may be the ones which drew the complaint of 1744.

(2)	Both maps illustrated the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion with openings in the same location in the kitchen’s north wall where an exterior door and window were depicted on maps 1726-1/1726-1a.

(3)	Map 1750-1 illustrated the [Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion hospital] officers’ ward which had been constructed in 1739

(4)	Map 1750-1, like map 1745-4, illustrated the masonry henhouse-sheep fold attached to the [Rue Royalle west] pavilion housing the cave/storehouse/horse-cow sheds/henhouse-sheep fold, though with different door and window arrangements

(5)	Map 1750-1 illustrated a new inner courtyard doorway in the storehouse of the [Rue Royalle west] pavilion housing the cave/storehouse/horse-cow sheds/henhouse-sheep fold

(6)	Both maps illustrated a well that stood where the original pre-1745 [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion well once stood in the inner courtyard

Less helpful was that map 1752-10 also illustrated the following:

(1)	That the roof of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion had at least two small dormers

(a)	Maps 1726-1/1726-1a indicate at least one dormer

(b)	Map 1745-4 did not illustrate any dormers

(c)	Map 1750-1 did not illustrate any dormers

(2)	That the ground floor parlour of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion did not have a fireplace

(a)	Maps 1726-1/1726-1a indicate a fireplace

(b)	Map ND-82 (1729) does not indicate a fireplace

(c)	Map 1745-4 indicates a fireplace

(3)	That the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion had brick chimney stacks with brick caps

(a)	Map ND-82 (1729) illustrated brick chimney stacks with cut stone caps and strings (In fact, this plan showed the entire hospital complex as well as the [Rue Royalle east] pavilion housing the bakery/wash-house with this arrangement)

(b)	Map 1745-4 illustrated brick stacks with cut stone caps

(4)	That the ground floor guard room in the [Rue d’Orléans hospital] central pavilion did not have a fireplace

(a)	Maps 1726-1/1726-1a indicate one fireplace

(b)	Map ND-82 (1729) does not indicate any fireplaces

(5)	That the ground floor latrines in the [Rue d’Orléans hospital] central pavilion had stairs that communicated with the upper latrines

(a)	Maps 1726-1/1726-1a does not indicate any stairs

(b)	Map ND-82 (1729) does not indicate any stairs

(6)	That the fireplace of the wash-house of the [Rue Royalle east] bakery/wash-house was accompanied by circular structures, no doubt associated with the heating of water

(a)	Maps 1726-1/1726-1a does not indicate any additional features

(b)	Map ND-82 (1729) indicates the same circular structures

Maps 1750-1 and 1752-10 also illustrated a number of construction changes that had occurred to the pre-1745 structure since 1745. Among the new work were the following:

(1)	Map 1750-1 did not illustrate the exterior steps from the terrace to the [Rue d’Orléans/Rue d’Estrées east] right aisle/right pavilion which were shown on maps 1726-1/1726-1a

(2)	Map 1750-1 did not illustrate the exterior steps from the terrace to the [Rue d’Orléans hospital’s] central pavilion which were shown on maps 1726-1/1726-1a

(3)	Map 1750-1 illustrated 1 more ground floor window and 2 more ground floor doors (one with exterior steps) along the Rue d’Orléans face of the [Rue d’Orléans] hospital than shown on maps 1726-1/1726-1a

(i)	Four [Rue d’Orléans] windows were added in 1751

(4)	Maps 1750-1 and 1752-10 illustrated 1 more ground floor window along the inner courtyard side of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion, than shown on maps 1726-1/1726-1a, as well as 1 more first storey window than shown on Map 1745-1

(5)	Maps 1750-1 and 1752-10 illustrated (though in different configurations) three soupiraux (ventilators) facing the inner courtyard of the [Rue d’Orléans/Rue Dauphine west] left aisle/left pavilion which were not shown on Map 1745-4

(i)	Three were added in 1749

(6)	Map 1750-1 illustrated the blocked doorway between the storehouse and the horse-cow sheds/henhouse-sheep fold of the [Rue Royalle west] pavilion housing the cave/storehouse/horse-cow sheds/henhouse-sheep fold

(i)	Maps 1726-1/1726-1a illustrate a functional doorway

(ii)	It was blocked in 1749

(7)	Map 1750-1 illustrated a new Rue Royalle doorway in the storehouse of the [Rue Royalle west] pavilion housing the cave/storehouse/horse-cow sheds/henhouse-sheep fold

(i)	According to the 1744 account, this doorway ought to have appeared in the Rue Royalle wall of the [Rue Royalle east] pavilion housing the bakery

(8)	Map 1750-1 illustrated a new window in the Rue d’Estrées wall of the bakery’s rear-room of the [Rue Royalle east] pavilion housing the bakery/wash-house

YEAR: 1753

Thomas Pichon, who was a secretary to Governor Raymond (1751-1753), considered the hospital to be very spacious, extremely well-built, and the most comfortable house in the colonies.

YEAR: 1756

A fire (September 29-30) had destroyed the Block One bakery. In response, the bakery of the Hospital's [Rue Royalle east] pavilion housing the bakery/wash-house was prepared to receive workers for baking the bread for the colonial troupes.

YEAR: 1758

On July 6 or 7, a single bomb fell upon the hospital, killing a surgeon and wounding two Brothers of Charity. Nevertheless, later when in English hands, plans were formulated to make it again wind and weather tight.

YEAR: 1759

The Hospital attracted some [minor] work. Its roof was slate covered, while those of the [Rue Royalle east] pavilion housing the bakery/wash-house and of the [Rue Royalle west] pavilion housing the cave/storehouse/horse-cow sheds/henhouse-sheep fold were shingled covered.

YEAR: 1766

Repair estimates were prepared circa 1766 for the hospital, but were never implemented.

YEAR: 1767

The hospital, like the other masonry structures in the town, was decaying now quite quickly, and, according to maps 1767-1 and 1767-1a, much out of repair. Maps 1767-1/1767-1a also clearly illustrated the 1751-1752 new kitchen addition to be a separate, attached structure. They also suggest that the 1738 masonry henhouse attached to the [Rue Royalle west] pavilion housing the storehouse/stables/menagerie was now of wooden construction, and that there also now existed another wooden outbuilding attached to the south wall of the storehouse

YEAR: 1768

The stone hospital was declared much out of repair, or bad, meaning that for the most part the floors and partitions had been removed for fuel. Still, it was serving as a hospital, though with just enough accommodations to house the sick. Plans were in the works to make it soon wind and weather tight.

YEAR: 1849

The foundations of the hospital were still visible.

YEAR: 1862

Two stories of the hospital were still standing.

YEAR: 1893

Still distinguishable were the remains of the hospital.

YEAR: 1901

Properties intersect where the Block Thirteen Hospital once existed

YEAR: 1908

The ruins of the hospital were still visible.

PART TWO

AN INTERPRETATION OF BUILT EVENTS

HISTORICAL CONSTRUCTION

CHECK LIST (1744 - 1745)

The key maps for a 1744-1745 structural interpretation are 1725-8, 1725-8a, 1726-1, 1726-1a, 1729-3, 1750-1, and 1752-10. As maps 1726-1 and 1726-1a are judged more accurate than maps 1725-8 and 1725-8a, so too is map 1752-10 to that of map 1750-1. Where these do not provide a particular detail, then maps ND-82, 1731-1, 1731-3 and 1745-4 may be used.

Many of the assertions have been assigned a credibility rating. They are as follows:

CREDIBILITY "A" - Very high reliability of information - the most relevant documents support the conclusion:

CREDIBILITY "B" - Some major documents do not support the conclusions, but the weight is still clearly for the conclusion:

CREDIBILITY "C" - The evidence seems to be relatively equal "for" and "against" the conclusion, but internal factors (within the documents) and such things as experience of the researcher tend to weigh in the direction of the conclusion:

CREDIBILITY "D" - Much conflicting evidence with no preponderance in any direction, based mostly on an experience with the type of problem (generally), or, no primary evidence but based on experience and secondary sources, etcetera.

(I)

BLOCK THIRTEEN (1744 - 1745)

(A) 	GENERAL DATA

(1) 	MEASUREMENTS

(a)	Rue Royalle				40 toises

(b)	Rue d’Orléans			40 toises

(c)	Rue d’Estrées				29 toises

(d)	Rue Dauphine				29 toises

(e)	Square pieds				240 pieds x 174 pieds = 41,760 square pieds

(II)

HOSPITAL (1744 - 1745)

- Rue d’Orléans -

(A) 	GENERAL DATA

(1) 	CONSTRUCTION DATE

(a)	1721- 1722: 	Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion and a portion of the Rue d’Orléans east hospital ward

(b)	1724 - 1730:	Rue d’Orléans hospital and the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(2)	 OWNER

(a)	King of France

(3)	 FUNCTION

(a)	Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion

(i)	Basement

(I)	Unspecified use

and

Work area

(ii)	Ground floor

(I)	Officers’ ward

(II)	Chapel

(III)	Sacristy

(IV)	Raised chapel ceiling

(ii)	First storey

(I)	Officers’ ward

(iv)	Attic

(I)	Unspecified use

(b)	Rue d’Orléans east and west hospital wards

(i)	Ground floor

(I)	Two hospital wards

(II)	Central passageway

(ii)	First storey

(I)	Two hospital wards

(II)	Central passageway

(III)	Entry

(iv)	Attic

(I)	Unspecified use

(c)	Rue d’Orléans hospital’s central pavilion

(i)	Ground floor

(I)	Latrines

(II)	Mortuary with its chapel

(III)	Apothecary

(IV)	Guard Room

(V)	Apothecary’s réserve (storeroom)

and

Prison

(VI)	Central passageway

(VII)	Central passageway terrace doorway’s exterior storm porch

(ii)	First storey

(I)	Latrines

(II)	Guard Room

(III)	Central passageway

(IV)	Linen room

(iii)	Attic

(I)	Unspecified use

(iv)	Roof

(I)	Bell Tower

(d)	Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(i)	Basement

(I)	Kitchen

(II)	Dépense (storeroom)

(III)	Cave

(ii)	Ground floor

(I)	Priest’s room

(II)	Priest’s cabinet

(II)	Parlour

(III)	Dining room

(IV)	Small room

(iii)	First storey

(I)	Brother's Residence

(iv)	Attic

(I)	Hospital attendants

(4) 	DIMENSIONS

(a)	General

(i)	Exterior

(I)	Rue d’Estrées		c. 63 pieds

(II)	Rue d’Orléans	c. 240 pieds

(III)	Rue Dauphine		c. 63 pieds

(IV)	Terrace

(east to west)		c. 25 pieds + c. 38 pieds + c. 57 pieds + c. 18 pieds + c. 78 pieds + c. 18 pieds + c. 38 pieds + c. 25 pieds

(ii)	Interior

(I)	Eastern section

(A)			20 pieds 6 pouces wide

(II)	Central and western sections

(A)			20 pieds 6 pouces wide

(b)	Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion

(i)	Exterior

(I)	Perimeter walls

(A)	Rue d’Estrées	c. 63 pieds

(B)	Rue d’Orléans c.25 pieds

(C)	Thickness	c. 2 pieds

(D)	Height		c. 5 pieds basement height + c. 18 pieds from ground floor to roof eave

(ii)	Interior

(I)	Length		c. 59 pieds

(II)	Width			c. 20 pieds

(c)	Rue d’Orléans east and west hospital wards

(i)	Exterior

(I)	Foundation

(A)	Thickness	c. 3 pieds(+-)

(B)	Height		Assorted

(II)	Perimeter walls

(A)	Rue d’Orléans c. 190 pieds

(B)	Depth		c. 25 pieds

(C)	Thickness	c. 2 pieds, centered upon the foundation, creating a 6 pouce (+-) interior and exterior ledge

(ii)	Interior

(I)	 East ward

(A)	Length	c. 87 pieds

(B)	Width		c. 20 pieds

(II)	Central passageway

(A)	Length	c. 20 pieds

(B)	Width		c. 12 pieds

(III)	 West ward

(A)	Length	c. 87 pieds

(B)	Width		c. 20 pieds

(d)	Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(i)	Exterior

(I)	Perimeter walls

(A)	Rue Dauphine  c. 63 pieds

(B)	Rue d’Orléans  c. 25 pieds

(C)	Thickness	  c. 2 pieds

(ii)	Interior

(I)	South division

(A)	Length	c. 20 pieds

(B)	Width		c. 20 pieds

(II)	Central division

(A)	Length	c. 17 pieds

(B)	Width		c. 20 pieds

(III)	North division

(A)	Length	c. 17 pieds

(B)	Width		c. 20 pieds

(e)	Rue d’Orléans hospital’s central pavilion

(i)	Exterior

(I)	Perimeter walls

(A)	Terrace side	c. 78 pieds

(B)	Depth		c. 18 pieds

(C)	Thickness	c. 2 pieds

(ii)	Interior

(I)	Guard room/apothecary’s réserve (storeroom)

(A)	Length	c. 10 pieds

(B)	Width		c. 16 pieds

(II)	Apothecary

(A)	Length	c. 16 pieds

(B)	Width		c. 16 pieds

(III)	Mortuary/staircase

(A)	Length	c. 16 pieds

(B)	Width		c. 16 pieds

(IV)	Latrines

(A)	Length	c. 7 pieds

(B)	Width		c. 16 pieds

(V)	Central passageway

(A)	Length	c. 18 pieds

(A)	Width		c. 12 pieds

(5)	GENERAL LAYOUT

(a) 	General

(i)	The rubble stone masonry building occupied the entire Rue d’Orléans front of Block 13. Standing 2 1/2 storey's tall, it had a pavilion at each end - with basements below - and, between, the hospital wards with an attached central pavilion topped by a bell tower.

It had casement style windows, numerous windows grilled with iron bars and/or shutters - including shuttered dormers - a mixture of interior mortar renderings and vertical wooden wall claddings, and a variety of finished doorways.

A number of architectural features included cut stone quoins for building corners; a simple, non-quoin stone, window surround design; chimney caps; a wide, and a weather-vane upon the bell tower. Window, dormer, and chimney stacks were generally of a balanced design.

Structurally, among other things, the roof consisted of a king post and purlin system and the ends of its ground floor and first storey joists were embedded into the perimeter walls.

(b)	Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion

(i)	This wing consisted of a ward for sick officers, a sacristy, and a chapel:

(I)	In the northern half of this wing, there was a ward consisting of 5 rooms for sick officers. Two were located on the ground-floor with the other 3 on the first storey above, accessed by a large U-shaped wooden stairway. Beneath this stair, a small stairway led to the basement below (a work area which was also serviced by two exterior basement doors). The attic function was unspecified.

(II)	There was one ground-floor dual fireplace complex, located in the officers' ward, serving two rooms, defined by the fireplace mass and one wooden partition. Above, in the first-storey above, there would have been additional fireplaces (or stoves) in the remaining three officers' rooms.

(III)	To the south of the ground-floor ward, there was a corridor - defined by the northern masonry bearing wall that carried the officers' fireplaces and by a southern wooden partition - which open upon the 2 officers' rooms to the north, upon the terrace to the west (by means of an exterior step), and upon the sacristy to the south.

(IV)	The sacristy in turn accessed the two-storey chapel, at the corner of Rue d’Orléans and Rue d’Estrées, which had a iron grilled door centered upon a distinctive partition wall to the west that opened upon a long east ward. Whether there was a second doorway leading, as it did in 1726, directly from the east ward to the sacristy, allowing the chapel to be by-passed, is problematic. The ceiling - both it and the walls were wood covered - rose two storeys high , allowing windows in the upper ward to overlook it. Possibly too, below, in the chapel door and partition, windows also existed, looking into the lower ward.

The roof was wood shingled. Fire prevention roof ladders were installed.

(c)	Rue d’Orléans east and west hospital wards and Rue d’Orléans hospital’s central pavilion

(i)	To the west of the chapel, the ground floor and first storey, Rue d’Orléans hospital, accessed from the exterior by a central passageway, consisted of long wards serviced by an attached, raised Rue d’Orléans hospital’s central pavilion - topped by a 8-sided bell tower - consisting of latrines, a mortuary with its chapel, an apothecary, an apothecary’s réserve (storeroom) and prison when required, and a guard room. The attic function was unspecified:

(I)	The ground-floor east and west wards - which were heated by iron, wood-burning stoves - were mirror images of each other, serviced by the arched doorways of the north/south central passageway which divided the hospital complex exactly in half. Each ward also opened upon a room to the north, located in the Rue d’Orléans hospital’s central pavilion: latrines (in the eastern portion of the Rue d’Orléans hospital’s central pavilion with its stack flue for venting cesspool gas from below), and the guard’s room with its fireplace (in the western portion). This configuration was the same on the first-storey. Above, in the attic, the floor was finished off with an upper surface of flooring planks.

(II)	The central passageway of the Rue d’Orléans east and west hospital wards opened up to the south, by means of an exterior step, upon Rue d’Orléans, and to the north, after passing through the Rue d’Orléans hospital’s central pavilion - again by means of a substantial exterior step covered by a storm porch when deemed necessary - upon the terrace. In the Rue d’Orléans hospital’s central pavilion, an east door of the central passageway accessed the mortuary with its chapel, and, to the north of it, set against a wooden partition, a stairway which led to the upper Rue d’Orléans east and west hospital wards via an upper entry (and, where no doubt, were to be found mirror images of the ground floor mortuary and apothecary room and storeroom but with other but unspecified functions such as linen storage and the like). Likewise, a west door accessed the apothecary with its furnace, and to the west of it, closed off from the guard room by a wooden partition, the apothecary’s réserve (storeroom) and prison.

The roof over a portion of the east ward was wood shingled. The remainder was covered with black slate.

(d)	Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(i)	From the ground-floor west ward of the hospital a door opened into the quarters of the Brothers in the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion consisting of basement, ground floor and first storey eating, living and sleeping accommodations respectively:

(I)	Along the east side of the ground-floor southern portion of the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion wing ran a corridor - defined by a wooden partition - that to the south serviced a priest’s room and wooden partitioned-off cabinet, and to the north, a parlour, then a U-shaped stairway set against a wooden partition- which, from the east side, accessed below the basement kitchen/dépense (storeroom) - indirectly through a second basement room (cave) located to the south of the kitchen - and above the first storey living quarters for the Brothers - then a doorway with exterior steps to the terrace, and finally the dining room in the extreme north portion of this wing. Behind the stairway, set along its western wooden-partitioned wall, stood a small room, which directly accessed both the parlour and the dining room.

Finally, there was an exterior basement door which also serviced the kitchen and an attic, where the hospital attendants were lodged.

The roof was slate covered..

(B) 	GENERAL CONSTRUCTION

(1) 	LAYOUT

(a) 	Basement and Finished Floor Elevation

(i)	Western section

(I)	Height to joist level 	5 1/2 pieds

(b)	Exterior Storm Porch

(i)	Type				1/2 Timber

(I)	Wall Covering

(A)	Type		Boston boards

(B)	Thickness	Single layer

(II)	Roof

(A)	Type		Boston boards

(B)	Thickness	Double layer

(III)	Floor

(A)	Type		Planks

(B)	Thickness	2 pouces

(2) 	CONSTRUCTION DETAILS

(a) 	Footing

(i)	Excavation

(I)	Width			3 pieds

(b) 	Foundation Wall

(i)	Type				Rubble stone masonry

(ii)	Mortar			Lime

(iii)	Thickness			2 pieds 6 pouces

(iv)	Silver and Copper medals

(I)				One of each placed somewhere in the eastern section

(c) 	Exterior Wall

(i)	Type				Rubble stone masonry

(ii)	Mortar			Lime

(ii)	Thickness			2 pieds

(iii)	Height				19 pieds 6 pouces, to the eave of the roof of the eastern section

(d) 	Masonry Bearing Partition Walls

(i)	Eastern basement (Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion)

(I)	Number		One

(II)	Type			Rubble stone masonry

(III)	Mortar		Lime

(IV)	Length		20 pieds

(V)	Thickness		2 pieds

(VI)	Height			6 pieds

(ii)	One in the eastern section (Rue d’Orléans/Rue d’Estrées east right aisle/right pavilion)

(I) 	Type		A replacement fireplace was re-set in a new interior bearing partition wall

(II)	Location	Centered in the eastern section

(III)	Height		High enough to strengthen the eastern section

(e) 	Building Corners Quoin Stones

(i)	Type				Imported cut grey sand stone

(f) 	Wall Plates (top of Wall)

(i)	Finish				Hewn timbers

(ii)	Material			Pine

(iii)	Thickness			4 pouces

(iv)	Width				8 pouces

(g)	Timber iron

(i)	Type				Large iron work

(3) 	FLOOR CONSTRUCTION

(a) 	Floor Joist

(i)	Ground floor

(1)	Types

(A)			Sleepers

(B)			Set into the perimeter wall

(II)	Finish			Hewn timber

(III)	Material		Pine

(IV)	Thickness		8 pouces x 9 pouces

(V)	Length		21 pieds

(ii)	First floor

(I)	Finish			Hewn timber

(II)	Material		Pine

(III)	Thickness		8 pouces x 9 pouces

(iii)	Attic floor

(I)	Finish			Hewn timber

(II)	Material		Pine

(III)	Thickness		8 pouces x 8 pouces

(b) 	Timber Hangers

(i)	 Type				Stirrups

(ii)	Material			Large iron work

(c) 	Flooring

(i)	Basement

(I)	Type			Pavé

(ii)	Ground floor

(I)	Type			Pine planks

(II)	Thickness		2 pouces

(iii)	First floor

(I)	Type			Pine planks

(II)	Thickness		2 pouces

(III)	Type			Hardwood

(iv)	Attic floor

(I)	Type			Pine planks

(II)	Thickness		2 pouces

(4) 	WOODEN PARTITION WALLS

(a)	Eastern section

(i)	Type				Boards

(ii)	Material			Pine

(iii)	Finish				Planed, both sides

(iv)	Joinery			Tongued and grooved

(v)	Thickness:			1 pouce

(vi)	Height				7 pieds 6 pouces

(b)	Chapel balustrade

(i)	Type				Boards

(ii)	Thickness			1 pouce

(iii)	Height				2 pieds 9 pouces

(iv)	Length			25 pieds

(5) 	EXTERIOR WALL FINISH

(a) 	Roughcast

(i)	Crépissage

(I)				Crépissage

(6) 	INTERIOR FINISH

(a) 	Roughcast

(i)	Crépissage/Enduit

(I)	Type			Enduit

(II)	Mortar		Lime and sand

(III)	Whitewash		2 coats

(b) 	Vertical Wall Cladding

(i)	Upper Ward

(I)	 Lambris

(ii)	Upper Central Passageway

(I)	 Lambris

(iii)	Sacristy

(I)	Lambris on the side of the sacristy

(c) 	Ceiling

(i)	Chapel

(I)	Nailers		Rafters

(II)	Nailer Finish		Hewn timber

(III)	Nailer Material	Pine

(IV)	Nailer Thickness	4 pouce x 4 pouce

(V)	Ceiling covering

(A)	Type		Boards

(B)	Thickness	1 pouce

(C)	Area		10 pieds x 10 pieds

(VI)	 Sides covering

(A)	Number	4 sides

(B)	Type		Boards

(C)	Thickness	1 pouce

(D)	Height		Each side: 6 pieds

(E)	Length	Each side: 12 pieds 6 pouces

(VII)	Small triangular area

(A)	Number	One

(B)	Type		Boards

(C)	Thickness	1 pouce

(D)	Length	2 pieds

(E)	Height		6 pieds

(7) 	DETAILS STAIRS, STEPS, ETC

(a) 	Stairway

(i)	East section small stairway:

(I)	Framing

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	4 pouces x 4 pouces

(II)	Stringer

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	2 pouces x 10 pouces

(ii)	East section large stairway

(I)	Newel

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	6 pouces x 6 pouces

(D)	Height		6 pieds

(II)	Stringers

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	2 pouces

(D)	Width		10 pouces

(III)	Risers

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	2 pouces

(D)	Width		10 pouces

(b)	Steps

(i)	Exterior

(I)	Common

(A)	Material	Unspecified

(B)	Dimensions	As required

(II)	Rue d’Orléans Entry

(A)	Material	Unspecified

(B)	Dimensions	Substantial

(C) 	ROOF CONSTRUCTION

(1) 	GENERAL

(a) 	Roof Covering

(i)	Eastern section

(I)	Covering Type	Wood shingles

(II)	Roof ladders		Fire prevention

(ii)	Rue d’Orléans hospital’s central pavilion and western section

(I)	Sheathing Type	Boards

(II)	Covering Type	Nailed black slate

(III)	Flashings		Table lead on the roof ridge, valleys, etc.

(IV)	Flashings		Cement

(b) 	Dormers

(i)	Ridge beam and other framing members

(I)	Finish			Hewn timber

(II)	Material		Pine

(III)	Thickness		4 pouces x 4 pouces

(IV)	Eastern section

(A)	Covering Type Wood shingles

(c)	Bell Tower

(i)	Type				Wooden, with internal structural iron-work

(ii)	 Design			8-sides

(iii)	Height				c. 32 1/2 pieds (excluding its weather-vane)

(iv)	Roofing Type			Black slate

(v)	Flashings			Sheet table lead on the roof ridge, hip rafters and in-behind, valleys, cornice, and the two pillars of the turret

(vi)	Bell

(I)	Number		One

(II)	Balance -weight	Weighs 6 livres

(d)	Finials

(i)	Type				Round

(ii)	Covering			Lead

(2) 	ROOF DETAILS

(a) 	Principal Truss

(i)	Main roof trusses

(I)	King post

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	8 pouces

(D)	Width		9 pouces

(E)	Length	7 pieds 8 pouces

(II)	2 principal rafters (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	4 pouces

(D)	Width		8 pouces

(E)	Length	12 pieds

(III)	2 struts (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	4 pouces

(D)	Width		4 pouces

(E)	Length	4 pieds

(ii)	Large Chapel truss

(I)	Number		One

(II)	Hip and valley rafters (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	8 pouces

(D)	Width		8 pouces

(III)	Collar beams (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	8 pouces

(D)	Width		8 pouces

(E)	Ironwork	Anchors

(IV)	One king post (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	8 pouces

(D)	Width		8 pouces

(V)	6 braces (each)

(A)	Finish		Hewn timber

(B)	Material	Pine

(C)	Thickness	4 pouces

(D)	Width		4 pouces

(E)	Length	2 pieds

(b) 	Braces

(i)	Finish				Hewn timber

(ii)	Material			Pine

(iii)	Thickness			4 pouces

(iv)	Width				8 pouces

(c) 	Purlins

(i)	Runs				One per roof slope

(ii)	Finish				Hewn timber

(iii)	Material			Pine

(iv)	Thickness			4 pouces

(v)	Width				8 pouces

(d) 	Ridge Beam

(i)	Finish				Hewn timber

(ii)	Material			Pine

(iii)	Thickness			4 pouces

(iv)	Width				8 pouces

(e) 	Rafters (Common)

(i)	Finish				Hewn timber

(ii)	Material			Pine

(iii)	Thickness			4 pouces

(iv)	Width				4 pouces thick

(v)	Length			Each 15 pieds

(f) 	Eaves

(i)	Finish				Hewn timber

(ii)	Material			Pine

(iii)	Thickness			4 pouces

(iv)	Width				4 pouces

(v)	Length			Each 15 pieds

(D) 	FIREPLACES, CHIMNEYS, LATRINE VENTS, ETC.

(1)	 GENERAL

(a) 	Fireplaces

(i)	Eastern section

(I) 	A replacement fireplace was re-set in a new interior bearing partition wall

(ii)	Rue d’Orléans hospital’s central pavilion

(I)	Mortared apothecary furnace

(II)	Chimney stack roughcast with crépissage à pierre apparent

(III)	Iron furnace ring

(iii)	Western section

(I)	Kitchen

(A)	Type		Kitchen fireplace with an oven

(B)	Material	Brick

(II)	Dépense (storeroom)

(A)	Type		Unspecified

(b) 	Stoves

(i)	Type	Iron Stoves

(I)	Fuel			Wood burning

(ii)	Type	Brick Stoves

(I)	Mortar		Clay

(II)	Fuel			Wood burning

(c) 	Latrines

(i)	Rue d’Orléans hospital’s central pavilion - For the sick

(I)	Vent stack flue for venting cesspool gas from below

(A)	Finish		Crépissage à pierre apparente

(B)	Flashings	Table lead

(2) 	CONSTRUCTION DETAILS

(a) 	Lintel Bars

(i)	Type		Fireplace bars

(ii)	Material	Large iron work

(E) 	DETAILS OF DOOR AND OPENING

(1) 	GENERAL

(a) 	Door

(i)	Basement

(I)	Type			Plank

(II)	Joinery		Embôiture

(III)	Thickness		2 pouce

(IV)	Height			5 pieds

(V)	Width			3 pieds

(ii)	General purpose

(I)	Type			Board

(II)	Material		Pine

(III)	Joinery		Oak embôitures at both ends

(IV)	Thickness		1 pouce

(V)	Height			5 pieds 9 pouces

(VI)	Width			2 pieds 6 pouces

(VII)	Strap hinges, with their pintles

(A)	Length	1 pied 6 pouces

(B)	Length	2 pieds

(iii)	Special

(I)	Type			Board

(II)	Joinery		Embôiture

(III)	Thickness		1 pouce

(IV)	Height			7 pieds 6 pouces

(V)	Width			3 pieds 6 pouces

(VI)	Architraves 		Included

(iv)	Chapel

(I)	Type			Iron grill in the door

(v)	Entry

(I)	Type			Plank

(II)	Thickness		2 pouces

(III)	Width			Large

(IV)	Leaves		Two

(V)	Joinery

(A)			Embôitures at either end

(B)			Tongued and grooved vertical planks

(VI)	Hardware

(A)	Locks		Four

(B)	Keys		Two

(C)	Arch-boutant

and its

pivot pin	One

(b) 	Arrière-Voussure

(i)	Eastern section

(I)	Type			Flat stone

(ii)	Rue d’Orléans hospital’s central pavilion and western sections

(I)	Type			Birch planks

(c)	Transoms

(i)	Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(I)	Location		Over the exterior terrace doorway

(II)	Lights 		4 over 4

(2) 	MISCELLANEOUS

(a) 	Door Casings (Architraves)

(i)	Some oak or birch architraves

(b)	Cornices

(i)	Type				Above some doors

(ii)	Material			Fir

(c) 	Locking Devices

(i)	Type				Bernard locks a tour et demy

(ii)	Dimensions

(I)	Length

(A)			7 pouces

(B)			8 pouces

(II)	Width			4 pouces

(d) 	Door Latches

(i)	Type				Latch

(F) 	DETAILS OF WINDOW AND OPENING

(1) 	GENERAL

(a) 	Basement Vents

(i)	Western section

(I)	Type			Cellar lights sashes or ventilators, with their dormant frames, for the kitchen

(b) 	Arrière-Voussure

(i)	Eastern section

(I)	Type			Flat stone

(ii)	Rue d’Orléans hospital’s central pavilion and western sections

(I)	Type			Birch planks

(2) 	WINDOW SURROUNDS (EXTERIOR)

(a) 	Sill

(i)	Type				Plank

(b) 	Surrounds

(i)	Type				Imported cut grey sand stone

(3) 	SASH DETAILS

(a) 	Window Type

(i)	Frames with their casement sashes

(I)	Height			3 pieds 9 pouces

(II)	Width			3 pieds

(III)	Number of panes	20 per sash

(IV)	Pane size		7 pouces x 8 pouces

(V)	Glazing		Glued glazing paper but, by 1745, some, if not all of the paper replaced by putty glazing

(VI)	Sash Dimensions	3 pouces or more x 2 pouces

(VII)	Strap hinges, with their pintles

(A)	Length	1 pied

(VIII)	Sash Locking Bolts

(A)	Type		Locking bolts

(ii)	Sash

(I)	Chapel

(A)	Type		Large

(iii)	Barred

(I)	Rue d’Orléans hospital’s central pavilion ground floor apothecary’s réserve (storeroom) and prison

(A)	Type		3 bars set in a frame

(II)	Officers’ Ward

(A)	Type		Bars

(iv)	Iron grates or grills

(I)	Chapel

(A)	Type		Large iron grates or grills for large windows

(b)	Dormer Type

(i)	Sash, with its dormant frame

(G) 	SHUTTERS

(1)	GENERAL

(a) 	Shutter Type (Location)

(i)	Windows

(I)	Type			Board

(II)	Joinery		Embôiture

(III)	Thickness		1 pouce

(IV)	Height			4 pieds

(V)	Width			3 pieds

(VI)	Strap hinges, with their pintles

(A)	Length 	1 pied 6 pouces

(ii)	Dormers

(I)	Type			Board

(II)	Joinery		Embôiture

(III)	Thickness		1 pouce

(IV)	Height			2 pieds 9 pouces

(V)	Width			1 pied 7 pouces

(b)	Hooks and eyes

(I)	Type			Keepers and fasteners

(H)	GENERAL HARDWARE: DOOR, WINDOWS AND SHUTTERS

(1)	GENERAL

(a)	Bolts on plates

(i)	Length			6 pouces

(I)	MATERIALS

(1)	GENERAL

(a)	Cut Lime Stone

(i)	Suitable for carving

(I)	Type			Cut white lime stone

(b)	Iron

(i)	Type				Flat iron

(c)	Steel

(i)	Type				Steel

(III)

BAKERY/WASH-HOUSE (1744 - 1745)

- Rue Royalle/Rue d’Estrées east Pavilion,

housing the Bakery/Wash-House -

(A) 	GENERAL DATA

(1) 	CONSTRUCTION DATE

(a)	1724 - 1726

(2)	 OWNER

(a)	King of France

(3)	 FUNCTION

(a)	Bakery

(i)	Ground floor

(I)	Bakery

(A)	Bakery fore-room

(B)	Bakery rear-room

(ii)	Attic

(I)	Unspecified use

(b)	Wash-house

(i)	Ground floor

(I)	Wash-house

(4) 	DIMENSIONS

(a)	General

(i)	Exterior

(I)	Rue d’Estrées		c. 51 pieds

(II)	Rue Royalle		c. 51 pieds

(III)	Inner Courtyard

(north to south)	c. 25 pieds + c. 25 pieds 6 pouces + c. 25 pieds 6 pouces + c. 25 pieds

(ii)	L-shaped perimeter walls

(I)	Rue Royalle		c. 51 pieds

(II)	Rue d’Estrées		c. 51 pieds

(III)	Width			c. 25 pieds

(IV)	Thickness		c. 2 pieds

(b)	Interior

(i)	Wash-house

(I)	Length		c. 22 pieds

(II)	Width			c. 20 pieds

(ii)	Bakery (fore-room)

(I)	Length		c. 23 pieds

(II)	Width			c. 20 pieds

(iii)	Oven room (rear-room)

(I)	Length		c. 19 pieds

(II)	Width			c. 20 pieds

(5)	GENERAL LAYOUT

(a) 	General

(i)	In the eastern corner of the property along Rue Royalle stood the rubble stone masonry Rue Royalle east pavilion housing both the Rue Royalle/Rue d’Estrées east wash-house and the Rue Royalle east bakery:

(I)	This pavilion consisted of three ground-floor rooms: Two of these, at the corner of the street, comprised the bakery - the fore-room, entered by a door facing the inner courtyard, servicing a single oven, and a rear-room partially occupied by the oven mass itself. In the fore-room, a straight run of stairs led to the attic area.

(II)	The third, to the south facing Rue d’Estrées, was the wash-house, likewise with a west doorway opening into the inner courtyard. An unusual shaped fireplace, no doubt for heating water, stood against its north wall. Probably too, adjacent to the fireplace, there would have been circular structures, also associated with the process.

Structurally - among other things - the wood shingled roof consisted of a king post and purlin system.

(B) 	GENERAL CONSTRUCTION

(1) 	CONSTRUCTION DETAILS

(a) 	Footing

(i)	Excavation

(I)	Width			3 pieds

(b) 	Foundation Wall

(i)	Type				Rubble stone masonry

(ii)	Mortar			Lime

(iii)	Thickness			2 pieds 6 pouces

(2) 	FLOOR CONSTRUCTION

(a) 	Flooring

(i)	Type				Pavé

(3) 	WOODEN PARTITION WALLS

(a)	Type					Boston board

(4) 	DETAILS STAIRS, STEPS, ETC

(a) 	Stairway

(i)	Type				Boston board

(C) 	ROOF CONSTRUCTION

(1) 	GENERAL

(a) 	Roof Covering

(i)	Type				Shingles

(ii)	Roof ladders			Fire prevention

(D) 	FIREPLACES, CHIMNEYS, LATRINE VENTS, ETC.

(1)	 GENERAL

(a) 	Oven

(i)	Mortar			Lime

(E) 	DETAILS OF DOOR AND OPENING

(1) 	DOOR FRAMES

(a)	Interior Frame

(i)	Finish				Hewn timber

(b)	Exterior Frame

(i)	Finish				Hewn timber

(2) 	DOOR CONSTRUCTION

(a) 	Exterior Door

(i)	Type				Batten with bars and diagonals

(ii)	Thickness			2 pouces

(3) 	MISCELLANEOUS

(a) 	Locking Devices

(i)	Type				Common

(b) 	Door Latches

(i)	Type				Latch

(F)	MATERIALS

(1)	GENERAL

(a)	Cut Sand Stone

(i)	Suitable for quoins, etc.

(I)	Type			Cut grey sand stone

(b)	Cut Lime Stone

(i)	Suitable for carving

(I)	Type			Cut white lime stone

(G)	GENERAL HARDWARE: DOOR, WINDOWS AND SHUTTERS

(1)	GENERAL

(a) 	Hinges

(i)	Type	Strap hinges, with their pintles

(I)	Length		2 pieds

(II)	Length		1 pied 6 pouces

(IV)

STOREHOUSE/STABLES/MENAGERIE (1744 - 1745)

- Rue Royalle/Rue Dauphine west Pavilion,

housing the Storehouse/Stables/Menagerie -

(A) 	GENERAL DATA

(1) 	CONSTRUCTION DATE

(a)	1724 - 1726

(2)	 OWNER

(a)	King of France

(3)	 FUNCTION

(a)	Storehouse

(i)	Ground floor

(I)	Storehouse

(b)	Stables

(i)	Ground floor

(I)	Horse and cow shed

(c)	Menagerie

(i)	Ground floor

(I)	Sheep fold

(II)	Henhouse

(4) 	DIMENSIONS

(a)	Exterior

(i)	General

(I)	Rue Dauphine		c. 51 pieds

(II)	Rue Royalle		c. 51 pieds

(III)	Inner Courtyard

(north to south)	c. 25 pieds + c. 25 pieds 6 pouces + c. 25 pieds 6 pouces + c. 25 pieds

(ii)	L-shaped perimeter walls

(I)	Rue Royalle		c. 51 pieds

(II)	Rue Dauphine		c. 51 pieds

(III)	Width			c. 25 pieds

(IV)	Thickness		c. 2 pieds

(b)	Interior

(i)	East stable/menagerie

(I)	Length		c. 23 pieds

(II)	Width			c. 20 pieds

(ii)	West stable/menagerie

(I)	Length		c. 46 pieds

(II)	Width			c. 20 pieds

(5)	GENERAL LAYOUT

(a) 	General

(i)	In the western corner of the property along Rue Royalle stood the Rue Royalle west rubble stone masonry pavilion housing the storehouse/stables/menagerie:

(I)	This pavilion served only as a stable and menagerie at the beginning. The addition of the henhouse reflected an increased use for this building. The storehouse and stabling functions likely began in the pre-1745 period but this conclusion remains problematic.

(II)	This pavilion consisted of three ground-floor rooms. If one accepts an increased function, then at the corner of the street there was a storehouse entered from the inner courtyard through an exterior door and, to the east, connected to the horse-cow sheds through an arched interior doorway. The horse-cow sheds were also entered through an exterior doorway facing the inner courtyard. The third room was created by extending the roof slope of the stables southwards and it housed hens, as well as sheep (with at least one, if not two, small doorways leading to the inner courtyard).

Structurally - among other things - the wood shingled roof consisted of a king post and purlin system.

(B) 	GENERAL CONSTRUCTION

(I) 	CONSTRUCTION DETAILS

(a) 	Footing

(i)	Excavation

(I)	Width			3 pieds

(2) 	EXTERIOR WALL FINISH

(a) 	Roughcast

(i)	Crépissage

(I)	Type			Crépissage à pierre apparente

(C) 	ROOF CONSTRUCTION

(1) 	GENERAL

(a) 	Roof Covering

(i)	Type				Shingles

(ii)	Roof ladders			Fire prevention

(D) 	DETAILS OF DOOR AND OPENING

(1) 	GENERAL

(a) 	Door

(i)	Type				Plank

(ii)	Thickness			2 pouces

(iii)	Joinery

(I)				Embôitures at either end

(II)				Tongued and grooved vertical planks

(b) 	Archways

(i)	Between the west and east rooms

(E)	MATERIALS

(1)	GENERAL

(a)	Cut Sand Stone

(i)	Suitable for quoins, etc.

(I)	Type			Cut grey sand stone

(b)	Cut Lime Stone

(i)	Suitable for carving

(I)	Type			Cut white lime stone

(V)

PERIMETER PROPERTY FENCES (1744 - 1745)

- Rue d’Estrées, Rue Dauphine, and Rue Royalle -

(A) 	GENERAL DATA

(1) 	CONSTRUCTION DATE

(a)	1724 - 1725

(2)	 OWNER

(a)	King of France

(3)	 FUNCTION

(a)	Perimeter property fence

(4) 	DIMENSIONS

(a)	Rue Dauphine perimeter property wall

(i)	Exterior

(I)	Foundation

(A)	Thickness	c. 3 pieds

(II)	Wall

(A)	Length	c. 62 pieds

(B)	Thickness

(1)		c. 2 pieds 6 pouces

(2)		or 4 feet

(C)	 Height	c. 12 pieds tall (inner courtyard side)

(b)	Rue Royalle perimeter property wall

(i)	Exterior

(I)	Wall

(A)	Length	c. 66 pieds [+c. 8 pieds wide gateway opening] + c. 66 pieds

(B)	Thickness	c. 2 pieds 6 pouces

(II)	Central arched gateway

(A)	Width of

opening	c. 8 pieds

(B)	Type		Arched, with a cross

(C)	Surrounds	Local Isle aux Justeaucorps cut stone

(c)	Rue d’Estrées perimeter property wall

(i)	Exterior

(I)	Foundation

(A)	Thickness	c. 3 pieds

(II)	Wall

(A)	Length	c. 62 pieds

(B)	Thickness	c. 2 pieds 6 pouces

(C)	Height		c. 12 pieds (inner courtyard side)

(5)	GENERAL LAYOUT

(a) 	General

(i)	Three perimeter property masonry fences closed off the remainder of Block 13 from the streets:

(I)	The perimeter property rubble stone walls stood approximately 12 pieds high, with a sloped cap, of a material other than rubble stone Centered upon the Rue Royalle fence was an arched entry gateway.

(B) 	GENERAL CONSTRUCTION

(1) 	CONSTRUCTION DETAILS

(a) 	Footing

(i)	Excavation

(I)	Width			3 pieds

(b) 	Foundation Wall

(i)	Type				Rubble stone masonry

(ii)	Mortar			Lime

(iii)	Thickness			2 pieds 6 pouces

(c)	Exterior Wall

(i)	Type				Rubble stone masonry

(ii)	Mortar			Lime

(iii)	Thickness			2 pieds thick for 3 pieds 6 pouces of height

(d)	Central gateway

(i)	Surrounds

(I)	Type			Local cut stone

(VI)

LANDSCAPE FEATURES (1744 - 1745)

- Terrace and Inner Courtyard -

(A) 	GENERAL DATA

(1) 	CONSTRUCTION DATE

(a)	Terrace with corner steps and a central ramp

(i)	1724 - 1730

(b)	Wells

(i)	1724 			Well of the Rue Royalle/Rue d’Estrées east wash-house

(ii)	1724 - 1730		Well of the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(c)	Interior fences

(i)	1724 - 1730

(d)	Gardens

(i)	1724 - 1730

(e)	Yards

(i)	1724 - 1730

(f)	Cesspool

(i)	1724 - 1730

(g)	Latrine drains

(i)	1724 - 1730

(h)	Exterior building perimeter pavé

(i)	1724 - 1730

(2)	 OWNER

(a)	King of France

(3)	 FUNCTION

(a)	Terrace

(i)	Terrace with corner steps and a central ramp

(b) 	Wells

(i)	Rue Royalle/Rue d’Estrées east wash-house inner courtyard

(ii)	Rue d’Orléans/Rue Dauphine west left aisle/left pavilion inner court yard

(c)	Interior fences

(i)	Inner courtyard interior fences

(ii)	Terrace interior fences

(d)	Gardens

(i)	Inner courtyard gardens

(e)	Yards

(i)	Inner courtyard lower courtyard yard

(f)	Cesspool

(i)	Terrace cesspool

(g)	Latrine drains

(i)	Drain

(h)	Pavé

(i)	Exterior building perimeter pavé

(4) 	DIMENSIONS

(a)	Well butting against the south wall of the bakery/wash-house and Rue d’Estrées perimeter property fence

(i)	Exterior

(I)	Wall

(A)	Diameter	c. 6 pieds

(b)	Cesspool

(i)	Exterior

(I)	Wall

(A) 	External reservoir

(1)	Length	c. 8 pieds

(2)	Width		c. 8 pieds

(B )	Interior Vault

(1)	Length	c. 12 pieds

(2)	Width		c. 11 pieds

(5)	GENERAL LAYOUT

(a)	General

(i)	Landscaping included a terrace and an inner courtyard and their associated features - wells, interior fences, gardens, yards, cesspool, drains, and exterior building perimeter pavé:

(I) 	From the terrace, ramps at either end - servicing the doorways of both the eastern and western pavilions of the hospital complex - and a pair in the centre, opposite the hospital’s exterior central passage way doorway, were to lead to the inner courtyard.

The terrace wall was faced with cut stone, with cut stone corner steps at each end. Set in the wall was an iron balustrade.

(II)	Two wells stood in the inner courtyard:

(A) 	One against the perimeter property fence, at the south wall of the Rue Royalle east pavilion, housing the bakery/wash-house.

(B) 	One against the perimeter property fence, near the north wall of the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion.

(III)	Interior piquet fences, with braced gates, creating two gardens in the inner courtyard - one to the east and to the west of the Rue Royalle central gateway - with a central passageway leading from the gateway to the terrace ramps, together with possibly other fenced and open yard areas, such as a separate lower yard, with their gates. Here might be found sun-dials as well.

(IV)	Underground cesspool located under the interior hospital latrines and continuing into under the terrace.

(V)	An underground hospital latrine drain across the terrace and inner courtyard that emptied into the Grand Étang. On the terrace, a brick man-hole serviced it.

(VI)	Both the terrace and the inner courtyard had pavé against the perimeter walls of one building, if not against all buildings.

(B) 	GENERAL CONSTRUCTION

(1) 	LAYOUT

(a) 	Terrace Sub-Drainage

(i)	Latrine drain (from the hospital terrace across the inner courtyard to the Grand Étang) extended.

(b) 	Terrace

(i)	Terrace Wall			Rubble stone masonry

(ii)	Coping-Stones		Cut stone

(iii)	Ramps			Rubble stone masonry walls

(iv)	Corner steps			Cut stone

(v)	Balustrade			Iron

(c) 	Garden

(i)	Garden

(d) 	Wells

(i)	Well of the Rue Royalle/Rue d’Estrées east wash-house

(I)	Excavation

(A)	Length	12 pieds

(B)	Width		12 pieds

(C)	Depth		7 pieds

(II)	Foundation and Elevation

(A)	Type		Rubble stone masonry

(B)	Mortar	Lime

(C)	Thickness	3 pieds

(D)	Circumference  25 pieds 1 pouce

(E)	Depth		7 pieds

(III)	Gallows

(A)	Iron

(ii)	Well of the Rue d’Orléans/Rue Dauphine west left aisle/left pavilion

(I)	Gallows

(A)	Iron

(II)	Hardware		For lowering and raising the well bucket

(e) 	Gates

(i)	Gardens

(I)	Type			Braced

(ii)	Yard

(I)	Hardware		Serrure a bosse and its chain

(II)	Type			Consisting of vertical and horizontal pieces with braces

(f)	Sun-Dials

(i)	Sun-dials

(g)	Exterior Building Perimeter Pavé

(i)	Terrace			Pavé

(ii)	Inner Courtyard		Pavé

PART THREE

REPRODUCED MAPS AND PLANS

(1713 - 1960)

1718-2

1722-1

1723-1

1723-2

1723-3

1723-4

1724-2

1725-8

1725-8a

1726-1

1726-1a

1726-1b

1727-1

1729-3

ND-82	 (1729)

1730-2

ND-45	 (1730-1735)

1731-1

1731-3

1731-4

1732-3

ND-24 	(1732-1737)

ND-76 	(1732-1737)

ND-78 	(1732-1737)

1733-7

ND-77 	(1733-1738)

1734-3

1734-4

1735-1

ND-13	 (1735-1740)

1737-1

1737-3

1737-6

1737-6a

ND-69	 (1737)

ND-89	 (1737)

1739-1

1742-6 	(1739)

1740-3

1741-2

1741-2a

1742-2

1744-5

1744-5a

1745-1

1745-1a

1745-1b

1745-4

1745-8

1745-11

1745-19

1745-21

1745-23

1745-23a

1745-24

1745-38

1745-40

ND-17 	(1745-1750)

1746-1

1746-2

1746-3

1746-4

1746-5

1746-8a

1746-13

1748-2

1750-1

ND-142 	(1750-1758)

ND-142a 	(1750-1758)

ND-142b 	(1750-1758)

ND-142c 	(1750-1758)

1751-7

1751-7a

1751-8

1751-10

1751-10a

1751-10b

1751-10c

1751-17

1751-17a

1751-17b

1751-17c

1752-10

1753-2

1754-8

ND-40	 (1755-1760)

1756 - 1 (1)

1756 - 1 (2)

1757-6

1757-8

1757-11

1757-12

1758-6

1758-6a

1758-9

1758-12c

1758-12d

1758-15

1758-21

1758-21a

1758-28

1758-36

ND-275	(c. 1758)

1759-1

ND-229	(April 7, 1759 to December 30, 1759)

1766-1

1767-1

1767-1a

1768-1

1768-1a

ND-246 	(1832)

1901-1

ENDNOTES

C11B, Volume 5, March 4, 1721, f. 324; B, Volume 44-2, March 5, 1721, f. 548; A3, Article 7, p. 97, March 5, 1721. See also: C11B, Volume 5, June 20, 1721, ff. 349-356 [f. 355]; F1A, Volume 22, June 21, 1721, ff. 84-85; B, Volume 45-2, May 12, 1722, pp. 1092-1097 (ff. 218-224); C11B, Volume 6, May 12, 1722, ff. 25-28; C11B, Volume 6, May 12, 1722, f. 21; B, Volume 45-2, May 12, 1722, pp. 925-929.

C11C, Volume 15, November 24, 1721, Pièce 206; C11B, Volume 5, December 7, 1721, ff. 148-155v; C11C, Volume 15, Pièce 210, March 24, 1722; C11B, Volume 9, November 15, 1727, ff. 234, 240. See also: C11B, Volume 12, September 1, 1731, f. 142.

C11B, Volume 5, December 7, 1721, ff. 148-148v, 150-151v, 153.

C11C, Volume 15, Pièce 210, March 24, 1722.

B, Volume 45-2, May 12, 1722, pp. 925-929; G1, Volume 466, Pièce 83, May 30, 1723; Map 1722-1; C11B, Volume 19, December 4, 1737, ff. 98-101; C11B, Volume 9, November 15, 1727, f. 249; C11B, Volume 12, September 1, 1731, f. 141; Map 1725-8; Map 1725-8a; Map 1726-1; Map 1726-1a.

C11B, Volume 9, November 15, 1727, ff. 240v, 244-245v, 249; C11B, Volume 12, September 1, 1731, ff. 130, 134v, 141.

C11A, Volume 126, Pièce 111, 1723; Map 1723-1.

C11B, Volume 7, October 1, 1725, ff. 311v-313v.

C11B, Volume 28, December 31, 1749, f. 335; C11B, Volume 29, December 31, 1750, f. 294v; G2, Volume 178, January 8, 1724, ff. 369-370.

C11B, Volume 7, 1724, f. 166; C11B, Volume 7, November 17, 1724, ff. 136v; C11B, Volume 7, November 20, 1724, ff. 57-58; C11B, Volume 9, November 15, 1727, ff. 233v, 240v; C11B, Volume 12, September 1, 1731, ff. 124, 130v; Map 1726-1; Map 1726-1a.

C11B, Volume 9, November 15, 1727, ff. 233v, 240v; C11B, Volume 12, September 1, 1731, ff. 124, 130v.

B, Volume 47, May 9, 1724, p. 1243; C11B, Volume 7, [1724], f. 167; C11C, Volume 16, Pièce 6, 1724.

C11B, Volume 7, November 20, 1724, ff. 57-58.

C11B, Volume 9, November 15, 1727, ff. 231-250v; C11B, Volume 12, September 1, 1731, ff. 122-143. See also: C11B, Volume 8, [May 23, 1725], ff. 222-225.

Isabeau contract of 1719: Dix livres pour Chacune Toise Cube de la fouille de L’Excavation du fossé du Bastion et fondation du Corps de cazernes Terre et Roche, C11B, Volume 4, March 7, 1719, f. 280.

Isabeau contract of 1719: Cent quarente livres pour Chacune Toise Cube de Mur en moeslon Tant Vuide que plain Le parement Brut Le Mortier Compose d’un tiers de Chaux et de deux Tiers de Sable tel quil Se trouvera dans le port de Louisbourg ...,C11B, Volume 4, March 7, 1719, f. 280v.

Isabeau contract of 1719: ... La pierre de Taille qui Sera placée dans led. mur Sera Toisée Comme le moeslon et il Sera en outre payé audit Isabeau Trois livres pour Chaque pied de lad. pierre de Taille de parement, C11B, Volume 4, p. 280v, March 7, 1719. See also: ... Prix des ouvrages qui estées obmis dans le marché du sieur Isabeau ... le pied ... quarré de pierre des taille venant de rochefort le Roy étant chargé du transport a cy ... offtrs du Sr ganet ... 3# ... C11B, Volume 8, May 23, 1725, f. 225.

Isabeau contract of 1719 had actually specified fir: Quarente Cinq Sols pour Chacune Solive de Sapin mise en oeuvre, C11B, Volume 4, March 7, 1719, f. 280v.

Isabeau contract of 1719 had actually specified fir: Dix sept livres pour Chacune Toise quarée de planchers de madriers de Sapin de Deux pouces d’Epaisseur chevillé de bois blanchy d’un Costé et emboufeté, Et Si le plancher de madriers est Cloué de Clouds de Cinq pouces au lieu d’estre Chevillé de bois il sera payé audit isabeau Trois Livres par Chacune toise de plus Ce qui fera Vingt Livres, C11B, Volume 4, March 7, 1719, f. 281v.

Isabeau contract of 1719: Quatorze livres pour Chacune Toise quarée de Cloisons de planches de sapin d’un pouce Blanchies des deux costés et emboufetés, C11B, Volume 4, March 7, 1719, f. 281v. However, later, when Ganet, the new fortification contractor, was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed he had substituted pine: la toise quarré de cloison de planches de pin d’un poulce d’Epaisseur blanchie des deux costés et Emboufetés á ... 14# .... C11B, Volume 8, May 23, 1725, f. 224.

Isabeau contract of 1719: Trente Six livres pour Chacune Toise quarée des portes des Escaliers a deux Venteaux de madriers de sapin de deux pouces emboitées par les deux bouts d'une traverse de Bois de chesne, C11B, Volume 4, March 7, 1719, f. 281v.

Isabeau contract of 1719: Trente Livres pour Chacune Toise quarée de Contrevents ou de portes pour les chambres de planches de sapin d’un pouce emboitées par les deux bouts de bois de chesne, C11B, Volume 4, March 7, 1719, f. 281v, However, later, when Ganet was tendering for his February 24, 1725, general contract (now missing), Ganet had proposed the following: ... la toise quarrée de contre vents ou portes pour les chambres de planche de pin d'une poulce d'Epoiseur Emboitées par les deux bouts de chesne á ... 25# ... For the same doors - having apparently substituted pine boards in place of fir - Isabeau had been charging 30 livres while local carpenters were charging 25 livres. C11B, Volume 8, May 23, 1725, f. 224.

Isabeau contract of 1719: Cinquante deux livres pour Chacune toise quarée de Croisées et dormants a Coulisses emboitées par la moitié de petit bois, C11B, Volume 4, March 7, 1719, f. 281v. Although the contract specifies double hung windows, subsequent documents clearly indicate that casement sashes were installed.

A 1726 a request for a supplementary contract also noted the following: ...15. toiser les chassit tant plain que viuide pour les verres Sur le pied de 24: Sols le pied quarré, Comme il et porté par le marché, ... le tant plain que viuide n’est que pour le petit bois Seulement cela peut fair Sur la longeur dun chassit 3: pouces de plus et 2: pouces Sur la largeur, ce n’est seulement que pour eviter beaucoup dembaras, ... C11B, Volume 8, November 12, 1726, f. 165.

Isabeau contract of 1719: Vingt quatre sols pour Chaque pied de Vitre quaré, C11B, Volume 4, March 7, 1719, f. 282. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224v. In 1726, concerning past and future work, a problem was identified: ...15. toiser les chassit tant plain que viuide pour les verres Sur le pied de 24: Sols le pied quarré, Comme il et porté par le marché, ... le tant plain que viuide n’est que pour le petit bois Seulement cela peut fair Sur la longeur dun chassit 3: pouces de plus et 2: pouces Sur la largeur, ce n’est seulement que pour eviter beaucoup dembaras, ... C11B, Volume 8, November 12, 1726, f. 165. On October 7, 1727, he received his supplementary contract, but no where is this point emphasized. C11B, Volume 9, October 7, 1727, ff.127-132.

Isabeau contract of 1719: Dix livres pour Chacune Toise quarée de Couverture de bardeau et lattes de chesne ou de Sapin ainsy quelles Se Trouveront, C11B, Volume 4, March 7, 1719, f. 280v.

Isabeau contract of 1719: Cinq sols six deniers pour chacune Livre de gros fer mise en oeuvre, C11B, Volume 4, March 7, 1719, f. 281.

Isabeau contract of 1719: Six livres pour chacune paire de penture d’un pied et un demy de long avec leurs gonds, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224v.

Isabeau contract of 1719: huit livres pour Chacune paire de pentures de deux pieds de long avec leurs gonds, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224v.

Isabeau contract of 1719: quatre livres pour Chacune paire de pentures d’un pied de Long avec leurs gonds, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224v.

Isabeau contract of 1719: quarente sols pour chacun Verouil sur plaque de six pouces, C11B, Volume 4,March 7, 1719, f. 281.When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224v.

Although latches were not specifically mentioned in Isabeau’s contract of 1719, and they were omitted in Ganet’s contract of February 24, 1725, general contract (now missing), Ganet in 1726 that he had adhered to the same specifications as ... passé de mesme a isabeau ... a 4.# chaque loquet ordinaire Comme Ceux ont esté passé aux portes des Casernes et Corps de garde du fort ... C11B, Volume 8, November 12, 1726, f. 165v. On October 7, 1727, he received a supplementary contract that described the latches as follows: 10. e ... Les Loquets, garnis seront d'un fer façonné, doux Liant, Solidement forgé, et posé avec soin afin que l'on puisse ouvrir et fermer les portes avec facilité, et Seront payez a la piece tout compris ... Chaque Loquet a quatre livres cy ... 4 ...C11B, Volume 9, October 7, 1727, ff. 128v, 131v.

Isabeau contract of 1719: huit livres pour chacune Serrure Bernarde de huit pouces de long Sur quatre de large a Tour et demy et pour chacune Serrure aussy Bernarde de sept pouces de Long, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications, C11B, Volume 8, May 23, 1725, f. 224.

C11B, Volume 20, November 3, 1738, ff. 209, 210; B, Volume 48-2, May 1, 1725, p. 930; B, Volume 48, July 25, 1725, pp. 941-943.

C11B, Volume 7, November 13, 1725, ff. 325v; C11B, Volume 7, December 3, 1725, ff. 264-265; C11B, Volume 7, December 16, 1725, ff. 330; C11B, Volume 7, December 17, 1725, f. 194v.

F1A, Volume 25, December 10, 1725, f. 30.

IE, Volume 139, [1745], p. 235; Louis Effingham De Forest (editor), Louisbourg Journals 1745, (New York: The Society of Colonial Wars in the State of New York, 1932), “First Journal - Anonymous,” October 17, 1745, pp. 50-51.

C11B, Volume 7, December 16, 1725, f. 330; C11B, Volume 7, December 16, 1725, f. 334.

B, Volume 49-2, May 28, 1726, f. 700; B, Volume 49-2, May 28, 1726, f. 701v; C11B, Volume 9, December 15, 1726, f. 207.

C11B, Volume 8, August 14, 1726, f. 89v; C11B, Volume 8, October 10, 1726, ff. 168-169; C11B, Volume 8, October 10, 1726, ff. 111v, 112v-113; C11B, Volume 8, November 28, 1726, f. 15; Map 1726-1; Map 1726-1a.

C11B, Volume 8, December 1, 1726, ff. 115-115v, 116v, 117v-118.

C11B, Volume 8, November 29, 1726, ff. 162 and 163; C11B, Volume 8, December 1, 1726, ff. 115-115v, 116v, 117v-118.

C11B, Volume 8, November 29, 1726, ff. 162 and 163; C11B, Volume 8, December 1, 1726, f. 72.

C11B, Volume 8, December 1, 1726, ff. 115-115v, 116v, 117v-118.

G1, Volume 466, Pièce 81, 1752, ff. 60-61; Sieur De La Roque, "Tour of Inspection Made By The Sieur De La Roque," (December 5, 1752) as translated in Report Concerning the Canadian Archives For The Year 1905, Volume 2 (1906), p. 37.

C11B, Volume 9, May 4, 1727, f. 229. See also: C11B, Volume 28, August 30, 1749, f. 319; C11B, Volume 28, December 31, 1749, f. 334.

C11B, Volume 9, November 15, 1727, ff. 231 - 250; C11B, Volume 12, September 1, 1731, ff. 122-143.

B, Volume 50, June 10, 1727, f. 570v; B, Volume 50, June 10, 1727, ff. 586v-587v; C11B, Volume 10, November 12, 1727, f. 148. See also: C11B, Volume 20, November 3, 1738, ff. 209, 210.

C11B, Volume 9, September 13, 1727, ff. 50-50v.

C11B, Volume 10, June 27, 1728, ff. 126-127; C11B, Volume 9, November 17, 1727, f. 142; C11B, Volume 9, November 19, 1727, ff. 60-60v; C11B, Volume 8, November 20, 1727, ff. 157-158; C11B, Volume 9, November 24, 1727, f. 94; C11B, Volume 9, November 26, 1727, f. 32v.

C11B, Volume 27, [1727], f. 317v. See also: C11B, Volume 13, February 3, 1732, ff. 11-13; C11B, Volume 18, November 13, 1736, ff. 95-99; C11B, Volume 29, November 20, 1750, ff. 171-173v.

C11B, Volume 27, [1727], f. 317v.

C11B, Volume 18, December 14, 1736, ff. 133, 135v-136. See also: C11B, Volume 18, December 14, 1736, ff.137, 140.

C11B, Volume 9, November 17, 1727, f. 152.

B, Volume 52, June 20, 1728, f. 589v; B, Volume 52, June 23, 1728, f. 595; C11B, Volume 10, November 3, 1728, ff. 50v, 53; C11B, Volume 10, November 3, 1728, f. 82; C11B, Volume 10, November 13, 1728, ff. 134-134v.

C11B, Volume 10, November 22, 1728, f. 109v; C11B, Volume 10, June 27, 1728, ff. 126-127.

C11B, Volume 10, November 13, 1728, ff. 134-134v.

C11B, Volume 18, May 1, 1736, f. 181.

C11B, Volume 10, November 13, 1728, f. 141.

C11B, Volume 10, November 13, 1728, f. 142; ND-82.

C11B, Volume 10, August 31, 1729, f. 240v; C11B, Volume 10, December 18, 1729, ff. 243-243v.

C11B, Volume 18, May 1, 1736, ff. 181, 183.

C11B, Volume 20, November 3, 1738, ff. 209, 210.

C11B, Volume 11, March 26, 1730, ff. 81v-82.

Because cornices - among other items - were not mentioned in the Ganet general fortification contract of February 24, 1725 (now missing), he insisted upon a supplementary contract. In 1726, concerning past and future work, a problem was identified: ...a 3:# le pied quarrée de menuserie en moulure d architecture, Comme Celle de la Corniche des pilliers de la Chapelle, en bois de pin et autres de mesme nature douvrage toisé au developement du cordeau, ... ce prist paroit un peu chere cela vient de la difficulté des ouvriers, que l’on a pas comme lon veu, et le tems et le bois qu’il y a emploié, il y en a plus de fait, quil ne sent fera, ... C11B, Volume 8, November 12, 1726, ff. 167-167v. On October 7, 1727, he received his supplementary contract: ... 24. Si Certains ouvrages Exigent quelques Corniches ou autres moulures repetées sur bois de sapin de meme Espece, on en donnera les profils Cette architecture sera mesurée au pied quarré developée au Cordeau ... Le pied Carré de corniches a deux Livres dix sols cy ... 2. 10... C11B, Volume 9, October 7, 1727, ff. 130, 132.

Because oak and birch architraves - among other items - were not mentioned in the Ganet general fortification contract of February 24, 1725 (now missing), he insisted upon a supplementary contract. In 1726, concerning past and future work, a problem was identified: ... 31 ... a 52:# la toise quarré de menuserie pour les chambranle des portes quand les dits chambranle Seront fait en bois de chesne ou de merisie ... la fason de l'ouvrage et beaucoup de bois perdu, et la chereté, et difficulté de travailler ce bois, fait que le prist et tres modique ... C11B, Volume 8, November 12, 1726, f. 167. On October 7, 1727, he received his supplementary contract: 22. e ... Les Chambranles de bois de chesne ou de merisier pour portes seront bien assemblez avec un quart de rond entre deux fillets, bien chevillez et assemblez par leurs angles, et LEpaisseur du bois qu'on Employera sera d'un pouce et quart, payez a la toise quarré ... La toise quarrée de chambranles a cinquante deux livres cy ... 52 .. C11B, Volume 9, October 7, 1727, ff. 130, 132.

Ganet, tendering for his February 24, 1725, general contract (now missing), had proposed the following: ... la toise quarrée de contre vents ou portes pour les chambres de planche de pin d'une poulce d'Epoiseur Emboitées par les deux bouts de chesne á ... 25# ... For the same doors, Isabeau had been charging 30 livres while local carpenters were charging 25 livres. C11B, Volume 8, May 23, 1725, f. 224.

Isabeau contract of 1719: quatre livres pour Chacune paire de pentures d’un pied de Long avec leurs gonds, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Isabeau’s compared price revealed that he was still adhering to the same specifications: chaque paire de pentures d’un pied de long avec leur gonds á cy ... Ganet ... 4# ... C11B, Volume 8, May 23, 1725, f. 224v.

Isabeau contract of 1719: huit livres pour Chacune paire de pentures de deux pieds de long avec leurs gonds, C11B, Volume 4, March 7, 1719, f. 281. When Ganet was tendering for his February 24, 1725, general contract (now missing), Ganet’s offer revealed that he was adhering to the same specifications: la paire de pentures de deux pieds de long avec leurs gonds a ... 8# ... C11B, Volume 8, May 23, 1725, f. 224v.

See also: C11B, Volume 28, December 31, 1749, f. 331v.

F1A, Volume 29, 1730, May 8, 1730, f. 58.

B, Volume 54-2, June 20, 1730, f. 501; See also: B, Volume 54-2, June 20, 1730, ff. 501-501v, 502v.

C11B, Volume 11, October 15, 1731, ff. 92-93.

B, Volume 55-3, July 10, 1731, ff. 557-562 (pp. 687-698); C11B, Volume 11, July 20, 1730, f. 14v; C11B, Volume 11, November 30, 1730, ff. 99-100v; C11B, Volume 11, December 3, 1730, ff. 16-22; C11B, Volume 11, December 2, 1730, f. 75v.; See also: C11B, Volume 19, December 1, 1737, ff. 97-97v.

C11B, Volume 11, December 2, 1730, f. 75v.

C11B, Volume 15, January 10, 1734, f. 180.

C11B, Volume 12, November 11, 1730, f. 113.

C11B, Volume 12, November 29, 1731, ff. 104-109v.

C11B, Volume 18, May 1, 1736, ff. 181, 184-184v.

C11B, Volume 14, December 31, 1733, ff. 238-241v.

C11B, Volume 18, May 1, 1736, ff. 181, 186v.

C11B, Volume 15, January 10, 1734, f. 180.

C11B, Volume 18, May 1, 1736, ff. 181, 187v-188; G2, Volume 183, Bushel 25, Pièce 201, May 12, 1734.

C11B Volume 15, October 24, 1734, ff. 26-50. "Nous arpenteur à lisle Royalle certifions le present Etat des terrains concédées dans la ville de louisbourg veritables a Louisbourg le 24e 8bre 1734 . [signé] Vallée [contresigné :] "Veu, St Ovide de Brouillan Veu, Lenormant" 1734 Ile Royale M. de St Ovide, gouverneur et M. LeNormant, ordonnateur [See also for Confirmations: G1, Volume 466, Pièce 85, October 24, 1734] - Transcription: By Claude Desaulniers (January, 1997).

C11B Volume 15, October 24, 1734, f. 36v, "Nous arpenteur à lisle Royalle certifions le present Etat des terrains concédées dans la ville de louisbourg veritables a Louisbourg le 24e 8bre 1734 . [signé] Vallée [contresigné :] "Veu, St Ovide de Brouillan Veu, Lenormant" 1734 Ile Royale M. de St Ovide, gouverneur et M. LeNormant, ordonnateur [See also for Confirmations: G1, Volume 466, Pièce 85, October 24, 1734] - Transcription: By Claude Desaulniers (January, 1997); See also, G1, Volume 466, Pièce 85, October 24, 1734, ff. 2-41.

C11B, Volume 18, May 1, 1736, ff. 181, 190-190v.

G2, Volume 196, Dossier 89, June 1, 1736; G2, Volume 196, Dossier 89, July 1, 1736, p. 3; See also: C11B, Volume 19, December 4, 1737, ff. 98-101.

C11B, Volume 28, December 31, 1749, ff. 330v, 333v.

C11B, Volume 20, November 3, 1738, ff. 209, 210; C11B, Volume 21, September 3, 1739, ff. 168-172v.

B, Volume 65, April 30, 1737, ff. 460-461; C11B, Volume 19, October 24, 1737, ff. 20-22v; C11B, Volume 19, December 4, 1737, ff. 98-101.

C11B, Volume 19, December 4, 1737, ff. 98-101. See also: C11B, Volume 20, January 2, 1738, ff. 222v-223.

C11B, Volume 20, January 2, 1738, ff. 222v-223.

B, Volume 66, May 6, 1738, ff. 295-295v (ff. 13-13v); C11B, Volume 20, October 24, 1738, ff. 70-70v; C11B, Volume 20, November 1, 1738, f. 228.

C11B, Volume 20, November 1, 1738, f. 228.

C11B, Volume 28, December 31, 1749, f. 334.

C11B, Volume 21, August 2, 1739, f. 266v; C11B, Volume 21, September 3, 1739, ff. 170v-171, 172-172v; C11B, Volume 21, November 6, 1739, ff. 116-117; IE, Volume 131, October 26 1739, p. 143.

C11B, Volume 25, November 12, 1743, ff. 192-194.

C11B, Volume 21, August 2, 1739, f. 266v.

C11B, Volume 28, July 24, 1749, ff. 298, 300v-301.

C11B, Volume 22, January 20, 1740, ff. 141-141v; See also: IE, Volume 139, [1745], p. 235.

C11B, Volume 22, October 17, 1740, ff. 167-170; See also: B, Volume 71, February 21, 1740, f. 235.

Dépot des Fortifications, Order Number 201, October 30, 1744; C11B, Volume 28, December 31, 1749, f. 330.

Dépot des Fortifications, Order Number 203, October 30, 1744.

Dépot des Fortifications, Order Number 209, November 18, 1744; Dépot des Fortifications, Order Number 204, November 4, 1744; Map 1745-4; C11B, Volume 28, December 31, 1749, ff. 330-333v.

IE, Volume 139, [1745], p. 235.

Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), “First Journal - Anonymous,” October 17, 1745, pp. 50-51.

C11B, Volume 28, August 30, 1749, f. 319.

”Benjamin Green’s Journal,” Proceedings (American Antiquarian Society, 1909), June 21, 1745, p. 166; Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), Ninth Journal - Chaplain Stephen Williams Journal, October 16, 1745, p. 153.

Colonial Office 5, Volume 900, September 21, 1745, ff. 232-233v.

For many examples, see Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), Ninth Journal - Chaplain Stephen Williams Journal, July 21, p. 123, through November 17, 1745, p. 167; Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), Ninth Journal - Chaplain Stephen Williams Journal, August 15, 1745, p. 132.

Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), Ninth Journal - Chaplain Stephen Williams Journal, September 20, p. 147; September 26, 1745, p. 147; September 27, 1745, p. 147.

Admiralty 1, Volume 2007, July 5, 1746; C.O. 217 (Nova Scotia A 28), 1746, pp. 191-200; C11B, Volume 28, July 24, 1749, ff. 298, 300v-301; C11B, Volume 28, August 30, 1749, f. 319; C11B, Volume 28, December 31, 1749, ff. 330-330v; C11B, Volume 29, December 31, 1750, f. 294; Map 1745-23; Map 1745-23a; Map 1746-1; Map 1746-4; Map 1746-4; Map 1746-5; Map 1746-8; Map 1746-8a; Map1748-2.

IE, Volume 145, November 11, 1748, pp. 725, 735-743.

C11B, Volume 28, December 31, 1749, ff. 330-335.

C11B, Volume 28, August 30, 1749, ff. 303, 318v-319.

C11C, Volume 13, October 24, 1749 f. 67.

C11B, Volume 28, August 20, 1749, ff. 144-149; C11B, Volume 28, December 31, 1749, f. 331-331v, 332v, 333v.

C11B, Volume 29, September 29, 1750, ff. 267-267v.

C11B, Volume 29, December 31, 1750, ff. 276, 294-296.

B, Volume 91, May 19, 1750, f. 335v.

C11B, Volume 29, September 29, 1750, ff. 267-267v; C11B, Volume 29, November 3, 1750, f. 53; Map 1750-1.

C11B, Volume 31, November 20, 1751, ff. 118, 119v-120; Map 1750-1.

C11B, Volume 29, September 29, 1750, ff. 267-267v.

C11B, Volume 33, October 9, 1753, ff. 221, 228v, 232v; Archives Génie, Article 14, Pièce 28, 1751.

C11B, Volume 33, October 9, 1753, ff. 221, 228v, 232v.

C11B, Volume 30, November 12, 1751, ff. 259-261v; B, Volume 95, July 10, 1752, 286 (21). See also: Louis Effingham De Forest (editor), Louisbourg Journals 1745 (New York: The Society of Colonial Wars in the State of New York, 1932), Ninth Journal - Chaplain Stephen Williams Journal, September 20, p. 147; September 26, 1745, p. 147; September 27, 1745, p. 147.

C11B, Volume 30, November 12, 1751, ff. 260-260v.

C11B, Volume 30, November 12, 1751, f. 261.

C11B, Volume 31, November 20, 1751, ff. 118, 119v-120.

C11B, Volume 31, November 20, 1751, ff. 118, 119v-120.

C11B, Volume 32, October 18, 1752, ff. 262v-263.

C11B, Volume 31, November 20, 1751, ff. 118, 119v-120.

Thomas Pichon, Lettres sur le Cap Breton, 1760, Lettre XI; Pichon Papers, Tableau de l'etat actuel des missions, 1753, Number 40 (The document is said to be in Pichon's hand), NAC, MG18, F12.

C11B, Volume 36, October 1, 1756, ff. 145v-146; C11B, Volume 36, December 9, 1756, f. 263v.

Bibliothèque Génie, Manuscript 66, 1758; C11C, Volume 16, Pièce 28, July 6, 1758; C11B, Volume 38, July 7, 1758, ff. 36-38v.

C.O. 5, Volume 53, August 13, 1758, ff. 181, 191v, 192v.

Treasury 1, Volume 396, May 3, 1759.

Map 1759-1.

Gage Papers, June 22, 1768.

William Owen, “Narratives of American Voyages and Travels of Capt. William Owen, R.N. and the Settlement of the Island of Campobelle in the Bay of Fundy 1766-1771 (part 2) “ in Bulletin of the New York Public Library, Volume 35, No. 2 (February, 1931).

Gage Papers, June 22, 1768; C.O. 217, Volume 25, September 26, 1768, pp. 140-154, pp. 135-148.

Gage Papers, June 25, 1768.

.C.O. 5, Volume 53, August 13, 1758, ff. 182-192v.

Abraham Gesner, The Industrial Resources of Nova Scotia (Halifax, n.p. 1849), p. 302.

Abraham Gesner, Canadian Journal, New Series, 1862, Volume 7.

John Milne Gow, Cape Breton Illustrated ... (Toronto: W. Briggs, 1893), pp. 146-152.

J.S. McLennan, "A Notable Ruin: Louisbourg". A paper read before the Nova Scotia Historical Society, November 10, 1908, Published by the Society, 1909.

BASIS: Narrative Report: 1734 ("A" credibility).

BASIS: Narrative Report: 1734 ("A" credibility).

BASIS: Narrative Report: 1734 ("A" credibility).

BASIS: Narrative Report: 1734 ("A" credibility).

BASIS: Narrative Report: 1734 ("A" credibility).

BASIS: Narrative Report: 1721; 1722 ("A" credibility).

BASIS: Narrative Report: 1724; 1725; 1730 ("A" credibility).

: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1738 ("C" credibility).

BASIS: Narrative Report: 1739 ("A" credibility).

BASIS: Narrative Report: 1722 ("A" credibility).

BASIS: Narrative Report: 1722 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1749 ("A" credibility).

BASIS: Narrative Report: 1739 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1725 ("A" credibility).

BASIS: Narrative Report: 1726; 1730 ("A" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1726; 1730 ("C" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1726; 1744 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1724; 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1736 ("A" credibility).

BASIS: Narrative Report: 1726; 1730 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726;1730 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: ("A" credibility).

BASIS: Narrative Report: 1726; 1727 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726; 1752 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1752 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726; 1730; 1745 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1726; 1727; 1729; 1736; 1744 ("A" credibility).

BASIS: Narrative Report: 1726; 1729; 1744 ("A" credibility).

BASIS: Narrative Report: 1739; 1738 ("C" credibility); BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1739 ("C" credibility).

BASIS: Narrative Report: 1739 ("C" credibility); BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1726; 1736; 1745-1748 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726; 1727 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1724; 1726; 1730 ("A" credibility).

BASIS: Narrative Report: 1726; 1733; 1745 ("A" credibility); BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1726; 1730 ("A" credibility).

BASIS: Narrative Report: 1726; 1752 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility); BASIS: Narrative Report: 1752 ("C" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1739 ("A" credibility).

BASIS: Narrative Report: 1737; 1739 ("A" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1728 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility) .

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1728 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1728 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1739 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1733 ("A" credibility).

BASIS: Narrative Report: 1733 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1726; 1739 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1727 ("C" credibility).

BASIS: Narrative Report: 1730; 1749 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1735 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1727 ("A" credibility).

BASIS: Narrative Report: 1727 ("A" credibility).

BASIS: Narrative Report: 1727 ("C" credibility).

BASIS: Narrative Report: 1727; 1730; 1745 ("A" credibility).

BASIS: Narrative Report: 1749; 1750 ("A" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1744 ("C" credibility).

BASIS: Narrative Report: 1744 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1744 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1737; 1739 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1724, 1733 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1724 ("C" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1750 ("C" credibility).

BASIS: Narrative Report: 1750 ("C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1745-1748; 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1750 ("A" credibility).

BASIS: Narrative Report: 1727 ("C" credibility).

BASIS: Narrative Report: 1727 ("A” credibility).

BASIS: Narrative Report: 1744 ("A” credibility).

BASIS: Narrative Report: 1744 ("A” credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility) .

BASIS: Narrative Report: 1721-1724 ("A" credibility) .

BASIS: Narrative Report: 1721-1724 ("A" credibility) .

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1744; 1749 ("A" credibility).

BASIS: Narrative Report: 1727 ("C" credibility).

BASIS: Narrative Report: 1727 ("A credibility).

BASIS: Narrative Report: 1736 ("A" credibility).

BASIS: Narrative Report: 1728; 1736 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1728; 1749 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1730 ("A" credibility).

BASIS: Narrative Report: 1730; 1736 ("A" credibility).

BASIS: Narrative Report: 1745; 1745-1748 ("A" credibility).

BASIS: Narrative Report: 1736 ("A" credibility).

BASIS: Narrative Report: 1740 ("A" credibility).

BASIS: Narrative Report: 1745; 1745-1748 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1730 ("A" credibility).

BASIS: Narrative Report: 1724; 1726 ("A" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1724 ("A credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726; 1744 ("A" credibility).

BASIS: Narrative Report: 1726; 1752 ("A" credibility).

BASIS: Narrative Report: 1726 ("A and C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1728 ("C" credibility).

Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1724 ("A" credibility); BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1724 ("A credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1726 ("A credibility).

BASIS: Narrative Report: 1727; 1738 ("A credibility).

BASIS: Narrative Report: 1738 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1727; 1738; 1749; 1752 ("A" credibility).

BASIS: Narrative Report: 1726 ("A and C" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("C" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1724; 1725 ("A" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1725 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1745 ("C" credibility).

BASIS: Narrative Report: 1726; 1745 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1726; 1744; 1745 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: ("C" credibility).

BASIS: Narrative Report: 1724 ("A" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: ("C" credibility).

BASIS: Narrative Report: 1737 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1724 ("A" credibility).

BASIS: Narrative Report: 1731 ("A" credibility).

BASIS: Narrative Report: 1729 ("A" credibility).

BASIS: Narrative Report: 1729 ("C" credibility).

BASIS: Narrative Report: 1729 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1744 ("A" credibility).

BASIS: Narrative Report: 1752 ("A and C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("C" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1745 ("A" credibility).

BASIS: Narrative Report: 1731; 1745 ("A" credibility).

BASIS: Narrative Report: 1729; 1745; 1749 ("A" credibility).

BASIS: Narrative Report: 1726 ("A" credibility).

BASIS: Narrative Report: 1744; 1749 ("A" credibility).

BASIS: Narrative Report: 1752 ("A and C" credibility).

BASIS: Narrative Report: 1744; 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1749 ("A" credibility).

BASIS: Narrative Report: 1733; 1745-1748; 1749 ("A" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1721-1724; 1745 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1721-1724 ("A" credibility).

BASIS: Narrative Report: 1733 ("A" credibility).

BASIS: Narrative Report: 1731; 1745 ("A" credibility).

BASIS: Narrative Report: 1733 ("A" credibility).

BASIS: Narrative Report: 1729 ("C" credibility).

BASIS: Narrative Report: 1729 ("A" credibility).

BASIS: Narrative Report: 1728 ("A" credibility).

BASIS: Narrative Report: 1729 ("A" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).

BASIS: Narrative Report: 1752 ("A" credibility).

BASIS: Narrative Report: 1745 ("A" credibility).