Report 98-195

Exterior Shutter Research

Fortress of Louisbourg N.H.S.

Isle Royale

(Cape Breton Island)

and

Isle St. Jean

(Prince Edward Island)

1713 - 1758

By Eric Krause

(Krause House Info-

Research Solutions)

Volume Three

The Appendix

November 24, 1998

Terminology (Select)

Barre

For the purposes of this study, in French, the horizontal or diagonal (called écharpe) batten (i.e. nailer) of a board (set vertically) and batten shutter. In French, bar (as in contrevents barrés de trois barres or avec barres et écharpes.

Battant [batten, battan, batant]

In French, leaf (expressed or suggested), as of a single or two-leaf shutter (e.g. contrevent à deux battant or deux Doubles Contrevents et un Simple.)

Battens

For the purposes of this study, in English, the horizontal or diagonal battens (i.e. nailers) of a board (set vertically) and batten shutter.

Écharpe

See Barre.

Charpente or Colombage

See 1/2 Timber.

Emboîture

For the purposes of this study, the horizontal boards assembled, top and bottom, perpendicularly to the vertical boards of an emboîture shutter.

Measurements

12 Lignes	= 1 Pouce

12 Pouces = 1 Pied

6 Pieds 	= 1 Toise

----------------------------------------

1 Ligne	= .0888 Inches

1 Pouce 	= 1.0656 Inches

1 Pied 	= 1.066 Feet

1 Toise	= 6.396 Feet

Merisier

For the purposes of this study, a tree as described at Louisbourg as a type of birch (Espece de bouleau), and, possibly, as described in Acadia by the Baron de La Hontant as being “a hard and whitish wood, with a grey bark. Some of them are as tall as the loftiest oaks, and as big as a hogshead. This tree grows straight; it has an oval leaf, and is made use of in beams, rafters, and other carpenter's work.”

1/2 Timber

On Isle Royale and Isle St. Jean, a heavy wood-framed building type commonly described as charpente or, occasionally, as colombage, set upon a separate foundation.

Piquet

An earth-fast building type whose perimeter walls consisted of vertical logs.

Prices

12 Deniers	= 1 Sol

20 Sols 	= 1 Livre

Private Sector

For the purposes of this report, that construction sector funded by private individuals, and commonly called today domestic or vernacular.

Public Sector

For the purposes of this report, that construction sector funded by the King (Royal), commonly called today government or public works.

SHUTTER

Contrevent or Volet? See Volume 3, The Appendix, Christian Pouyez, “Preliminary Report on Outside Shutters.”

(I)

Incidence of Exterior Shutters

Interpretation by Building Type

(All Inclusive)

(A) 	1/2 Timber Buildings

Louisbourg: Block 18, Ancien Gouvernement (Public Building)

- Emboîture Shutter Type

Mirmiliguesche: Church (Public Building)

- Unspecified Shutter Type

Baleine: Church (Private Building)

- Unspecified Shutter Type

Port La Joye: Magasin Des Vivres (PROPOSAL - Public Building)

- Unspecified Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (PROPOSAL - Public Building)

- Unspecified Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- Emboîture Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 21 East, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 21 East, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West ?, Lot A OR Block 32 East?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Island Battery, Bakery (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Magasin Du Roi Additional Victualling Storehouse (English Public Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 33?, Lot C? OR Block 37?, Lot D? (Private Building)

- Emboîture Shutter Type

Louisbourg: Block 15, Lot C (1752) (And E) (Private Building)

- Batten Shutter Type

Louisbourg: Block 38 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 4, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West?, Lot C? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1756) - (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1757) - (Private Building)

- Unspecified Shutter Type

(B) 	Piquet Buildings

Louisbourg: Block 18, Guard House (Public Building)

- Batten Shutter Type

Louisbourg: Block 1, Beaucours (Public Building)

- Batten Shutter Type

Louisbourg: Block 18, Casernes (Public Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

- Batten Shutter Type

Louisbourg: Block 1, Dupont (Private Building)

- Unspecified Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Batten Shutter Type

Port Toulouse: Barracks, Officers' Lodgings, Chapel and Prison (Public Building)

- Batten Shutter Type

Port Toulouse: Chaplain and Surgeon’s Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Commander's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Chaplain and Surgeon's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Sub-Delegate's Lodging and Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Captain's Lodging (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Forge (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Bakery (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Aux Legumes (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Officers' Lodgings (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 34, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A ? OR Block Isle Du Quay, Lot E ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 ?, Lot ? OR Block 37 ?, Lot ? OR Block 38 ?, Lot ? (Private Building)

- Batten Shutter Type

Louisbourg: Block 32 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Dauphin Fauxbourg, Property [16] - (Private Building)

- Unspecified Shutter Type

(C) 	Masonry Buildings

Baye Royalle: Redoubt "A", Serving as a Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Redoubt "C", Serving as a Building for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "K", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "O", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Redoubt "I", Buildings for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Battery "B", Crenellated Guard House Serving as a Storehouse (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Redoubt "E", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Battery "L", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: King's Bastion “C”, Barracks (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Royal Battery “K”, Storehouse (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Royal Battery “K”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Powder Magazine (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Block 18, Prison and Dungeon (Public Building)

- [Possible Unspecified Shutter Type]

Louisbourg: King's Bastion, Barracks and Pavilions (Public Building)

- Emboîture Shutter Type

- Batten Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Block 13, Hospital (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, L’Ancien Magasin (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Angar [First Engineer’s House] - (Public Building)

- Emboîture Shutter Type

- Unspecified Shutter Type

Louisbourg: Royal Battery, Barracks and Towers (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Magasin Du Roi (Public Building)

- Emboîture Shutter Type

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Dauphin Demi-Bastion, Powder Magazin (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Guard House (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Block 23, Lot A (Public Building)

- Unspecified Shutter Type

Louisbourg: Island Battery, (Public Building)

- Unspecified Shutter Type

Louisbourg: Lighthouse (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks, Guardhouse, Prison and Detached Latrine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Commander's Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Officers' Lodgings, Chapel Storehouse, and Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Powder Magazine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Block 34, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block Presqu'Isle Du Quay, Lot A (Private Building)

- Unspecified Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks and Guardhouse and Prison and Latrines (Public Building)

- Emboîture Shutter Type

Port Toulouse: Magasin Des Vivres and Chapel and Officers Lodging and Bakery (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 33, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg, Block 1, Engineer’s House (Public Building)

- Emboîture Shutter Type

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Block 2, Lot H (PROPOSAL - Never-Built Private Building)

- Emboîture Shutter Type

Louisbourg: Island Battery, Guard House (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 2, Lot I (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Forge (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 2, Lot K (Private Building)

- Unspecified Shutter Type

(D) 	1/2 Timber/masonry Combination Buildings

Louisbourg: Block 13, Hospital (PROPOSAL - New Kitchen - Public Building)

- Unspecified Shutter Type

(E) 	Palisaded Redoubt Buildings

Isle Royale: Coast (PROPOSAL - Never-Built Sundry Public Buildings)

- Unspecified Shutter Type

(F) 	Unspecified Building Types

Louisbourg: Dauphin Fauxbourg, Property 5 ? (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Royal Battery, Barraque (Engineer's) - (Wooden Building - Public Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 20? (Private/public Building)

- Unspecified Shutter Type

* Niganiche (Private Building)

- Without Shutters

Louisbourg: Block 16, Lot A (Wooden House - Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot A (Wooden Storehouse - Private Building)

- Unspecified Shutter Type

St. Esprit (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 10 (Private Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 19, Lot B (Wooden Building - Private Building)

- Unspecified Shutter Type

Louisbourg: Barrachois, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: King’s Bastion, Pavilion (Masonry Public Building) AND/OR Block ?, Lot ? (Private Buildings)

- Unspecified Shutter Type

Louisbourg: Queen's Bastion, Barracks (1/2 Timber Public Building) OR Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

(II)

Summary of Actual Exterior Shutter Prices

(Does Not Include Proposed Prices or

Accounts Without Descriptive Details)

(A) 	By Date

■	5 livres (1716)

5 livres each 			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

[Note: For doors, at the same price, a 1718 description noted: of six pieds high x 3 pieds wide, tongued and grooved, planed two sides, with 3 battens]

■	8 livres (1716)

8 livres per set 		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

■	13 livres (1716)

5 livres each			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	16 livres (1716)

8 livres per set		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	20 livres (June 10, 1718; June 18, 1718)

20 livres per set		For 1 volet, 1 double hung window sash and its fixed window frame

■	10 sols (December 30, 1718)

10 sols each			For 1 turnbuckle per shutter for holding it open

■	15 sols (December 30, 1718)

15 sols per set		For 1 hook and 1 eye per shutter for holding it closed

■	1 livre (December 30, 1718)

1 livre each			For 1 fir board, 10 pieds long, 10-12 pouces wide, 1 pouce thick for shutters

■	 1 livre 10 sols (December 30, 1718)

1 livre 10 sols each 		For 1 [2-pouce thick] plank for shutters

[Note: For planks, at the same price, in 1718, a description noted: of pine, 2 pouces thick, 12 pieds long, 10-12 pouces wide]

■	3 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten, shutter, 4 pieds high x 3 pieds wide, planed both sides, with 2 battens

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

■	4 livres 5 sols (December 30, 1718)

3 livres each 			For 1 board and batten shutter

- Possible sizes:

- For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens;

- For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding it closed

10 sols each			For 1 turnbuckle for holding it open

■	17 livres (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	18 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	19 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	20 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	30 livres (March 7, 1719; 1719-1720; November 25, 1723)

30 livres

per square toise 		For emboîture shutters, of fir boards, 1 pouce thick, with oak emboîtures at both ends

■	2 livres (1719 - 1727)

2 livres each 			For 1 turnbuckle per shutter for holding it open

■	16 livres (1719-1727)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	2 livres ([1719?] - 1726; November 12, 1726; October 7, 1727)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 ordinary hooks per shutter for holding it closed

■	6 livres (1719-1727; 1725-1727)

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	10 livres (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	15 livres (1719-1727; 1719-1730)

15 livres each			For a shutter measuring 4 pieds 6 pouces x 4 pieds (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	27 livres 7 sols (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

■	2 livres (1719-1730)

2 livres per set		For 1 turnbuckle per shutter [for holding it open] For 1 hook per shutter [for holding it closed]

■	14 livres (1719-1730)

14 livres

per square toise 		For shutters, of boards, 1 pouce thick

■	30 livres (1719-1730; 1725-1727)

30 livres

per square toise 		For shutters, 1 pouce thick

■	3 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

■	6 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

3 livres each 			For 1 window sash

■	25 livres (February 24, 1725 - [See May 23, 1725])

25 livres

per square toise 		For emboîture shutters, of pine boards, 1 pouce thick, with oak emboîtures at both ends

■	10 sols (1725 - 1727)

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	10 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

■	17 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	34 livres 15 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

■	1 livre (November 1, 1727)

20 sols per set		For 2 hooks and 2 rings per shutter

■	12 livres 10 sols (November 1, 1727)

12 livres 10 sols each		For a shutter measuring 3 pieds 4 pouces x 5 pieds 6 pouces (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 25 livres per square toise

■	2 livres ? sols (1730)

2 livres ? sols	per set	For 2 keepers per shutter [for holding it open]

For 2 hooks per shutter [for holding it closed]

■	6 livres (1730)

6 livres per set		For 1 window sash, 1 fixed window frame, and 1 shutter

■	30 livres (December 3, 1731)

30 livres

per square toise 		For emboîture shutters with hardwood emboîtures

■	18 livres (September 10, 1733; September 23, 1733)

18 livres

per square toise		For emboîture shutters, assembled, of [vertical] pine boards, 1 full pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved, using only dry and sound wood

■	25 livres (September 18, 1733; [November 14, 1736])

25 livres

per square toise		For emboîture shutters, assembled, of [vertical] [pine] boards, 1-1/4 pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved

■	2 livres (September 23, 1733)

2 livres per set		For the keepers and hooks for shutters

■	2 livres (September 28, 1733)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 hooks per shutter for holding it closed

[Note: 1 hook [and keeper] cost 10 sols]

■	23 livres (September 28, 1733)

23 livres

per square toise		For emboîture shutters, assembled, of [vertical] boards, 1-1/4 pouce thick, with [horizontal] well-pegged emboîtures of oak or merisier [type of birch] at each end, everything planed, assembled tongued and grooved

■	18 sols (September 1, 1734)

18 sols each			For French boards sent from Paris for shutters

■	9 livres (September 1, 1734)

9 livres per set		For fashioning 1 two-leaf casement window sash with its two-leaf shutter

■	1 livre 12 sols (May 10, 1737)

1 livre 12 sols per set	2 hooks per shutter

2 keepers per shutter

■	20 livres (May 10, 1737)

20 livres

per square toise		For emboîture shutters of [vertical] pine boards, 1 full pouce thick, with [horizontal] emboîtures of merisier [type of birch] or oak at each end, everything well planed, assembled tongued and grooved, and dry and sound

■	20 livres (October 30, 1744)

20 livres

per square toise		For shutters, 1 pouce thick

■	12 sols (1749)

12 sols per set		For 1 hook and 1 eye per shutter

■	(circa) 2 livres (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

■	4 livres (1749)

4 livres per pair		For 2 small strap hinges with their pintles per shutter

■	(circa) 5 livres 4 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	(circa) 9 livres 16 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

4 livres per pair		For 2 small strap hinges with their pintles per shutter

12 sols per set		For 1 hook and eye per shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	1 livre 10 sols (December 31, 1749)

1 livres 10 sols each		For 1 shutter

■	(circa) 2 livres (December 31, 1749)

(circa) 2 livres each		For the nails and fashioning of 1 board and batten shutter made from salvaged materials

■	3 livres 10 sols (1749: September 3, 1751)

3 livres 10 sols per pair	For 1 pair of strap hinges with its pintles per shutter

■	6 livres (1749: September 3, 1751)

6 livres per pair		For strap hinges with their pintles for shutters

■	16 livres (1749: September 3, 1751)

16 livres

per square toise		For the wood, nails and fashioning of board and batten shutters, part tongued and grooved planed [2-pouce thick] planks, part tongued and grooved planed [1-pouce thick] boards, with its battens

■	12 sols (1749: September 9, 1751)

12 sols per set		For 1 hook and 1 eye per shutter

■	4 livres (1749: September 9, 1751)

4 livres per pair		For 1 pair of strap hinges per small shutter

■	5 livres (1749: September 9, 1751)

5 livres per pair		For 1 pair of strap hinges per shutter

■	10 livres 10 sols (1749: September 9, 1751)

10 livres 10 sols

per square toise		For the wood, nails and fashioning of board and batten shutters

■	22 livres (October 18, 1751)

22 livres

per square toise		For shutters, of planed tongued and grooved [local] boards, 1 pouce thick, [with emboîtures at both ends]

[Note: For doors, at the same price, a 1749 description noted: of local boards, 1 pouce thick, tongued and grooved, with emboîtures at both ends]

■	30 livres (January 12, 1752)

30 livres per set		For 1 window sash and 1 shutter

■	10 sols (April 19, 1753)

10 sols per set		For 1 hook with 1 eye per shutter

■	19 livres (April 19, 1753)

19 livres

per square toise		For custom fitted emboîture shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, fitted tongued and grooved style with oak emboîtures at both ends, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

19 livres

per square toise		For custom fitted board and batten shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, with [horizontal] and diagonal oak battens, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

■	6 livres (December 17, 1755)

6 livres per set		For a window sash of 4 glass panes and a small shutter with its peep-hole

(B) 	By Price

■	10 sols (December 30, 1718)

10 sols each			For 1 turnbuckle per shutter for holding it open

■	10 sols (1725 - 1727)

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	10 sols (April 19, 1753)

10 sols per set		For 1 hook with 1 eye per shutter

■	12 sols (1749)

12 sols per set		For 1 hook and eye per shutter

■	12 sols (1749: September 9, 1751)

12 sols per set		For 1 hook and 1 eye per shutter

■	15 sols (December 30, 1718)

15 sols per set		For 1 hook and 1 eye per shutter for holding it closed

■	18 sols (September 1, 1734)

18 sols each			For French boards sent from Paris for shutters

■	1 livre (December 30, 1718)

1 livre each			For 1 fir board, 10 pieds long, 10-12 pouces wide, 1 pouce thick for shutters

■	1 livre (November 1, 1727)

20 sols per set		For 2 hooks and 2 rings per shutter

■	 1 livre 10 sols (December 30, 1718)

1 livre 10 sols each 		For 1 [2-pouce thick] plank for shutters

[Note: For planks, at the same price, in 1718, a description noted: of pine, 2 pouces thick, 12 pieds long, 10-12 pouces wide]

■	1 livre 10 sols (December 31, 1749)

1 livres 10 sols each		For 1 shutter

■	1 livre 12 sols (May 10, 1737)

1 livre 12 sols per set	2 hooks per shutter

2 keepers per shutter

■	2 livres (1719 - 1727)

2 livres each 			For 1 turnbuckle per shutter for holding it open

■	2 livres ([1719?] - 1726; November 12, 1726; October 7, 1727)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 ordinary hooks per shutter for holding it closed

■	2 livres (1719-1730)

2 livres per set		For 1 turnbuckle per shutter [for holding it open] For 1 hook per shutter [for holding it closed]

■	2 livres ? sols (1730)

2 livres ? sols	per set	For 2 keepers per shutter [for holding it open]

For 2 hooks per shutter [for holding it closed]

■	2 livres (September 23, 1733)

2 livres per set		For the keepers and hooks for shutters

■	2 livres (September 28, 1733)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 hooks per shutter for holding it closed

[Note: 1 hook [and keeper] cost 10 sols]

■	(circa) 2 livres (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

■	(circa) 2 livres (December 31, 1749)

(circa) 2 livres each		For the nails and fashioning of 1 board and batten shutter made from salvaged materials

■	3 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten, shutter, 4 pieds high x 3 pieds wide, planed both sides, with 2 battens

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

■	3 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

■	3 livres 10 sols (1749: September 3, 1751)

3 livres 10 sols per pair	For 1 pair of strap hinges with its pintles per shutter

■

4 livres (1749)

4 livres per pair		For 2 small strap hinges with their pintles per shutter

■	4 livres (1749: September 9, 1751)

4 livres per pair		For 1 pair of strap hinges per small shutter

■	4 livres 5 sols (December 30, 1718)

3 livres each 			For 1 board and batten shutter

- Possible sizes:

- For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens;

- For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding it closed

10 sols each			For 1 turnbuckle for holding it open

■	5 livres (1716)

5 livres each 			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

[Note: For doors, at the same price, a 1718 description noted: of six pieds high x 3 pieds wide, tongued and grooved, planed two sides, with 3 battens]

■	5 livres (1749: September 9, 1751)

5 livres per pair		For 1 pair of strap hinges per shutter

■	(circa) 5 livres 4 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	6 livres (1719-1727; 1725-1727)

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	6 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

3 livres each 			For 1 window sash

■	6 livres (1730)

6 livres per set		For 1 window sash, 1 fixed window frame, and 1 shutter

■	6 livres (1749: September 3, 1751)

6 livres per pair		For strap hinges with their pintles for shutters

■	6 livres (December 17, 1755)

6 livres per set		For a window sash of 4 glass panes and a small shutter with its peep-hole

■	8 livres (1716)

8 livres per set 		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

■	9 livres (September 1, 1734)

9 livres per set		For fashioning 1 two-leaf casement window sash with its two-leaf shutter

■	(circa) 9 livres 16 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

4 livres per pair		For 2 small strap hinges with their pintles per shutter

12 sols per set		For 1 hook and eye per shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	10 livres (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	10 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

■	10 livres 10 sols (1749: September 9, 1751)

10 livres 10 sols

per square toise		For the wood, nails and fashioning of board and batten shutters

■	12 livres 10 sols (November 1, 1727)

12 livres 10 sols each		For a shutter measuring 3 pieds 4 pouces x 5 pieds 6 pouces (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 25 livres per square toise

■	13 livres (1716)

5 livres each			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	14 livres (1719-1730)

14 livres

per square toise 		For shutters, of boards, 1 pouce thick

■	15 livres (1719-1727; 1719-1730)

15 livres each			For a shutter measuring 4 pieds 6 pouces x 4 pieds (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	16 livres (1716)

8 livres per set		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	16 livres (1719-1727)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	16 livres (1749: September 3, 1751)

16 livres

per square toise		For the wood, nails and fashioning of board and batten shutters, part tongued and grooved planed [2-pouce thick] planks, part tongued and grooved planed [1-pouce thick] boards, with its battens

■	17 livres (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	17 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	18 livres (September 10, 1733; September 23, 1733)

18 livres

per square toise		For emboîture shutters, assembled, of [vertical] pine boards, 1 full pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved, using only dry and sound wood

■	18 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	19 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	19 livres (April 19, 1753)

19 livres

per square toise		For custom fitted emboîture shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, fitted tongued and grooved style with oak emboîtures at both ends, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

19 livres

per square toise		For custom fitted board and batten shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, with [horizontal] and diagonal oak battens, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

■	20 livres (June 10, 1718; June 18, 1718)

20 livres per set		For 1 volet, 1 double hung window sash and its fixed window frame

■	20 livres (May 10, 1737)

20 livres

per square toise		For emboîture shutters of [vertical] pine boards, 1 full pouce thick, with [horizontal] emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved, and dry and sound

■	20 livres (October 30, 1744)

20 livres

per square toise		For shutters, 1 pouce thick

■	20 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	22 livres (October 18, 1751)

22 livres

per square toise		For shutters, of planed tongued and grooved [local] boards, 1 pouce thick, [with emboîtures at both ends]

[Note: For doors, at the same price, a 1749 description noted: of local boards, 1 pouce thick, tongued and grooved, with emboîtures at both ends]

■	23 livres (September 28, 1733)

23 livres

per square toise		For emboîture shutters, assembled, of [vertical] boards, 1-1/4 pouces thick, with [horizontal] well-pegged emboîtures of oak or merisier [type of birch] at each end, everything planed, assembled tongued and grooved

■	25 livres (February 24, 1725 - [See May 23, 1725])

25 livres

per square toise 		For emboîture shutters, of pine boards, 1 pouce thick, with oak emboîtures at both ends

■	25 livres (September 18, 1733; [November 14, 1736])

25 livres

per square toise		For emboîture shutters, assembled, of [vertical] [pine] boards, 1-1/4 pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved

■	27 livres 7 sols (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

■	30 livres (March 7, 1719; 1719-1720; November 25, 1723)

30 livres

per square toise 		For emboîture shutters, of fir boards, 1 pouce thick, with oak emboîtures at both ends

■	30 livres (1719-1730; 1725-1727)

30 livres

per square toise 		For shutters, 1 pouce thick

■	30 livres (December 3, 1731)

30 livres

per square toise 		For emboîture shutters with hardwood emboîtures

■	30 livres (January 12, 1752)

30 livres per set		For 1 window sash and 1 shutter

■	34 livres 15 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

(C) 	By Subject

(1) 	Shutters

(With Or Without Their Hardware)

■	1 livres 10 sols (December 31, 1749)

1 livres 10 sols each		For 1 shutter

■	(circa) 2 livres (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

■	(circa) 2 livres (December 31, 1749)

(circa) 2 livres each		For the nails and fashioning of 1 board and batten shutter made from salvaged materials

■	3 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten, shutter, 4 pieds high x 3 pieds wide, planed both sides, with 2 battens

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

■	3 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

■	4 livres 5 sols (December 30, 1718)

3 livres each 			For 1 board and batten shutter

- Possible sizes:

- For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens;

- For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding it closed

10 sols each			For 1 turnbuckle for holding it open

■	5 livres (1716)

5 livres each 			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

[Note: For doors, at the same price, a 1718 description noted: of six pieds high x 3 pieds wide, tongued and grooved, planed two sides, with 3 battens]

■	(circa) 5 livres 4 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	6 livres (November 21, 1721)

3 livres each 			For 1 board and batten shutter, with its 2 battens

3 livres each 			For 1 window sash

■	6 livres (1730)

6 livres per set		For 1 window sash, 1 fixed window frame, and 1 shutter

■	6 livres (December 17, 1755)

6 livres per set		For a window sash of 4 glass panes and a small shutter with its peep-hole

■	8 livres (1716)

8 livres per set		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

■	9 livres (September 1, 1734)

9 livres per set		For fashioning 1 two-leaf casement window sash with its two-leaf shutter

■	(circa) 9 livres 16 sols (1749)

(circa) 2 livres each		For the wood, nails and fashioning of 1 [board and batten] shutter

4 livres per pair		For 2 small strap hinges with their pintles per shutter

12 sols per set		For 1 hook and eye per shutter

(circa) 3 livres 4 sols

each			For 1 window sash, of 4 panes, including the glass

■	10 livres (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	10 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

■	10 livres 10 sols (1749: September 9, 1751)

10 livres 10 sols

per square toise		For the wood, nails and fashioning of board and batten shutters

■	12 livres 10 sols (November 1, 1727)

12 livres 10 sols each		For a shutter measuring 3 pieds 4 pouces x 5 pieds 6 pouces (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 25 livres per square toise

■	13 livres (1716)

5 livres each			For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	14 livres (1719-1730)

14 livres

per square toise 		For shutters, of boards, 1 pouce thick

■	15 livres (1719-1727; 1719-1730)

15 livres each			For a shutter measuring 4 pieds 6 pouces x 4 pieds (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 30 livres per square toise

■	16 livres (1716)

8 livres per set		For 1 volet with its hardware

[Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware]

8 livres per set		For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame

■	16 livres (1719-1727)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	16 livres (1749: September 3, 1751)

16 livres

per square toise		For the wood, nails and fashioning of board and batten shutters, part tongued and grooved planed [2-pouce thick] planks, part tongued and grooved planed [1-pouce thick] boards, with its battens

■	17 livres (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	17 livres 8 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	18 livres (September 10, 1733; September 23, 1733)

18 livres

per square toise		For emboîture shutters, assembled, of [vertical] pine boards, 1 full pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved, using only dry and sound wood

■	18 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

14 livres per set		For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame

■	19 livres (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	19 livres (April 19, 1753)

19 livres

per square toise		For custom fitted emboîture shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, fitted tongued and grooved style with oak emboîtures at both ends, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

19 livres

per square toise		For custom fitted board and batten shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, with [horizontal] and diagonal oak battens, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers

■	20 livres (June 10, 1718; June 18, 1718)

20 livres per set		For 1 volet, 1 double hung window sash and its fixed window frame

■	20 livres (May 10, 1737)

20 livres

per square toise		For emboîture shutters of [vertical] pine boards, 1 full pouce thick, with [horizontal] emboîtures of merisier [type of birch] or oak at each end, everything well planed, assembled tongued and grooved, and dry and sound

■	20 livres (October 30, 1744)

20 livres

per square toise		For shutters, 1 pouce thick

■	20 livres 5 sols (December 30, 1718)

3 livres each 			For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens

15 sols per set		For 1 hook and 1 eye for holding the shutter closed

10 sols each			For 1 turnbuckle for holding the shutter open

16 livres per set		For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame

■	22 livres (October 18, 1751)

22 livres

per square toise		For shutters, of planed tongued and grooved [local] boards, 1 pouce thick, [with emboîtures at both ends]

[Note: For doors, at the same price, a 1749 description noted: of local boards, 1 pouce thick, tongued and grooved, with emboîtures at both ends]

■	23 livres (September 28, 1733)

23 livres

per square toise		For emboîture shutters, assembled, of [vertical] boards, 1-1/4 pouces thick, with [horizontal] well-pegged emboîtures of oak or merisier [type of birch] at each end, everything planed, assembled tongued and grooved

■	25 livres (February 24, 1725 - [See May 23, 1725])

25 livres

per square toise 		For emboîture shutters, of pine boards, 1 pouce thick, with oak emboîtures at both ends

■	25 livres (September 18, 1733; [November 14, 1736])

25 livres

per square toise		For emboîture shutters, assembled, of [vertical] [pine] boards, 1-1/4 pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved

■	27 livres 7 sols (1719-1727; 1719-1730)

10 livres each			For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

■	30 livres (March 7, 1719; 1719-1720; November 25, 1723)

30 livres

per square toise 		For emboîture shutters, of fir boards, 1 pouce thick, with oak emboîtures at both ends

■	30 livres (1719-1730; 1725-1727)

30 livres

per square toise 		For shutters, 1 pouce thick

■	30 livres (December 3, 1731)

30 livres

per square toise 		For emboîture shutters with hardwood emboîtures

■	30 livres (January 12, 1752)

30 livres per set		For 1 window sash and 1 shutter

■	34 livres 15 sols 4 deniers (1725-1727)

10 livres 8 sols

4 deniers each		For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

17 livres 7 sols

each				For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise

(2) 	Hardware

■	10 sols (December 30, 1718)

10 sols each			For 1 turnbuckle per shutter for holding it open

■	10 sols (1725 - 1727)

10 sols per set		For 1 eye [and 1 hook] per shutter for holding it open

10 sols per set		For 1 hook [and 1 eye] per shutter for holding it closed

■	10 sols (April 19, 1753)

10 sols per set		For 1 hook with 1 eye per shutter

■	12 sols (1749)

12 sols per set		For 1 hook and eye per shutter

■	12 sols (1749: September 9, 1751)

12 sols per set		For 1 hook and 1 eye per shutter

■	15 sols (December 30, 1718)

15 sols per set		For 1 hook and 1 eye per shutter for holding it closed

■	1 livre (November 1, 1727)

20 sols per set		For 2 hooks and 2 rings per shutter

■	1 livre 12 sols (May 10, 1737)

1 livre 12 sols	per set	2 hooks per shutter

2 keepers per shutter

■	2 livres (1719 - 1727)

2 livres each 			For 1 turnbuckle per shutter for holding it open

■	2 livres ([1719?] - 1726; November 12, 1726; October 7, 1727)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 ordinary hooks per shutter for holding it closed

■	2 livres (1719-1730)

2 livres per set		For 1 turnbuckle per shutter [for holding it open] For 1 hook per shutter [for holding it closed]

■	2 livres ? sols (1730)

2 livres ? sols	per set	For 2 keepers per shutter [for holding it open]

For 2 hooks per shutter [for holding it closed]

■	2 livres (September 23, 1733)

2 livres per set		For the keepers and hooks for shutters

■	2 livres (September 28, 1733)

2 livres per set		For 2 keepers per shutter for holding it open

For 2 hooks per shutter for holding it closed

[Note: 1 hook [and keeper] cost 10 sols]

■	3 livres 10 sols (1749: September 3, 1751)

3 livres 10 sols per pair	For 1 pair of strap hinges with its pintles per shutter

■	4 livres (1749)

4 livres per pair		For 2 small strap hinges with their pintles per shutter

■	4 livres (1749: September 9, 1751)

4 livres per pair		For 1 pair of strap hinges per small shutter

■	5 livres (1749: September 9, 1751)

5 livres per pair		For 1 pair of strap hinges per shutter

■	6 livres (1719-1727; 1725-1727)

6 livres per pair		For 1-1/2 pieds long strap hinges with their pintles for shutters

■	6 livres (1749: September 3, 1751)

6 livres per pair		For strap hinges with their pintles for shutters

(3) 	Associated Shutter Materials

■	18 sols (September 1, 1734)

18 sols each			For French boards sent from Paris for shutters

■	1 livre (December 30, 1718)

1 livre each			For 1 fir board, 10 pieds long, 10-12 pouces wide, 1 pouce thick for shutters

■	 1 livre 10 sols (December 30, 1718)

1 livre 10 sols each 		For 1 [2-pouce thick] plank for shutters

[Note: For planks, at the same price, in 1718, a description noted: of pine, 2 pouces thick, 12 pieds long, 10-12 pouces wide]

(III)

Styles and Detailing of Exterior Shutters:

Interpretation

(Based on the Summary of Actual Prices

which Does Not Include Proposed Prices

or

Accounts Without Descriptive Details)

(A) 	By Item/Set

| PRICE | DATE | BATTEN | EMBOÎTURE |
| --- | --- | --- | --- |
| 1 livres 10 sols each | December 31, 1749 (Louisbourg, Block 1, Engineer’s House) | - For 1 shutter |  |
| (circa) 2 livres each | 1749 (Port Toulouse, Chapel) | - For the wood, nails and fashioning of 1 [board and batten] shutter |  |
| (circa) 2 livres each | December 31, 1749 (Louisbourg, King’s Bastion, Barracks) | - For the nails and fashioning of 1 board and batten shutter made from salvaged materials |  |
| 3 livres each | December 30, 1718 (Louisbourg, Block 18, Guard House, Prison and Dungeon) | - For 1 tongued & grooved board and batten, shutter, 4 pieds high x 3 pieds wide, planed both sides, with 2 battens |  |
| 3 livres each | December 30, 1718 (Louisbourg, Block 18, Ancien Gouvernement) | - For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens |  |
| 3 livres each | December 30, 1718 (Louisbourg, Block 18, Beaucours) | - For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens |  |
| 3 livres each | November 21, 1721 (Louisbourg, Block 5, Lot A) | - For 1 board and batten shutter, with its 2 battens |  |
| 4 livres 5 sols per set - 3 livres each - 15 sols per set - 10 sols each | December 30, 1718 (Louisbourg: Block 18, Ancien Gouvernement; Block 18, Beaucours) | - For 1 board and batten shutter (possible sizes: For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens; For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens) - For 1 hook and 1 eye for holding it closed - For 1 turnbuckle for holding it open |  |
| 5 livres each | 1716 (Louisbourg, Redoubt “C” Serving as a Building for the Troops; Port Toulouse, Redoubt “I” Buildings for the Troops; Port Dauphin, Redoubt “E” Barracks) | - For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide [Note: For doors, at the same price, a 1718 description noted: of six pieds high x 3 pieds wide, tongued and grooved, planed two sides, with 3 battens] |  |
| (circa) 5 livres 4 sols per set - (circa) 2 livres each - (circa) 3 livres 4 sols each | 1749 (Port Toulouse, Chapel) | - For the wood, nails and fashioning of 1 [board and batten] shutter - For 1 window sash, of 4 panes, including the glass |  |
| 6 livres per set - 3 livres each - 3 livres each | November 21, 1721 (Louisbourg, Block 5, Lot A) | - For 1 board and batten shutter, with its 2 battens - For 1 window sash |  |
| 6 livres per set | 1730 (Baleine: Church) | - For 1 window sash - For 1 fixed window frame - For 1 shutter |  |
| 6 livres per set | December 17, 1755 (Louisbourg, King’s Bastion and/or Block ?, Lot ?) | - For a window sash of 4 glass panes - For a small shutter with its peep-hole |  |
| 8 livres per set | 1716 (Baye Royalle, Redoubt “A” Serving as a Guard House; Louisbourg, Battery “K” Barracks; Louisbourg, Battery “O” Guard House; Port Toulouse, Battery “B” Crenellated Guard House serving as a Storehouse; Port Dauphin, Battery”L” Guard House | - For 1 volet with its hardware [Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware] |  |
| 9 livres per set | September 1, 1734 (Louisbourg, Block 33, Lot C) | - For fashioning 1 two-leaf casement window sash - For fashioning its two-leaf shutter |  |
| (circa) 9 livres 16 sols per set - (circa) 2 livres each - 4 livres per pair - 12 sols per set - (circa) 3 livres 4 sols each | 1749 (Port Toulouse, Chapel) | - For the wood, nails and fashioning of 1 [board and batten] shutter - For 2 small strap hinges with their pintles per shutter - For 1 hook and eye per shutter - For 1 window sash, of 4 panes, including the glass |  |
| 10 livres each | 1719-1727; 1719-1730 (Louisbourg, Block 13, Hospital) |  | - For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise |
| 10 livres 8 sols 4 deniers each | 1725-1727 (Louisbourg, King’s Bastion: Pavilion and Barracks) |  | - For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise |
| 12 livres 10 sols each | November 1, 1727 (Louisbourg, Block 1, Magasin du Roi) |  | - For a shutter measuring 3 pieds 4 pouces x 5 pieds 6 pouces (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 25 livres per square toise |
| 13 livres per set - 5 livres each - 8 livres per set | 1716 (Louisbourg, Redoubt “C” Serving as a Building for the Troops; Port Toulouse, Redoubt “I” Buildings for the Troops; Port Dauphin, Redoubt “E” Barracks) | - For 1 volet , 5 pieds 6 pouces high x 3 pieds 6 pouces wide [Note: For doors, at the same price, a 1718 description noted: of six pieds high x 3 pieds wide, tongued and grooved, planed two sides, with 3 battens] - For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame |  |
| 15 livres each | 1719-1727; 1719-1730 (Louisbourg, Block 1, L’Ancien Magasin) |  | - For a shutter measuring 4 pieds 6 pouces x 4 pieds (i.e. 0 square toise 3 pieds 0 pouces 0 lignes) @ 30 livres per square toise |
| 16 livres per set - 8 livres per set - 8 livres per set | 1716 (Baye Royalle, Redoubt “A” Serving as a Guard House; Louisbourg, Battery “K” Barracks; Louisbourg, Battery “O” Guard House; Port Toulouse, Battery “B” Crenellated Guard House serving as a Storehouse; Port Dauphin, Battery”L” Guard House | - For 1 volet with its hardware [Note: Most probable - a 5 livre shutter with 3 livre strap hinge hardware] - For 1 double hung window sash, 5 pieds high x 3 pieds 6 wide, with its fixed window frame |  |
| 16 livres per set - 10 livres each - 6 livres per pair | 1719-1727 (Louisbourg, Block 13, Hospital) |  | - For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise -For 1-1/2 pieds long strap hinges with their pintles for shutters |
| 17 livres per set - 3 livres each - 14 livres per set | December 30, 1718 (Block 18, Beaucours) | - For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens - For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame |  |
| 17 livres 8 sols 4 deniers per set - 10 livres 8 sols 4 deniers each - 6 livres per pair - 10 sols per set - 10 sols per set | 1725-1727 (Louisbourg, King’s Bastion: Pavilion and Barracks) |  | - For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise - For 1-1/2 pieds long strap hinges with their pintles for shutters - For 1 eye [and 1 hook] per shutter for holding it open - For 1 hook [and 1 eye] per shutter for holding it closed |
| 18 livres 5 sols per set - 3 livres each - 15 sols per set - 10 sols each - 14 livres per set | December 30, 1718 (Block 18, Beaucours) | - For 1 tongued and grooved board and batten shutter, 5 pieds high x 2 pieds 6 pouces wide, planed, with 3 battens - For 1 hook and 1 eye for holding the shutter closed - For 1 turnbuckle for holding the shutter open - For 1 double casement window, 5 pieds high x 2 pieds 6 pouces wide, of fir, with its fixed frame |  |
| 19 livres per set - 3 livres each - 16 livres per set | December 30, 1718 (Louisbourg, Block 18, Ancien Gouvernement) | - For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens - For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame |  |
| 20 livres per set | June 10, 1718; June 18, 1718 (Louisbourg: Royal Battery “K”, Storehouse and Guard House; Island Battery “I”, Powder Magazine and Guard House | - For 1 volet - For 1 double hung window sash and its fixed window frame |  |
| 20 livres 5 sols per set - 3 livres each - 15 sols per set - 10 sols each - 16 livres per set | December 30, 1718 (Block 18, L’Ancien Gouvernement) | - For 1 tongued & grooved board and batten shutter, 5 pieds high x 3 pieds wide, of fir, planed both sides, with 3 battens - For 1 hook and 1 eye for holding the shutter closed - For 1 turnbuckle for holding the shutter open - For 1 double casement window, 5 pieds high x 3 pieds wide, of fir, with its fixed frame |  |
| 27 livres 7 sols per set - 10 livres each - 17 livres 7 sols each | 1719-1727; 1719-1730 (Louisbourg, Block 13, Hospital) |  | - For a shutter measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 30 livres per square toise - For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise |
| 30 livres per set | January 12, 1752 (Isle Royale: Coast) |  | - For 1 window sash - For 1 shutter |
| 34 livres 15 sols 4 deniers per set - 10 livres 8 sols 4 deniers each - 6 livres per pair - 10 sols per set - 10 sols per set - 17 livres 7 sols each | 1725-1727 (Louisbourg: King’s Bastion: Pavilion and Barracks) |  | - For a 1-pouce thick shutter measuring 4 pieds 2 pouces x 3 pieds (i.e. 0 square toise 2 pieds 1 pouce 0 lignes) @ 30 livres per square toise - For 1-1/2 pieds long strap hinges with their pintles for shutters - For 1 eye [and 1 hook] per shutter for holding it open - For 1 hook [and 1 eye] per shutter for holding it closed - For a window sash measuring 4 pieds x 3 pieds (i.e. 0 square toise 2 pieds 0 pouces 0 lignes) @ 52 livres per square toise |

(B) 	By Square Toise

| PRICE | DATE | BATTEN | EMBOÎTURE |
| --- | --- | --- | --- |
| 10 livres 10 sols per square toise | 1749: September 9, 1751 (Port Toulouse: Commander’s Lodging; Barracks, Officers’ Lodgings, Chapel and Prison; Chaplain and Surgeon’s Lodging) | - For the wood, nails and fashioning of board and batten shutters |  |
| 14 livres per square toise | 1719-1730 (Louisbourg, Block 1, Angar [First Engineer’s House] | - For shutters, of boards, 1 pouce thick |  |
| 16 livres per square toise | 1749: September 3, 1751 (Port La Joye, Commander’s Lodging) | - For the wood, nails and fashioning of board and batten shutters, part tongued and grooved planed [2-pouce thick] planks, part tongued and grooved planed [1-pouce thick] boards, with its battens |  |
| 18 livres per square toise | September 10, 1733; September 23, 1733 (Port La Joye: Lieutenant de Roi and Officers’ Lodgings; Magasin des Vivres |  | - For emboîture shutters, assembled, of [vertical] pine boards, 1 full pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved, using only dry and sound wood |
| 19 livres per square toise | April 19, 1753 (Louisbourg: General Contract) |  | - For custom fitted emboîture shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, fitted tongued and grooved style with oak emboîtures at both ends, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers |
| 19 livres per square toise | April 19, 1753 (Louisbourg: General Contract) | - For custom fitted board and batten shutters, of planed two sides, tongued and grooved pine boards, 1 pouce thick, with [horizontal] and diagonal oak battens, with the wood cut during the good season, without wane, square cut, and fashioned and assembled by good workers |  |
| 20 livres per square toise | May 10, 1737 (Louisbourg: General Contract) |  | - For emboîture shutters of [vertical] pine boards, 1 full pouce thick, with [horizontal] emboîtures of merisier [type of birch] or oak at each end, everything well planed, assembled tongued and grooved, and dry and sound |
| 20 livres per square toise | October 30, 1744 (Louisbourg: Island Battery, Bakery; Island Battery, Guard House) |  | - For shutters, 1 pouce thick |
| 22 livres per square toise | October 18, 1751 (Louisbourg: Block 33?, Lot C? or Block 37?, Lot D?) |  | - For shutters, of planed tongued and grooved [local] boards, 1 pouce thick, [with emboîtures at both ends] [Note: For doors, at the same price, a 1749 description noted: of local boards, 1 pouce thick, tongued and grooved, with emboîtures at both ends] |
| 23 livres per square toise | September 28, 1733 (Port Toulouse: Commander’s Lodging; Barracks and Guardhouse and Prison and Latrines; Magasin des Vivres and Chapel and Officers Lodging and Bakery) |  | - For emboîture shutters, assembled, of [vertical] boards, 1-1/4 pouce thick, with [horizontal] well-pegged emboîtures of oak or merisier [type of birch] at each end, everything planed, assembled tongued and grooved |
| 25 livres per square toise | February 24, 1725 [See May 23, 1725] (Louisbourg: Tendered Prices) |  | - For emboîture shutters, of pine boards, 1 pouce thick, with oak emboîtures at both ends |
| 25 livres per square toise | September 18, 1733; [November 14, 1736] (Port Toulouse: Commander’s Lodging; Barracks and Guardhouse and Prison and Latrines; Magasin des Vivres and Chapel and Officers Lodging and Bakery) |  | - For emboîture shutters, assembled, of [vertical] [pine] boards, 1-1/4 pouce thick, with [horizontal] pegged emboîtures of oak or merisier [type of birch] at each end, everything well planed, assembled tongued and grooved |
| 30 livres per square toise | March 7, 1719; 1719-1720; November 25, 1723 (Louisbourg: King’s Bastion, Barracks) |  | - For emboîture shutters, of fir boards, 1 pouce thick, with oak emboîtures at both ends |
| 30 livres per square toise | 1719-1730; 1725-1727 (Louisbourg: King’s Bastion, Barracks and Pavilion (Left); Block 1, L’Ancien Magasin; Block 13, Hospital |  | - For shutters, 1 pouce thick |
| 30 livres per square toise | December 3, 1731 (Louisbourg: Lighthouse) |  | - For emboîture shutters with hardwood emboîtures |

(IV)

Incidence of Exterior Shutters:

Interpretation by Shutter Type

(All Inclusive)

(A) 	By Shutter Type and Building Sector

(1)	Emboîture Shutter Type

(i) 	1/2 Timber Buildings

Louisbourg: Block 18, Ancien Gouvernement (Public Building)

- Emboîture Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- Emboîture Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (Public Building)

- Emboîture Shutter Type

Louisbourg: Island Battery, Bakery (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 33?, Lot C? OR Block 37?, Lot D? (Private Building)

- Emboîture Shutter Type

(ii)	Piquet Buildings

N/A

(iii)	Masonry Buildings

Louisbourg: King's Bastion “C”, Barracks (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: King's Bastion, Barracks and Pavilions (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 13, Hospital (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, L’Ancien Magasin (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Angar [First Engineer’s House] - (Public Building)

- Emboîture Shutter Type

Louisbourg: Royal Battery, Barracks and Towers (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Magasin Du Roi (Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Powder Magazin (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Guard House (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Lighthouse (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks, Guardhouse, Prison and Detached Latrine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Commander's Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Officers' Lodgings, Chapel Storehouse, and Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Powder Magazine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks and Guardhouse and Prison and Latrines (Public Building)

- Emboîture Shutter Type

Port Toulouse: Magasin Des Vivres and Chapel and Officers Lodging and Bakery (Public Building)

- Emboîture Shutter Type

Louisbourg, Block 1, Engineer’s House (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 2, Lot H (PROPOSAL - Never-Built Private Building)

- Emboîture Shutter Type

Louisbourg: Island Battery, Guard House (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Forge (Public Building)

- Emboîture Shutter Type

(2) 	Batten Shutter Type

(i) 	1/2 Timber Buildings

Louisbourg: Block 15, Lot C (1752) (And E) (Private Building)

- Batten Shutter Type

(ii) 	Piquet Buildings

Louisbourg: Block 18, Guard House (Public Building)

- Batten Shutter Type

Louisbourg: Block 1, Beaucours (Public Building)

- Batten Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Batten Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Batten Shutter Type

Port Toulouse: Barracks, Officers' Lodgings, Chapel and Prison (Public Building)

- Batten Shutter Type

Port Toulouse: Chaplain and Surgeon’s Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Commander's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Chaplain and Surgeon's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Sub-Delegate's Lodging and Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Captain's Lodging (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Forge (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Bakery (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Aux Legumes (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Officers' Lodgings (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

Louisbourg: Block 32 ?, Lot ? OR Block 37 ?, Lot ? OR Block 38 ?, Lot ? (Private Building)

- Batten Shutter Type

(iii)	Masonry Buildings

Baye Royalle: Redoubt "A", Serving as a Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Redoubt "C", Serving as a Building for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "K", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "O", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Redoubt "I", Buildings for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Battery "B", Crenellated Guard House Serving as a Storehouse (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Redoubt "E", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Battery "L", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: King's Bastion, Barracks and Pavilions (Public Building)

- Batten Shutter Type (Aberrant 1749 Repair Work)

(3) 	Unspecified Shutter Type

(i) 	1/2 Timber Buildings

Mirmiliguesche: Church (Public Building)

- Unspecified Shutter Type

Baleine: Church (Private Building)

- Unspecified Shutter Type

Port La Joye: Magasin Des Vivres (PROPOSAL - Public Building)

- Unspecified Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (PROPOSAL - Public Building)

- Unspecified Shutter Type

Louisbourg: Block 21 East, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 21 East, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West ?, Lot A OR Block 32 East?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Magasin Du Roi Additional Victualling Storehouse (English Public Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 38 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 4, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West?, Lot C? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1756) - (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1757) - (Private Building)

- Unspecified Shutter Type

(ii) 	Piquet Buildings

Louisbourg: Block 18, Casernes (Public Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Dupont (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 34, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A ? OR Block Isle Du Quay, Lot E ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Dauphin Fauxbourg, Property [16] - (Private Building)

- Unspecified Shutter Type

(iii) 	Masonry Buildings

Louisbourg: Royal Battery “K”, Storehouse (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Royal Battery “K”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Powder Magazine (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Block 18, Prison and Dungeon (Public Building)

- [Possible Unspecified Shutter Type]

Louisbourg: Block 1, Angar [First Engineer’s House] - (Public Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Magasin Du Roi (Public Building)

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Block 23, Lot A (Public Building)

- Unspecified Shutter Type

Louisbourg: Island Battery, (Public Building)

- Unspecified Shutter Type

Louisbourg: Block 34, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block Presqu'isle Du Quay, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 33, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg, Block 1, Engineer’s House (Public Building)

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Block 2, Lot I (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 2, Lot K (Private Building)

- Unspecified Shutter Type

(iv) 	1/2 Timber/Masonry Combination Buildings

Louisbourg: Block 13, Hospital (PROPOSAL - New Kitchen - Public Building)

- Unspecified Shutter Type

(v) 	Palisaded Redoubt Buildings

Isle Royale: Coast (PROPOSAL - Never-Built Sundry Public Buildings)

- Unspecified Shutter Type

(vi) 	Unspecified Building Types

Louisbourg: Dauphin Fauxbourg, Property 5 ? (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Royal Battery, Barraque (Engineer's) - (Wooden Building - Public Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 20? (Private/Public Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot A (Wooden House - Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot A (Wooden Storehouse - Private Building)

- Unspecified Shutter Type

St. Esprit (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 10 (Private Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 19, Lot B (Wooden Building - Private Building)

- Unspecified Shutter Type

Louisbourg: Barrachois, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: King’s Bastion, Pavilion (Masonry Public Building) AND/OR Block ?, Lot ? (Private Buildings)

- Unspecified Shutter Type

Louisbourg: Queen's Bastion, Barracks (1/2 Timber Public Building) OR Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

(4)	Without Shutters

(i)	Unspecified Building Types

Niganiche (Private Building)

- Without Shutters

(B)	By Building Sector and Shutter Type

(1)	Public Constructions

(i)	Emboîture Shutter Type

(a) 	1/2 Timber Buildings

Louisbourg: Block 18, Ancien Gouvernement (Public Building)

- Emboîture Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- Emboîture Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (Public Building)

- Emboîture Shutter Type

Louisbourg: Island Battery, Bakery (Public Building)

- Emboîture Shutter Type

(b) 	Piquet Buildings

N/A

(c) 	Masonry Buildings

Louisbourg: King's Bastion “C”, Barracks (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: King's Bastion, Barracks and Pavilions (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 13, Hospital (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, L’Ancien Magasin (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Angar [First Engineer’s House] - (Public Building)

- Emboîture Shutter Type

Louisbourg: Royal Battery, Barracks and Towers (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Magasin Du Roi (Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Powder Magazin (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Guard House (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Dauphin Demi-Bastion, Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Louisbourg: Lighthouse (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks, Guardhouse, Prison and Detached Latrine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Commander's Lodging (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Officers' Lodgings, Chapel Storehouse, and Bakery (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Powder Magazine (PROPOSAL - Public Building)

- Emboîture Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Emboîture Shutter Type

Port Toulouse: Barracks and Guardhouse and Prison and Latrines (Public Building)

- Emboîture Shutter Type

Port Toulouse: Magasin Des Vivres and Chapel and Officers Lodging and Bakery (Public Building)

- Emboîture Shutter Type

Louisbourg, Block 1, Engineer’s House (Public Building)

- Emboîture Shutter Type

Louisbourg: Island Battery, Guard House (Public Building)

- Emboîture Shutter Type

Louisbourg: Block 1, Forge (Public Building)

- Emboîture Shutter Type

(ii) 	Batten Shutter Type

(a) 	1/2 Timber Buildings

N/A

(b) 	Piquet Buildings

Louisbourg: Block 18, Guard House (Public Building)

- Batten Shutter Type

Louisbourg: Block 1, Beaucours (Public Building)

- Batten Shutter Type

Port Toulouse: Commander's Lodging (Public Building)

- Batten Shutter Type

Port Toulouse: Barracks, Officers' Lodgings, Chapel and Prison (Public Building)

- Batten Shutter Type

Port Toulouse: Chaplain and Surgeon’s Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Commander's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Chaplain and Surgeon's Lodging (Public Building)

- Batten Shutter Type

Port La Joye: Sub-Delegate's Lodging and Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Captain's Lodging (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Forge (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Bakery (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Aux Legumes (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Officers' Lodgings (Public Building)

- [Possible Batten] Shutter Type

Port La Joye: Magasin Des Vivres (Public Building)

- [Possible Batten] Shutter Type

(c) 	Masonry Buildings

Baye Royalle: Redoubt "A", Serving as a Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Redoubt "C", Serving as a Building for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "K", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: Battery "O", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Redoubt "I", Buildings for the Troops (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Toulouse: Battery "B", Crenellated Guard House Serving as a Storehouse (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Redoubt "E", Barracks (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Port Dauphin: Battery "L", Guard House (PROPOSAL - Never-Built Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type but Perhaps Batten

Louisbourg: King's Bastion, Barracks and Pavilions (Public Building)

- Batten Shutter Type (Aberrant 1749 Repair Work)

(iii) 	Unspecified Shutter Type

(a) 	1/2 Timber Buildings

Mirmiliguesche: Church (Public Building)

- Unspecified Shutter Type

Port La Joye: Magasin Des Vivres (PROPOSAL - Public Building)

- Unspecified Shutter Type

Port La Joye: Lieutenant De Roi and Officers' Lodgings (PROPOSAL - Public Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Magasin Du Roi Additional Victualling Storehouse (English Public Building)

- Unspecified Shutter Type

(b) 	Piquet Buildings

Louisbourg: Block 18, Casernes (Public Building)

- Unspecified Shutter Type

(c) 	Masonry Buildings

Louisbourg: Royal Battery “K”, Storehouse (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Royal Battery “K”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Powder Magazine (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Island Battery “I”, Guard House (PROPOSAL - Public Building)

- Possible Exterior Shutters

- Unspecified Shutter Type

Louisbourg: Block 18, Prison and Dungeon (Public Building)

- [Possible Unspecified Shutter Type]

Louisbourg: Block 1, Angar [First Engineer’s House] - (Public Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Magasin Du Roi (Public Building)

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

Louisbourg: Block 23, Lot A (Public Building)

- Unspecified Shutter Type

Louisbourg: Island Battery, (Public Building)

- Unspecified Shutter Type

Louisbourg, Block 1, Engineer’s House (Public Building)

- Unspecified Shutter Type (Aberrant 1749 Repair Work)

(d) 	1/2 Timber/Masonry Combination Buildings

Louisbourg: Block 13, Hospital (PROPOSAL - New Kitchen - Public Building)

- Unspecified Shutter Type

(e) 	Palisaded Redoubt Buildings

Isle Royale: Coast (PROPOSAL - Never-Built Sundry Public Buildings)

- Unspecified Shutter Type

(f) 	Unspecified Building Types

Louisbourg: Royal Battery, Barraque (Engineer's) - (Wooden Building - Public Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 20? (Private/Public Building)

- Unspecified Shutter Type

Louisbourg: King’s Bastion, Pavilion (Masonry Public Building) AND/OR Block ?, Lot ? (Private Buildings)

- Unspecified Shutter Type

Louisbourg: Queen's Bastion, Barracks (1/2 Timber Public Building) OR Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

(2) 	Private Constructions

(i)	Emboîture Shutter Type

(a) 	1/2 Timber Buildings

Louisbourg: Block 33?, Lot C? OR Block 37?, Lot D? (Private Building)

- Emboîture Shutter Type

(b) 	Piquet Buildings

N/A

(c) 	Masonry Buildings

Louisbourg: Block 2, Lot H (PROPOSAL - Never-Built Private Building)

- Emboîture Shutter Type

(ii) 	Batten Shutter Type

(a) 	1/2 Timber Buildings

Louisbourg: Block 15, Lot C (1752) (And E) (Private Building)

- Batten Shutter Type

(b) 	Piquet Buildings

Louisbourg: Block 5, Lot A (Private Building)

- Batten Shutter Type

Louisbourg: Block 32 ?, Lot ? OR Block 37 ?, Lot ? OR Block 38 ?, Lot ? (Private Building)

- Batten Shutter Type

(c) 	Masonry Buildings

N/A

(iii) 	Unspecified Shutter Type

(a) 	1/2 Timber Buildings

Baleine: Church (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 21 East, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 21 East, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West ?, Lot A OR Block 32 East?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 38 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 4, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West?, Lot C? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1756) - (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 15, Lot C (1757) - (Private Building)

- Unspecified Shutter Type

(b) 	Piquet Buildings

Louisbourg: Block 5, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 1, Dupont (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 34, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 5, Lot A ? OR Block Isle Du Quay, Lot E ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 32 West, Lot E (Private Building)

- Unspecified Shutter Type

Louisbourg: Dauphin Fauxbourg, Property [16] - (Private Building)

- Unspecified Shutter Type

(c) 	Masonry Buildings

Louisbourg: Block 34, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block Presqu'isle Du Quay, Lot A (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 33, Lot C (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 2, Lot I (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 2, Lot K (Private Building)

- Unspecified Shutter Type

(d) 	Unspecified Building Types

Louisbourg: Dauphin Fauxbourg, Property 5 ? (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 20? (Private/Public Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot A (Wooden House - Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot A (Wooden Storehouse - Private Building)

- Unspecified Shutter Type

St. Esprit (Private Building)

- Unspecified Shutter Type

Louisbourg: North Shore, Property 10 (Private Building)

- Unspecified Shutter Type

Louisbourg: Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 16, Lot B (Private Building)

- Unspecified Shutter Type

Louisbourg: Block 19, Lot B (Wooden Building - Private Building)

- Unspecified Shutter Type

Louisbourg: Barrachois, Property ? (Private Building)

- Unspecified Shutter Type

Louisbourg: King’s Bastion, Pavilion (Masonry Public Building) AND/OR Block ?, Lot ? (Private Buildings)

- Unspecified Shutter Type

Louisbourg: Queen's Bastion, Barracks (1/2 Timber Public Building) OR Block ?, Lot ? (Private Building)

- Unspecified Shutter Type

(iv) 	Without Shutters

(a) 	Unspecified Building Types

Niganiche (Private Building)

- Without Shutters

(V)

Revised Report - English Version

(A) 	Christian Pouyez, “Preliminary Report On Outside Shutters” in Historians, PRELIMINARY ARCHITECTURAL STUDIES, Volume 02, Unpublished Report H G 02 (Fortress of Louisbourg, 1972)

This report on outside shutters is based exclusively on documentation now indexed in the Fichier d'Architecture Civile, (Civil Architecture) [i.e. the Domestic Architecture File]. We have a fairly large amount of information at our disposal since we found references to outside shutters [contrevents] for thirty-two private houses and references to inside shutters [volets] for two houses. However, details concerning the size and price of such outside shutters are only rarely provided in the documents dealing with private houses; we must therefore to some extent rely on the accounts and measurements for military buildings, which are much more precise.

This study will consist of seven sections, as follows:

(i)	definition and location,

(ii) 	size,

(iii) 	materials used,

(iv) 	method of manufacture,

(v) 	hardware

(vi) 	price of outside shutters.

(vii) 	a seventh section will deal with inside shutters.

(1) 	Definition and Location

According to the definition given by Paul Robert, a contrevent is a large outside shutter used to protect the window against bad weather. It is important to distinguish between contrevents and volets, a word originally used to describe wooden panels hung on the inside of a window to protect it or to shut out the light; we shall refer to such inside shutters again in the last section of this report.

[TRANSLATOR: In French there may be some confusion between contrevents, here called "outside shutters", and volets, referred to as "inside shutters", since although contrevents can be only outside shutters, the term volet can be used in modern French to designate either inside or outside shutters.]

Outside shutters were in fairly common use at Louisbourg; although it is impossible to give exact statistics at this time, we can say that there was a tendency to hang outside shutters on ground-floor windows, especially if such windows opened on to the street. On the other hand, we do not find many references to shutters on second-floor windows or on attic windows; it is possible that shutters provided protection not only against bad weather but also against possible thieves and this would explain the less frequent use of shutters on the upper floors. In military buildings, in particular in barracks, we find shutters on all the windows, including dormers. They were installed mainly to protect the window against bad weather, but it certainly seems that the soldiers did not worry much about such weather; in 1728 Maurepas wrote to Verrier that the major should pay more attention to making sure the soldiers closed the shutters when it was raining.

(2) 	Size

Only the measurements and accounts for the king's buildings furnish some indication of the size of outside shutters. The problem here is not so much the size of the shutters themselves as their size in relation to that of the window bay or window sash, when there is one. In Table 1 we have grouped together some shutter sizes compared with window bay or sash sizes. Although the shutters are sometimes the same size as the sash, they are sometimes larger than the bay of the window, especially when there is no sash (e.g. Magasin des vivres). In the first case (shutter same size as sash), the closed shutter must have been flush with the wall, in short, set into the window opening, while in the second case (shutter larger than sash or window opening), it projected.

TABLE 1

COMPARISON OF SIZES OF OUTSIDE SHUTTERS,

SASHES AND WINDOW OPENINGS

[Note: 1 pouce = 1.0656 inches; 1 pieds = 1.066 feet]

(01) BUILDING (1718): Guard-room and prison (King’s building)

SIZE OF OUTSIDE SHUTTERS: = 4 pieds x 3 pieds

SIZE OF SASH = 4 pieds x 3 pieds

SIZE OF WINDOW BAYS = n/a

(02) BUILDING (1727): King’s Barracks - Officers' Quarters (King’s building)

SIZE OF OUTSIDE SHUTTERS = 4 pieds x 3 pieds

SIZE OF SASH = 4 pieds x 3 pieds; n/a

(03) BUILDING (1727): Barracks- Pavilion (King’s building)

SIZE OF OUTSIDE SHUTTERS = 4 pieds 8 pouces x 3 pieds

SIZE OF SASH = 4 pieds 8 pouces x 3 pieds

SIZE OF WINDOW BAYS = n/a

(04) BUILDING (1727): Magasin des vivres (King’s building)

SIZE OF OUTSIDE SHUTTERS = 3 pieds 4 pouces x 5 pieds 6 pouces

SIZE OF SASH = n/a

SIZE OF WINDOW BAYS = 3 pieds x 4 pieds 8 pouces (approximate)

(05) BUILDING (1731): Barracks - Left Pavilion (King’s building)

SIZE OF OUTSIDE SHUTTERS = 4 pieds 2 pouces x 3 pieds

SIZE OF SASH = 4 pieds x 3 pieds

SIZE OF WINDOW BAYS = n/a

(3) 	Materials

Pine was the wood most frequently used in shutter construction, (Table 2). Sometimes fir was used, especially in the colony's early years. Sometimes imported lumber (from New England) was used; we have an example of this in the lawsuit between Pierre Merle, carpenter and Pierre Eon and Dollé; Eon and Dollé had first asked Pierre Merle to make shutters of imported lumber and then, changing their minds, they wanted them made of local lumber, but at the same price. Since local lumber was more expensive than imported lumber, this change of mind led to a lawsuit.

TABLE 2

OUTSIDE SHUTTERS - MATERIALS

(01) BUILDING (1718): Governor's house (King’s building)

- Fir

(02) BUILDING (1719): King's buildings

- Fir

(03) BUILDING (1733): Ile St. Jean (King’s buildings)

- Pine

(04) BUILDING (1733): Port Toulouse (King’s building)

- Oak or merisier [type of birch]

[NOTE: Oak or merisier used for the emboîtures only]

(05) BUILDING (1737): Duperrier-Rodrigue house (Private building)

- Pine

(06) BUILDING (1737): [King's buildings]

- Pine

(07) BUILDING (1753): [King's buildings]

- Pine

(08) BUILDING (1753): [King’s buildings]

- Local lumber

(09) BUILDING (1754): Aurieu house (Private building)

- Local lumber

(10) TOTAL: 9 buildings

- 4 buildings where the shutters are of Pine

- 2 buildings were the shutters are of Fir

- 1 building where the emboîtures of the shutters are of Oak or Merisier [type of birch]

- 2 buildings where the shutters are of Local Lumber

(4) 	Method of Manufacture and

Types of Outside Shutter

There are two types of outside shutter, single-leaf shutters and double-leaf shutters. Double-leaf shutters are found on both private houses  and public buildings;  the use of double-leaf shutters is obviously determined by the size and shape of the window.

As for the method of manufacture, we must distinguish between emboîture fitted shutters and batten shutters. Unfortunately we have only two documents in which the type of shutter used in private houses is mentioned: Laurent Dybarat’s cabin whose shutters are described as consisting "of two battens."  The second reference concerns the house of Pierre Aurieu, innkeeper, where the shutters have battens; the number of battens is not specified.  In the documents regarding public buildings (barracks, Magasin des vivres, etc.) we find more detail; of seven references, three deal with shutters with battens while the other four concern emboîture shutters.

(i) Shutters with battens

These are always made with one pouce [vertical] softwood boards (pine or fir) except for the battens which are sometimes of oak or merisier [type of birch]. The boards are tongued and grooved and planed on both sides. The number of battens varies; there are three battens in the shutters of the governor's house, two in those of the guard-room. Sometimes the number is not specified.

(ii) Emboîture Shutters

Except in one case [1-1/4 pouce],  they are all made of one pouce [vertical] softwood boards except for the [horizontal end] emboîtures which are normally oak or merisier [type of birch] [with pine always a possibility]. The boards are tongued and grooved and planed on both sides and in one case it is specified that the [emboîtures] must be [well-]pegged.

(5) 	Hardware

The shutters were usually hung on a hinge pivoting on a pintle. Only one document specifies that there were two hinges per shutter,  but it seems probable that this was true of all shutters. Most of the time it was enough to say that the shutters were to be properly mounted or else "permanently mounted" without providing any further detail.

The same closing apparatus seems to have been used everywhere; in the nine documents where this is mentioned, the shutters are always closed by means of a hook and eye.

To keep the shutters open, a turnbuckle was attached to the wall and used to hold the shutter open in such a way that it did not bang in the wind.

(6) 	Price

There is little to say about prices. Only the measurements and accounts for public buildings specify the price of shutters; they are always included in the same section as the doors and they are paid for by the square toise. Table 3 shows the prices we noted.

TABLE 3

PRICE OF OUTSIDE SHUTTERS:

(01) BUILDING (1727): Barracks (King’s building)

- PRICE PER SQUARE TOISE = 30 livres

- TYPE OF SHUTTER = n/a

(02) BUILDING (1727): Magasin des vivres (King’s building)

- PRICE PER SQUARE TOISE = 25 livres

- TYPE OF SHUTTER = n/a

(03) BUILDING (1731): Barracks, hospital, storehouse (King’s buildings)

- PRICE PER SQUARE TOISE = 30 livres

- TYPE OF SHUTTER = n/a

(04) BUILDING (1733): Port Toulouse (King’s buildings)

- PRICE PER SQUARE TOISE = 23 livres

- TYPE OF SHUTTER =emboîture

(05) BUILDING (1734): Engineer's house (King’s buildings)

- PRICE PER SQUARE TOISE = 25 livres

- TYPE OF SHUTTER = n/a

(06) BUILDING (1737): [King’s buildings]

- PRICE PER SQUARE TOISE = 20 livres

- TYPE OF SHUTTER = emboîture

(7) 	Inside Shutters

We must now take a quick look at inside shutters which, as we have seen, are attached to the inside of the window; when there is a sash, the shutters are attached to the fixed frame. In the [privately owned] house of Pierre Aurieu, innkeeper, built in 1754, there is even an example of windows whose sashes carry at the same time both outside and inside shutters:

... the batten doors and outside shutters shall also be made of local lumber; at each gable-end there shall be a fire-place on each of the two floors, built of brick, lime and sand; the sashes shall be of local lumber, shall have inside shutters and shall be fitted, like the outside shutters, to the fixed frame [however, what may have been meant here was that two-leaf sashes were to fitted, like the outside shutters, to the fixed frame] ... the glass in the sashes shall consist of twenty-four panes, each seven pouces by eight pouces."

In the [privately owned] building constructed for Beaubassin, Sylvain and Co. we find another example of inside shutters; the windows of the "warehouse" section are simply fitted with iron bars and "closed inside by means of an inside shutter [fermées en dedans d’un contrevent] with its two pintles, two strap-hinges, and single [locking] bolt."  In this case we can certainly ask ourselves if the term volet is really applicable; indeed, here there is neither sash nor fixed-frame. Since inside shutters are specifically intended to protect a sash and shut out the light, we have a problem here. It seems to us that in this [Beaubassin] case we should instead refer to an outside shutter (providing protection against the wind and weather) hung on the inside of a window.

ENDNOTES

C11A, Volume 126, Folio 54-55, March 15, 1752; Bibliothèque Génie, Article 14, pièce 45, 1752; Le merisier est un bois dur, son ecorce est grise le bois est blanchâtre. Il y en a de gros comme des bariques et de la hauteur des chenes les plus élevé. Cet arbre est droit Il a la feuille ouvals On en fait des poutres, des Soliveaux et autre ouvrages de Charpente ..., C11E, Volume 13, Page 82-83, c. 1680's.

C11B, Volume 3, Folio 121, December 30, 1718 [Door description: ... pour deux portes au corps de garde, l'une d'entrée et l'autre a la chambre de l'officier de six pieds de haut sur trois de large embouvetées, parées des deux côtez et barrées de trois barres a raison de cent sols piece la somme de ... 10"-0-0- ...]

See for an example: C11B, Volume 1, Folio 495, 1716.

See for an example: C11B, Volume 1, Folio 495, 1716.

C11B, Volume 3, Folio 121, 123, December 30, 1718.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

G3, Carton 2057, Pièce 23-24, November 21, 1721.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 28, Folio 270, September 9, 1751.

C11B, Volume 28, Folio 270, September 9, 1751.

C11B, Volume 28, Folio 333, December 31, 1749 [Door description: ... Portes de planches du pays d'1 pouce d'Epaisseur Embouveté et Emboitié par les deux bouts ... 22# La Toise quarrée ...]

C11B, Volume 28, Folio 333, December 31, 1749.

C11B, Volume 3, Folio 121, 123, December 30, 1718.

C11B, Volume 3, Folio 121, December 30, 1718 [Door description: ... pour deux portes au corps de garde, l'une d'entrée et l'autre a la chambre de l'officier de six pieds de haut sur trois de large embouvetées, parées des deux côtez et barrées de trois barres a raison de cent sols piece la somme de ... 10"-0-0- ...]

C11B, Volume 28, Folio 270, September 9, 1751.

G3, Carton 2057, Pièce 23-24, November 21, 1721.

C11B, Volume 28, Folio 270, September 9, 1751.

See for an example: C11B, Volume 1, Folio 495, 1716.

See for an example: C11B, Volume 1, Folio 495, 1716.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

C11B, Volume 28, Folio 333, December 31, 1749 [Door description: ... Portes de planches du pays d'1 pouce d'Epaisseur Embouveté et Emboitié par les deux bouts ... 22# La Toise quarrée ...]

C11B, Volume 28, Folio 333, December 31, 1749.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 3, Folio 121, December 30, 1718 [Door description: ... pour deux portes au corps de garde, l'une d'entrée et l'autre a la chambre de l'officier de six pieds de haut sur trois de large embouvetées, parées des deux côtez et barrées de trois barres a raison de cent sols piece la somme de ... 10"-0-0- ...]

C11B, Volume 28, Folio 270, September 9, 1751.

G3, Carton 2057, Pièce 23-24, November 21, 1721.

C11B, Volume 28, Folio 270, September 9, 1751.

See for an example: C11B, Volume 1, Folio 495, 1716.

See for an example: C11B, Volume 1, Folio 495, 1716.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

C11B, Volume 28, Folio 333, December 31, 1749 [Door description: ... Portes de planches du pays d'1 pouce d'Epaisseur Embouveté et Emboitié par les deux bouts ... 22# La Toise quarrée ...]

C11B, Volume 28, Folio 333, December 31, 1749.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 3, Folio 121, 123, December 30, 1718.

C11B, Volume 3, Folio 121, December 30, 1718 [Door description: ... pour deux portes au corps de garde, l'une d'entrée et l'autre a la chambre de l'officier de six pieds de haut sur trois de large embouvetées, parées des deux côtez et barrées de trois barres a raison de cent sols piece la somme de ... 10"-0-0- ...]

C11B, Volume 28, Folio 270, September 9, 1751.

G3, Carton 2057, Pièce 23-24, November 21, 1721.

C11B, Volume 28, Folio 270, September 9, 1751.

C11B, Volume 3, Folio 121, December 30, 1718 [Door description: ... pour deux portes au corps de garde, l'une d'entrée et l'autre a la chambre de l'officier de six pieds de haut sur trois de large embouvetées, parées des deux côtez et barrées de trois barres a raison de cent sols piece la somme de ... 10"-0-0- ...]

See for an example: C11B, Volume 1, Folio 495, 1716.

See for an example: C11B, Volume 1, Folio 495, 1716.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 127v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

For example, see: C11B, Volume 3, Folio 124v, December 30, 1718.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 9, Folio 248v, November 15, 1727; C11B, Volume 12, Folio 139v, September 1, 1731.

C11B, Volume 28, Folio 333, December 31, 1749 [Door description: ... Portes de planches du pays d'1 pouce d'Epaisseur Embouveté et Emboitié par les deux bouts ... 22# La Toise quarrée ...]

C11B, Volume 28, Folio 333, December 31, 1749.

Paul Robert, Dictionnaire alphabétique et analogique de la langue française, Article CONTREVENT.

Paul Robert, Dictionnaire..., Article VOLET.

B, Volume 52, Folio 591, June 20, 1728.

C11B, Volume 3, Folio 119-130v, December 31, 1718; C11B, Volume 9, Folio 171v, et Folio 174, May 4, 1727; C11B Volume 9, Folio 182 and Folio 188-189, November 1, 1727; C11B, Volume 12, Folio 138-139, September 1, 1731.

G3, Carton 2042, Pièce 69, July 26, 1754; G2, Volume 184, Folio 192-194, 1737; G2, Volume 203, Dossier 304, Folio 53v, August 3, 1753; C11B, Volume 3, Folio 125-126, December 31, 1718; Archives du Séminaire de Québec, Papiers Surlaville, September 25, 1753; C11B, Volume 14, Folio 312v, September 28, 1733;C11B, Volume 4, Folio 278-282, March, 1719; C11B, Volume 14, Folio 319v, September 23, 1733; C11B, Volume 19, Folio 183, May 10, 1737.

G2 Volume 182, Folio 186, March 5, 1733; G3 Carton 2045, Pièce 67, June 1, 1756.

C11B Volume 9, Folio 188-189, November 1, 1727.

G3, Carton 2057, Pièce 23-24, November 21, 1721.

G3, Carton 2042, Pièce 69, July 26, 1754.

C11B, Volume 3, Folio 119-130, December 31, 1718; Archives du Séminaire de Québec, Papiers Surlaville, September 25, 1753.

C11B, Volume 14, Folio 312v, September 28, 1733 [Shutter description: Pour les ouvrages à faire au Port Toulouse, on exigeait l'emploi de planches d'un pouce et quart d'épaisseur.]

Archives du Séminaire de Québec, Papiers Surlaville, September 25, 1753, Article 34; C11B, Volume 14, Folio 312v, September 28, 1733; C11B Volume 4, Folio 278-282, March 7, 1719; C11B, Volume 14, Folio 319v, September 23, 1733, Article 7; C11B Volume 9, Folio 183, May 10, 1737, Article 40.

G2, Volume 179, Folio 429-432, May 12, 1726.

G3, Carton 2045, Pièce 37, September 12, 1757; G2, Volume 179, Folio 149, April 10, 1726.

C11B, Volume 3, Folio 119-130, December 31, 1718; G3, Carton 2045, Pièce 37, September 12, 1757; G2 Volume 208, Dossier 475, Pièce 72, August 7, 1752; G2, Volume 194, Folio 238, September 13, 1735;G2, Volume 183, Folio 179, November 6, 1743; G2, Volume 181, Folio 491, October 9, 1732; G2, Volume 179, Folio 429-432, May 12, 1726;

G2, Volume 179, Folio 149, April 10, 1726.

C11B, Volume 3, Folio 119-130, December 31, 1718; C11B, Volume 9, Folio 145, November 17, 1727.

C11B, Volume 9, Folio 171-173, May 4, 1727; C11B Volume 9, Folio 188-189, November 1, 1727; C11B, Volume 12, Folio 138-139, September 1, 1731; C11B, Volume 14, Folio 312v, September 28, 1733; C11B Volume 16, Folio 208-209, September 30, 1734; C11B, Volume 19, Folio 183, May 10, 1737.

G3, Carton 2042, Pièce 69, July 26, 1754.

G3, Carton 2044, Pièce 53, May 30, 1756.